User Guide and Data Dictionary

In-year Fire Perimeter

Geospatial Ontario Data Class

Aviation, Forest Fire and Emergency Services Branch

Provincial Services Division

Ministry of Natural Resources

Ontario Public Service (OPS)

January 08, 2026

© King’s Printer for Ontario, 2026. Licensed under the Open Government Licence – Ontario (ontario.ca/open)

Table of Contents

Dataset Overview	3

Access & Usage	3

Add the layer in QGIS	3

Add the layer in ArcGIS Pro	3

Add the layer in ArcGIS Online	3

Attribute Data Dictionary	5

Domains & Coded Values	6

Metadata Summary	6

Visualization Suggestions	6

Contact & Support	6

Appendix	6

# Dataset Overview

Dataset Name: In Year Fire Perimeter

Description: Fire perimeters for this year's forest fires in Ontario. Perimeters are illustrations only and may not be exact; updated daily during fire season.

Source: Aviation, Forest Fire and Emergency Services Branch

Spatial Reference: WGS 1984 (EPSG:4326)

Geometry Type: Polygon

Extent: West -93.753, East -75.737, South 41.918, North 59.934

Service URL: https://ws.lioservices.lrc.gov.on.ca/arcgis2/rest/services/LIO_OPEN_DATA/LIO_Open09/MapServer/51

Update Frequency: Daily (during fire season)

# Access & Usage

## Add the layer in QGIS

1. Open QGIS >  Layer > Add Layer > Add ArcGIS MapServer Layer.

2. Click ‘New’, give it a name (e.g., “GEO Open Data Service 9”), and paste the service URL:   https://ws.lioservices.lrc.gov.on.ca/arcgis2/rest/services/LIO_OPEN_DATA/LIO_Open09/MapServer

3. Click ‘OK’, then ‘Connect’.

4. Select the desired layer (e.g., layer 51, In-year Fire Perimeter) and click ‘Add’.

## Add the layer in ArcGIS Pro

1. Open ArcGIS Pro > Insert > Connections > New Map Service Connection.

2. Paste the service URL: https://ws.lioservices.lrc.gov.on.ca/arcgis2/rest/services/LIO_OPEN_DATA/LIO_Open09/MapServer/51

3. Expand the service and add the layer to your map.

## Add the layer in ArcGIS Online

1. Sign in to ArcGIS Online > Map Viewer.

2. Add > Add Layer from Web > Type: ArcGIS Server Web Service.

3. Paste the service URL: https://ws.lioservices.lrc.gov.on.ca/arcgis2/rest/services/LIO_OPEN_DATA/LIO_Open09/MapServer/51 and select the layer.

# Attribute Data Dictionary

| Field Name | Alias | Description | Example | Type | Length | Nullable | Domain | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OBJECTID | OBJECTID |  | 123 | OID | — | No | — | — |
| FIRENUMB | Fire Number |  | RED022 | String | 16 | Yes | — | — |
| CUR_SIZE | Current Size |  | 4433.956 | Double | — | Yes | — | — |
| REFERENCE | Reference |  | MB | String | 30 | Yes | — | — |
| FIRETYPE | Fire Type |  | IFR | String | 3 | Yes | {IFR=Inside Fire Region, OFR=Outside Fire Region} | — |
| STATUS | Status |  | F | String | 1 | Yes | {F=Final, I=Interim} | — |
| DATE_MAPPED | Date Mapped |  | 2025-08-29 17:00 | Date | 8 | Yes | — | — |

# Domains & Coded Values

If the service exposes coded value domains, they will be listed here.

STATUS: F = Final, I = Interim

FIRETYPE: IFR = Inside Fire Region, OFR = Outside Fire Region

# Metadata Summary

Abstract: Fire perimeters are illustrations only and may not be exact; captured as resources allow; updated daily during fire season.

Keywords: GeospatialOntario, AFFES, Fire, Forest Fire, Fire Perimeter, wildfire, boundaries, open data

Constraints: Open Government Licence – Ontario

Temporal Extent: 2025-01-04 to 2025-10-31

# Visualization Suggestions

Use a red outline (2 pt) with semi-transparent fill for perimeters. Label by Fire Number (FIRENUMB) if desired.

# Contact & Support

Contact: AFFES GIS Specialist (AFFES.GIS@ontario.ca)

Ontario GeoHub: https://geohub.lio.gov.on.ca

OPS Open Government Licence: https://www.ontario.ca/page/open-government-licence-ontario

# Appendix

Primary Service URL: https://ws.lioservices.lrc.gov.on.ca/arcgis2/rest/services/LIO_OPEN_DATA/LIO_Open09/MapServer/51