Geospatial Ontario (GEO)

Logical/Physical Data Dictionary

For

MNRF Road Segment (MNRRDSEG) and MNR Road Network (MNRRDNET)

Subject Area: Global Model

Revision Date: March 31, 2026

# Model Diagram - Physical

# Model Diagram - Logical

# Data Class Metadata

| Full Name | Short Name | Version | Description | Abstract Class |
| --- | --- | --- | --- | --- |
| MNRF Road Segment | MNRRDSEG | 6 | This layer contains linear features spatially representing roads crossing Crown land, maintained by or is of interest to the Ontario Ministry of Natural Resources. Road segments can be consolidated into networks using the MNRF Road Network entity. When a road segment is no longer accessible by conventional four-wheel vehicles, the STATUS field is updated, and the geometry is retained (never deleted). This layer should not include ATV trails or temporary paths used for forest harvesting e.g. skidder trails. There may be road segments whose geometry are coincident with the Ontario Road Network (ORN) features. Any updates between the Road Segment layer and the ORN database must be done interactively. | SPSLINE |
| MNRF Road Network | MNRRDNET | 3 | A road network is a consolidation of one or more road segments and relies on the road segment for its geometry. A road segment may belong to one or more road network. | AGGREGATE |

# Data Dictionary

## Table Name: MNRF_ROAD_NETWORK

A road network is a consolidation of one or more road segments and relies on the road segment for its geometry. A road segment may belong to one or more road network consolidations.

| # | ATTRIBUTE/COLUMN NAME | DATATYPE | REQ | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 1 | OGF Id OGF_ID | NUMBER(13) | Yes | Ontario Geospatial Feature (OGF) Id. A unique numeric provincial identifier assigned to each record. |
| 2 | Network Business Identifier NETWORK_BUSINESS_IDENT | VARCHAR2(80) | Yes | A text string containing the business identifier assigned to the road network e.g., Old Wagon Road Network. Road networks can be used to name routes and group road segments for forest or road use management This field can be used to annotate maps and link to a Road Network Management Strategy database for SFL and MNR road networks. |
| 3 | Network Group NETWORK_GROUP | VARCHAR2(80) | No | A user-defined text string allowing one or more road networks to be assigned to a unique road system or Forest Management Unit e.g., Mazinaw-Lanark Forest. |
| 4 | Network Description NETWORK_DESCR | VARCHAR2(500) | No | Description of the road network. May include why the network was created. |
| 5 | Network Comment NETWORK_COMMENT | VARCHAR2(100) | No | Brief comments about the road network. |
| 6 | Network Maintenance NETWORK_MAINTENANCE | VARCHAR2(250) | No | Description of the maintenance plan for the road network. Used for audit and reporting purposes related to road use strategies |
| 7 | Network Monitoring NETWORK_MONITORING | VARCHAR2(250) | No | Description of the monitoring program for the road network. Used for audit and reporting purposes related to road use strategies |
| 8 | Network Access NETWORK_ACCESS | VARCHAR2(250) | No | Description of the access controls planned or in place on the road network. Used for audit and reporting purposes related to road use strategies. |
| 9 | Network Future NETWORK_FUTURE | VARCHAR2(80) | No | Intent for the future use of the road once it is transferred to Ministry responsibility. Used for audit and reporting purposes related to road use strategies. |
| 10 | Network Transfer Year NETWORK_TRANSFER_YEAR | VARCHAR2(15) | No | Year that the road segments of the network were or are expected to be transferred to MNR responsibility. Used for audit and reporting purposes related to road use strategies. |
| 11 | Effective Datetime EFFECTIVE_DATETIME | DATE | Yes | Date/time the record was created or last modified in the source database. |
| 12 | System Datetime SYSTEM_DATETIME | DATE | Yes | Date/time the record was loaded into or last modified in the Geospatial Ontario (GEO) database. DEFAULT: SYSDATE |

## Table Name: MNRF_ROAD_NETWORK_AND_SEGMENT

Intersection table between MNRF Road Network and MNRF Road Segment tables.

| # | ATTRIBUTE/COLUMN NAME | DATATYPE | REQ | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 1 | MNR Road Network ID MNRF_ROAD_NETWORK_ID | NUMBER(13) | Yes | Foreign Key (FK) to parent MNRF Road Network table record OGF_ID. |
| 2 | MNR Road Segment ID MNRF_ROAD_SEGMENT_ID | NUMBER(13) | Yes | Foreign Key (FK) to parent MNRF Road Segment table spatial feature record OGF_ID. |
| 3 | Effective Datetime EFFECTIVE_DATETIME | DATE | Yes | Date/time the record was created or last modified in the source database. |
| 4 | System Datetime SYSTEM_DATETIME | DATE | Yes | Date/time the record was loaded into or last modified in the LIO database. DEFAULT: SYSDATE |

## Table Name: MNRF_ROAD_SEGMENT_FT

This layer contains linear features spatially representing roads crossing Crown land, maintained by or is of interest to the Ontario Ministry of Natural Resources. Road segments can be consolidated into networks using the MNRF Road Network entity. When a road segment is no longer accessible by conventional four-wheel vehicles, the STATUS field is updated, and the geometry is retained (never deleted).

| # | ATTRIBUTE/COLUMN NAME | DATATYPE | REQ | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 1 | OGF Id OGF_ID | NUMBER(13) | Yes | Ontario Geospatial Feature (OGF) Id. A unique numeric provincial identifier assigned to each record. |
| 2 | Road Network Identifier ROAD_NETWORK_IDENTIFIER | VARCHAR2(6) | No | The road network identifier for road asset management. Made up of three letter codes for the district, and three digits for a sequential ID of the network within the district. Ex: TIM001, TIM002, CHA001, etc. This attribute provides a direct link to network attributes for segments belonging to Asset Management Plan (AMP) networks. To access network attributes for non-AMP networks, use the intermediate table MNRF_ROAD_NETWORK_AND_SEGMENT to allow a many-to-many relationship between segments and networks. |
| 3 | Priority Road Indicator PRIORITY_ROAD_IND | VARCHAR2(3) | Yes | Yes/No indicator if the feature belongs to a high priority road. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 4 | Road Authority ROAD_AUTHORITY | VARCHAR2(10) | Yes | The entity that has legal authority to enter into agreements with respect to the use and maintenance of the road segment, described as follows: 1] MNRF - Ministry of Natural Resources. 2] MTO - Ministry of Transportation. 3] Municipal - Municipal Government Road Authority. 4] Federal - Government of Canada (including First Nations). 5] Private - private citizen, company, or corporation. DEFAULT: MNRF VALID VALUES: MTO Municipal Federal Private MNRF DEFAULT: 'MNRF' |
| 5 | Ownership Class OWNERSHIP_CLASS | VARCHAR2(10) | Yes | Legal ownership of the road right-of-way and authority to assign responsibility for a road to another party. This field is used by the MNR to track which roads are on Crown land even though the responsibility for the road may be assigned to a third party (e.g. SFL, Local Roads Board, etc.). The owner of the right-of-way is a stakeholder in the road regardless of who is the Road Authority and who has Responsibility for the road. In some areas it may be possible to derive this information from a topological overlay with land tenure information. Make sure however that the tenure information is accurate and up to date, noting that there may be text in the tenure agreements describing a road allowance that is not shown in the LIO tenure layers. DEFAULT: Crown VALID VALUES: Federal Crown Private Municipal DEFAULT: 'Crown' |
| 6 | Responsibility Class RESPONSIBILITY_CLASS | VARCHAR2(30) | Yes | A classification of the agents listed in the Road Responsibility Detail attribute. DEFAULT: MNRF VALID VALUES: Federal First Nations MNRF FRL LRB MOU MTO Municipal Ontario Parks Private SFL DEFAULT: 'MNRF' |
| 7 | Responsibility Detail RESPONSIBILITY_DETAIL | VARCHAR2(80) | No | Name of the specific group or individual legally responsible for activities associated with a road including condition and maintenance. May refer to the reference number of a shared use agreement involving a number of parties. Responsibility Detail must be filled out when the Responsibility Class is neither MTO nor MNRF. Identify the reference to the legal agreement name and number (e.g. LUP District#-XXXXXXX ; MOU-District-XX-YR) and/or identify the name of the corporation (ex: Responsibility Class = SFL, Responsibility Detail = Tembec). If the agreement is with a private individual, do not enter the name of the individual (or any personal contact information), only the agreement number, as this layer is non-sensitive and freely distributed. |
| 8 | National Road Class NATIONAL_ROAD_CLASS | VARCHAR2(30) | Yes | A functional (versus administrative) based classification system. The National Road Network (NRN) Class list can be described as follows: 1] Local / Strata - A low speed thoroughfare dedicated to providing access to properties with potential public restriction, trailer parks, first nations, strata or private estates. 2] Resource / Recreation - A narrow passage which has as a primary function access for resources extraction and may have a role in providing access for the public to back country. 3] Winter-A Road that is only useable during the winter months when conditions allow for passage over lakes, rivers and wetlands. Notes: 1] If Winter is selected then it is implied that all values for Status, Passable, etc. all refer to the period when the road would be in operation (e.g. Dec to April). 2] While the NRN defines this attribute with more choices than what we are allowing here, if the road is not described by one of the values above then it does not belong in this layer. VALID VALUES: Local / Strata Resource / Recreation Winter |
| 9 | Geometric Standard Class GEOMETRIC_STANDARD_CLASS | VARCHAR2(1) | No | The geometric standard road classification. Classes include A, B, C, and D. Guidelines specify minimum-maximum clearing width, speed, roadway width, horizontal curvature, maximum gradient, minimum sight stopping distance, minimum ditch depth, minimum culvert size, maximum aggregate size and granular structure. (PL 10.06.00 ARM-1992). This is the primary classification field for deriving the road value. Note: once this field is filled in we will drop Surface Type and Number of lanes. DEFAULT: C VALID VALUES: A B C D DEFAULT: 'C' |
| 10 | FMP Road Class FMP_ROAD_CLASS | VARCHAR2(1) | No | Annual report road class is described as follows: P=Primary, B=Branch, O=Operational. VALID VALUES: P B O |
| 11 | Road Name ROAD_NAME | VARCHAR2(150) | No | The primary or official name of the road. |
| 12 | Alternate Name ALTERNATE_NAME | VARCHAR2(150) | No | Road name that has no official status but is used or known by the general public. |
| 13 | Status STATUS | VARCHAR2(15) | Yes | Access roads on Crown land are generally open for public use. However, there may be occasions when access must be limited to specific areas or when a road is temporarily or permanently closed to all users (Access Roads Manual, 1992). DEFAULT: Open VALID VALUES: Closed Internal Open Restricted DEFAULT: 'Open' |
| 14 | Closure Intent CLOSURE_INTENT | VARCHAR2(15) | No | Specifies the management purpose for a closed road segment, as determined by the current use of the road or the applicable road use management strategy. VALID VALUES: Caribou ESA Public Safety Regeneration Remote Tourism Unauthorized |
| 15 | Closed State CLOSED_STATE | VARCHAR2(17) | No | Management state for a closed and not maintained road, tracking the degree of financial and management investment in renewal or rehabilitation, described as follows: 1] Untreated: Undergoing natural deterioration with no treatment applied to accelerate roadbed breakdown or for the purpose of vegetation establishment. 2] Treated: roadbed destruction, site preparation, and/or vegetation regeneration treatment have been prescribed and applied to facilitate road breakdown or vegetative renewal. 3] Regenerated-Objective Achieved: Vegetation renewal efforts have achieved a prescribed regeneration objective in the UMS (Ex remote tourism, minimizing loss of productive land). 4] Regenerated-Objective Not Achieved: Vegetation renewal efforts have NOT achieved a prescribed regeneration objective in the UMS. 5] Rehabilitated-Caribou: A habitat renewal objective has been achieved to satisfy caribou habitat requirements. 6] Rehabilitated-ESA: A habitat renewal objective has been achieved to satisfy other ESA species requirements. The typical progression stages are as follows: Untreated to Treated, Treated to Regenerated, Regenerated to Rehabilitated-C or Rehabilitated-ESA. Note: If populated MAINTENANCE_LEVEL must be "Not Maintained". VALID VALUES: Untreated Treated Regenerated-OA Regenerated-ONA Rehabilitated-C Rehabilitated-ESA |
| 16 | Treatment Type TREATMENT_TYPE | VARCHAR2(20) | No | The type of treatment applied to the roadbed of a closed road, described as follows: 1] Natural - natural regeneration. 2] Plant - reclamation plant. 3] Scarify - reclamation scarify. 4] Scarify/Other - combination of reclamation scarify and other reclamation method. 5] Scarify/Plant - combination of reclamation scarify and reclamation plant. 6] Scarify/Seed - combination of reclamation scarify and reclamation seed. 7] Seed - reclamation seed. 8] Other - other reclamation method (to be described in General Comments). This field is mandatory if CLOSED_STATE cannot be equal to "Untreated". VALID VALUES: Natural Plant Scarify Scarify/Plant Scarify/Other Scarify/Seed Seed Other |
| 17 | Road Use ROAD_USE | VARCHAR2(20) | No | The management intent for the road segment as defined by the current use of the road or by the road use management strategy. VALID VALUES: Forest Management Cottage/Residential Aggregate Mining Ministry Operations Recreation Other Commercial Renewable Energy Resource Management Tourism |
| 18 | Activity Restriction Indicator ACTIVITY_RESTRICTION_IND | VARCHAR2(3) | Yes | Yes/No indicator if certain activities are restricted on this road segment. Specific details regarding this restriction to be captured in USE_RESTRICTION_DETAIL. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 19 | Destination Restriction Indicator DESTINATION_RESTRICTION_IND | VARCHAR2(3) | Yes | Yes/No indicator if access to specific destinations is restricted via use of this road segment. Specific details regarding this restriction to be captured in USE_RESTRICTION_DETAIL. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 20 | Vehicle Type Restriction Indicator VEHICLE_TYPE_RESTRICTION_IND | VARCHAR2(3) | Yes | Yes/No indicator if certain vehicle types are restricted on this road segment. Specific details regarding this restriction to be captured in USE_RESTRICTION_DETAIL. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 21 | Other Restriction Indicator OTHER_RESTRICTION_IND | VARCHAR2(3) | Yes | Yes/No indicator if there are other restrictions on this road segment. Specific details regarding this restriction to be captured in USE_RESTRICTION_DETAIL. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 22 | Date Restriction Indicator DATE_RESTRICTION_IND | VARCHAR2(3) | Yes | Yes/No indicator if travel is restricted to certain dates on this road segment. Specific details regarding this restriction to be captured in USE_RESTRICTION_DETAIL. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 23 | Use Restriction Detail USE_RESTRICTION_DETAIL | VARCHAR2(160) | No | Brief details of the road use restriction (if applicable). Mandatory when STATUS is "Restricted". |
| 24 | Future Use Indicator FUTURE_USE_IND | VARCHAR2(3) | Yes | Yes/No indicator if the road segment can be opened for future use once it is transferred to ministry responsibility. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 25 | Rehabilitation Year REHABILITATION_YEAR | NUMBER(4) | No | Year in which the road was declared rehabilitated according to caribou or other ESA species evaluation criteria or the year the rehabilitation criteria are expected to be met after documented achievement of naturalized state. Upon reaching the rehabilitation year, a Naturalized road changes to a Rehabilitated-C or Rehabilitated-ESA road in Closed State. Note: specific authorization is required to assign rehabilitation year, usually a supervisor on advice from management biologist. |
| 26 | Road Transfer Year ROAD_TRANSFER_YEAR | NUMBER(4) | No | Year in which the road responsibility was transferred back to MNRF or to a 3rd party. Valid value is a four-digit number that is equal to or greater than YEAR_CONSTRUCTED. |
| 27 | Road Closed Year ROAD_CLOSED_YEAR | NUMBER(4) | No | Year in which the road was closed. Valid value is a four-digit number that is equal to or greater than YEAR_CONSTRUCTED. |
| 28 | Year Closed Modifier YEAR_CLOSED_MODIFIER | VARCHAR2(12) | No | An optional modifier to the Road Closed Year attribute, described as follows: 1] Actual - the year is known. 2] Approximate - the year is not known for sure. 3] Before - the actual year is not known but it was sometime before the year entered. VALID VALUES: Actual Approximate Before |
| 29 | Maintenance Level MAINTENANCE_LEVEL | VARCHAR2(15) | Yes | The frequency with which maintenance activities are carried out on the road segment, described as follows: 1] Routine - road maintained on a continuous basis to keep expected condition constant. 2] Limited - maintenance is limited to things like maintaining water crossings required to keep the road passable. 3] Not Maintained - no official maintenance is carried out except for action to address environmental or public safety issues. DEFAULT: Not Maintained VALID VALUES: Routine Limited Not Maintained DEFAULT: 'Not Maintained' |
| 30 | Passable Indicator PASSABLE_IND | VARCHAR2(3) | Yes | Yes/No indicator if a 4x4 vehicle is able to drive the segment of road. DEFAULT: Yes VALID VALUES: Yes No DEFAULT: 'Yes' |
| 31 | Berm Indicator BERM_IND | VARCHAR2(3) | Yes | Yes/No indicator if a berm is used for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 32 | Gate Indicator GATE_IND | VARCHAR2(3) | Yes | Yes/No indicator if a gate is used for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 33 | Roadbed Planted Indicator ROADBED_PLANTED_IND | VARCHAR2(3) | Yes | Yes/No indicator if the roadbed has been planted over for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 34 | Private Road Indicator PRIVATE_ROAD_IND | VARCHAR2(3) | Yes | Yes/No indicator if a private road is used for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 35 | Roadbed Destroyed Indicator ROADBED_DESTROYED_IND | VARCHAR2(3) | Yes | Yes/No indicator if the roadbed has been destroyed for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 36 | Sign Indicator SIGN_IND | VARCHAR2(3) | Yes | Yes/No indicator if a sign is used for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 37 | Slashpile Indicator SLASHPILE_IND | VARCHAR2(3) | Yes | Yes/No indicator if a slashpile is used for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 38 | Water Crossing Removed Indicator WATER_CROSSING_REMOVED_IND | VARCHAR2(3) | Yes | Yes/No indicator if a water crossing has been removed for road access control or decommissioning. DEFAULT: No VALID VALUES: Yes No DEFAULT: 'No' |
| 39 | Surface Type SURFACE_TYPE | VARCHAR2(10) | Yes | The surface type of the road. DEFAULT: Unsurfaced Note: this attribute is to be dropped once Geometric Class is filled in. VALID VALUES: Paved Gravel Unsurfaced DEFAULT: 'Unsurfaced' |
| 40 | Number Of Lanes NUMBER_OF_LANES | VARCHAR2(3) | Yes | The number of lanes the road has. Note: this attribute is to be dropped once GEOMETRIC_CLASS is filled in. VALID VALUES: 1 1.5 2 |
| 41 | Year Constructed YEAR_CONSTRUCTED | NUMBER(4) | Yes | The year in which the road segment was originally constructed. If construction spans more than one year then the year value should reflect the year that the segment was first opened to vehicular traffic. |
| 42 | Year Constructed Modifier YEAR_CONSTRUCTED_MODIFIER | VARCHAR2(12) | Yes | A mandatory modifier to the Road Year Constructed attribute, described as follows: 1] Actual - the year is known. 2] Approximate - the year is not known for sure. 3] Before - the actual year is not known but it was sometime before the year entered. DEFAULT: Actual VALID VALUES: Actual Approximate Before DEFAULT: 'Actual' |
| 43 | Service Year SERVICE_YEAR | NUMBER(4) | No | The year in which the road segment last re-entered service. Valid value is a four-digit number that is equal to or greater than the Year Constructed. |
| 44 | Date Last Maintained DATE_LAST_MAINTAINED | DATE | No | The date of the last known maintenance performed on the road segment (if applicable). |
| 45 | Date Last Inspected DATE_LAST_INSPECTED | DATE | No | The last time the road segment was inspected (if applicable). |
| 46 | Year Decommissioned YEAR_DECOMMISSIONED | NUMBER(4) | No | The year in which the road segment was decommissioned (if applicable). There is a constraint forcing the value to be four digits in a logical range (i.e. 1990-2040) VALID VALUES: BETWEEN 1990 AND 2040 |
| 47 | Year Decommissioned Modifier YEAR_DECOMMISSIONED_MODIFIER | VARCHAR2(12) | No | Conditional Mandatory modifier to the Year Decommissioned attribute (if applicable), described as follows: 1] Actual - the year is known. 2] Approximate - the year is not known for sure. 3] Before - the actual year is not known but it was sometime before the year entered. VALID VALUES: Actual Approximate Before DEFAULT: 'Actual' |
| 48 | General Comments GENERAL_COMMENTS | VARCHAR2(2000) | No | This field should be used to capture all relevant notes for the segment in question. Including reasons for changes in attributes, past attribute values, management considerations, issues that have come up, or issues that the segment has been involved in. |
| 49 | Geometry Update Datetime GEOMETRY_UPDATE_DATETIME | DATE | No | Date/time the geometry was created or last modified in the source database. |
| 50 | Effective Datetime EFFECTIVE_DATETIME | DATE | Yes | Date/time the record was created or last modified in the source database. |
| 51 | System Datetime SYSTEM_DATETIME | DATE | Yes | Date/time the record was loaded into or last modified in the Geospatial Ontario (GEO) database. DEFAULT: SYSDATE |
| 52 | SHAPE SHAPE | SDO_GEOMETRY | No | Geometry attribute. |