Quick Start Guide:

How to Order Ontario Imagery

# Overview

Ordering imagery from Land Information Ontario (LIO) requires:

- A completed Imagery Order Form.

- A list of required FileNames (MS Excel, CSV, Notepad)

Ortho tile                        Stereo line

# Quick Links

Compiling a FileName List	2

Imagery Orders Web App	2

Select by Point, Polygon or Lasso	2

Spatial Query	4

Desktop GIS Software	6

Submitting Your Order	6

The information in this document may not be compatible with assistive technologies. For an alternative format please contact imagery@ontario.ca or call LIO Support at 705-772-5891.

©King’s Printer for Ontario, 2024

# Compiling a FileName List

A list of ortho tiles or stereo frames/lines must be included with your completed Imagery Order Form.

Compile the list using:

- The Imagery Orders Web App (for select projects); or,

- Your own desktop GIS Software (i.e., ESRI ArcGIS or QGIS)

## Imagery Orders Web App

- Visit the Imagery Orders Web App

- Select the tiles that intersect your area of interest using:

  - Point, Polygon, or Lasso tools; or,

  - Spatial query with your area of interest

Tip: See instructions for selecting ortho tiles or stereo frames using the app, below.

- Export selection to CSV file.

### Select by Point, Polygon or Lasso

- Click the ‘Select’ widget in the upper left corner of the app.

- Select your selecting tool from the list of options (point, polygon, or lasso).

- Select the product using the checkbox.

- Perform the selection on the map based on your area of interest.

Tip: Make multiple selections by holding down the Shift key.

- Click the ellipsis button beside the product name.

- Select ‘Export to CSV file.’

### Spatial Query

- Click the ‘Add Data’ widget in the upper left corner of the app.

- Add your area of interest to the map. Do this by adding data from ArcGIS Online, a URL, or a File (.shp, CSV, KML, GPX or GeoJSON).

- Click the ‘Query’ widget in the upper left corner of the app.

- From the list of Tasks, select the project and product type.

- Ensure the spatial filters are set up to intersect the layer that represents your area of interest. You can also apply a search distance or buffer as required.

- Click Apply.

- Click the ellipsis button beside the query results.

- Select ‘Export to CSV file.’

## Desktop GIS Software

- Visit Ontario Geohub and click on the Ontario Imagery Program button to see a program overview, or browse all imagery collections using the search bar using keyword “imagery.”

- Download the corresponding ortho or stereo index (shapefile format) for your project. Click ‘Read More’ to reveal the rest of the page and the index is under ‘Available Products.’

- Using your desktop GIS software and the index(es) downloaded in Step 2, compile a list of FileNames for your area of interest and export to a shapefile or .dbf file.

# Submitting Your Order

Submit your tile/stereo list(s) and completed Imagery Order Form to imageryorders@ontario.ca.  A LIO Support Officer will contact you within two (2) business days regarding your order to discuss pricing and delivery options.