Final Report

For project:

Cataraqui LiDAR

Prepared for:

Ontario Ministry of Natural Resources

2nd floor, North Tower, 300 Water Street,

Peterborough, ON, K9J 3C7

Prepared by:

Airborne Imaging

2700 – 61 Avenue SE

Calgary, AB T2C 4V2

April, 2025

Contents

Introduction	1

Project Schedule & Personnel	2

LiDAR System & Flight Parameters	2

Project Control / Datum	5

Control Network	5

Check Points	6

LiDAR Data Processing	9

Calibration Procedures	9

Data Tiling and Classification	9

Point Cloud Manipulation	10

Water Classification & Hydro-Flattening	10

Class definitions	11

Deliverables	12

Vertical Accuracy Assessment	13

Conclusion	14

Appendix A - Control Points – Published Coordinates	15

Appendix B – RTK Survey & Control Check	16

Appendix C – Published Station Coordinate Sheets	17

# Introduction

The Ontario Ministry of Natural Resources and Forests contracted Airborne Imaging to acquire and deliver digital elevation data derived from airborne LiDAR (Light Detection and Ranging) to cover various regions in Ontario.

This report covers the Cataraqui area, collected during the fall of 2024. The report focuses on LiDAR acquisition details, such as flight parameters, project control, ground truthing results, data processing technique and deliverables.

The contracted area for the Cataraqui area is 570 km².

# Project Schedule & Personnel

The ground control occurred in November 2024. The surveyor collecting ground truth points during this period was Alistair Lornie. The Lidar data was collected in one mission on November 3, 2024. See table below for the mission’s flight parameters.

A typical Lidar acquisition crew consisted of one Lidar Operator and one Pilot.

# LiDAR System & Flight Parameters

A Riegl VQ-1560ii was used to collect the Lidar data. The system was installed in Revolution Flight’ Cessna Caravan call sign N331RF.

Below is a table of the lifts with their flight parameters.

Table 1: Project Parameters for the Cataraqui area.

| Riegl VQ1560-ii | Flight plan |
| --- | --- |
| Altitude | 1650m AGL |
| Field of View (FOV) | 60° |
| Air Speed | 160 Kts |
| Pulse Rate | 2100 kHz |
| Sidelap | 20% |
| Point Density | > 8 pts/m2 |

The flight lines were designed optimally to maximize time online versus time in turn.

Below is a map of the flight lines including cross-ties.

Below is a map of the flight line footprints.

Fall 2024 (Blue)

# Project Control / Datum

Several baselines were measured by GPS static occupation between Can-Net Continuously Operating Reference Stations (CORS), existing NRCan 3D Passive Control Monuments, as well as newly established points by Airborne Imaging. The existing control used for the Cataraqui area consisted of three Can-Net stations and two NRCan passive stations.

Appendix A contains a list of Control Points and their published coordinates.

Appendix B shows the coordinate list of deliverable check points.

Appendix C shows the published station coordinate sheets.

The datum used for processing the Ground Control and all subsequent Lidar data are the Nad83 CSRS (epoch 2010.0) horizontal datum and the CGVD2013 vertical datum (CGG2013 undulation model).

# Control Network

The ground control survey was collected using a combination Trimble R8 Model 3 receivers and Hemisphere S631 receivers. The Trimble receivers were employed in establishing control and collecting kinematic cross sections and the Hemisphere receivers were used for RTK collection. The ground control data was processed using the NovAtel Waypoint software GraftNet and GrafNav.

First, to establish control in the area, baselines were measured via GPS observations between a network of CanNet’s Continuously Operating Reference Stations (CORS), NRCan 3D Passive Control Monuments, and Airborne’s intermediate base (2022-71), which was established by the ground crew on the day of collection. See below for the control network established in the Cataraqui AOI.

Datasheets for all published Control Monuments can be found in Appendix A at the end of this report. A PPP Solution for the observed base 2022-71 was computed using NRCan’s PPP service and used as an a-priori value for processing the network. It is included in Appendix B.

Airborne Imaging collected ground control to provide a vertical accuracy assessment for the deliverable LiDAR outputs. Following ASPRS guidelines, Airborne planned to collect and deliver 30 control points spread throughout the AOI. Of those 30 points, 20 would be Non-Vegetated Vertical Accuracy (NVA) points, and 10 would be Vegetated Vertical Accuracy (VVA) points. Additionally, kinematic cross-sections were collected to calibrate the LiDAR data in preliminary processing and assess the need for applying vertical shifts. Pictures of observed control monuments, as well as NVA and VVA control points are included with the submission of this report.

Below is an overview map of the control stations.

Figure 1 Cataraqui Control Network

# Check Points

RTK GPS

The NVA and VVA control points were collected via Real-Time Kinematic Survey, using hemisphere S631 GNSS receivers.  Using 30 second observations, Real Time Kinematic are individual points surveyed on open terrain, mounted on a pole while receiving real time differential corrections from the GPS receiver positioned over a nearby control point (2022-71). These points are used just for the vertical accuracy assessment of the Lidar point cloud and were not used to determine any vertical shift applied to the point cloud.

The ground crews collected a total of 22 NVA points and 13 VVA points. In processing, 5 NVA points and 3 VVA points were determined to be unsuitable and were subsequently removed from the dataset. Pictures of the final delivered RTK points (as well as the existing control) are provided with this report in a folder included with the submission of this report.

Figure 2 Cataraqui NVA points

Figure 3 Cataraqui VVA points

Trajectory Processing

The aircraft trajectory positioning for the missions over the Cataraqui area were processed using the Precise Point Positioning (PPP) technique.

PPP is a positioning technique that removes and models GNSS system errors via precise satellite orbit data and clock corrections that are applied to the trajectory post mission, providing centimeter level positional accuracy without the use of ground base stations.

The TerraPOS software by Terratec was used to postprocess the PPP solutions to produce final kinematic trajectories accurate to 5 centimeters for each flight mission.

The horizontal datum used is Nad83 CSRS UTM zone 18 with the CGVD2013 vertical datum (CGG2013 undulation model).

# LiDAR Data Processing

## Calibration Procedures

After each mission, the field project manager creates the preliminary calibrated point cloud with each sensor’s set boresight values for each mission. Proprietary QC tools using the Lastools software are run on each mission to check for coverage, relative accuracy, point density, etc… If any data is found to be not meeting specifications, or coverage missing, the field crew is immediately notified and the areas affected reflown as soon as possible.

This preliminary calibrated point cloud is inspected in conjunction with the kinematic ground truth points to apply bulk shifts if necessary and further refine the calibration by applying corrections to the preliminary calibrated strips. The refined calibration is done using Bayesmap’s StripAlign software. Intensity values are also normalized by mission to be 16 bits (0 to 65535).

## Data Tiling and Classification

The raw LiDAR data is processed into calibrated point cloud strips corresponding to flight lines. For ease of data manipulation, the strips are combined and saved in tiles of 1 km by 1 km. The format the strips and tiles are saved in is .Laz version 1.4. The tiling scheme is based on the UTM18 projection at the even kilometers.

The tiled point cloud is then processed with a series of algorithms to separate the “most-likely” ground returns from other returns. The point classification is done using a product by the TerraSolid software running on MicroStation Connect called TerraScan and TerraModeler.

In addition to our regular field QC and calibration procedures, we also generate various raster products using the Lastools software to assess coverage and calibration accuracy.

## Point Cloud Manipulation

The TerraScan software uses macros that are set-up to measure the angles and distances between points to determine what classification a point should be: ground, vegetation, noise, other.  The angle and distance values in the macros can be adjusted to be more or less aggressive with the classification of points by varying the incidence angles and estimated distances among neighboring points.  The lower points are generally classified as ground returns, with the points above classified as undefined (class 1). Outliers and noise described in the table below are normally clouds, outliers or atmospheric noise in the data that are separated into above or below the ground level (high point or low points).

After the automated macro is run to determine classes, a systematic visual inspection is performed to fine tune the ground definition.  To better understand areas for improvement for the ground surface, the points that are classified as ground are used to create viewable TINs and grid surfaces.  These surfaces are inspected for areas that appear rough, artificially flattened or truncated, no data areas, or have other viewable errors.

In cleaning up ground points, the focus is concentrated in areas where few ground points have been left in the bare earth model and the ground appears rough or lower and flatter than it may be in reality.  The scarcity of ground points may be a result from no penetration through a dense vegetation layer, water bodies, low reflectivity objects, or too aggressive values with the macro.  A manual/visual inspection of these areas plays a major role in resolving any issues or irregularities with the bare earth model.

Once the first edits are completed, a one-meter grid of the ground surface is re-exported and hill shade images produced to QC the edited ground surface. This QC step is done iteratively as many times as necessary. Once the point cloud data has reached a satisfactory level of classification accuracy, the final deliverables are produced.

## Water Classification & Hydro-Flattening

Once the ground class has reached the final level of classification accuracy, the water delineation process is initiated. The rivers and water bodies are digitized as break lines according to USGS specifications of a minimum of 8000 m² for water bodies and 30 m nominal width for streams and rivers. Ponds and lakes have their elevations set constant to just below ground. Stream and river hydro lines are collected using the downstream method. Once all hydro lines have been collected for an area, the ground points inside the hydro lines are classified to class 9 – Water. The ground points outside of the hydro lines but within 35 cm are classified to class 20 – Ignored ground (near breaklines). Using the TIN of class 2 (ground points), the hydro lines are used as constraints to generate the final 50 cm hydro-flattened DEMs in ESRI Ascii format, then converted to 32bit Geotiff format with the Global Mapper software.

## Class definitions

The classification follows the ASPRS classification codes. The table below shows the classes for the delivered point cloud.

Table 2: ASPRS Point Classes

| Classification Value | Description |
| --- | --- |
| 1 | Unclassified |
| 2 | Ground |
| 6 | Building |
| 7 | Low Points |
| 9 | Water |
| 17 | Bridge Decks |
| 18 | High Points |
| 20 | Ignored Ground (breakline proximity) |
| 21 | Snow |
| 22 | Temporal exclusion |

# Deliverables

The data was delivered at the end of March 2025.

See below for a table of the folder structure and description of each deliverable.

DIFF

50cm rasters of flight lines differences, 1 X 1 km tiles in 32bit Geotiff format

e.g. 1km183850494302024LCATARAQUI_diff.tif

DTM

Hydro-flattened 50cm Bare Earth DEMs, 1 X 1 km tiles in 32bit Geotiff format

e.g. 1km183850494302024LCATARAQUI.tif

METADATA

Metadata files for Breaklines, DEM, LAS and Project in .XML format

e.g. CATARAQUI_2024_Lidar_metadata_Breaklines.xml

POINTCLOUD

Point cloud data, 1 X 1 km tiles in .LAZ format 1.4

e.g. 1km183850494302024LCATARAQUI.laz

PROJECT_POLYGON

Shape files of: Lift_metadata, Hydro_breaklines, Project_extent & Tile_index

e.g. Mission_5124308a_z18.shp

# Vertical Accuracy Assessment

Non Vegetated Vertical Accuracy (NVA)

Below is a summary of the statistics between the point cloud TIN from the ground class and the NVA points surveyed on flat hard surfaces:

| Number of points: | 17 |  |
| --- | --- | --- |
| Average dz: | -0.003 | m |
| Minimum dz: | -0.058 | m |
| Maximum dz: | +0.034 | m |
| Average magnitude: | 0.022 | m |
| Root mean square: | 0.027 | m |
| Std deviation: | 0.028 | m |
| NVA Vertical Accuracy (95%): | 0.053 | m |

Unfortunately, three NVA points were found to be unreliable and had to be rejected. This puts us slightly below the minimum of 20 points.

For a complete listing of the RTK points and their differences, please refer to the provided spreadsheet named "Cataraqui_NVA_Accuracy_Final_z18.xlsx".

As seen in the NVA comparison above, the accuracy observed of 0.088 m meets the project requirement of QL0 NVA vertical accuracy of 9.8 cm (95%).

Vegetated Vertical Accuracy (VVA)

The elevations of the points surveyed in various vegetated land cover types were compared to the Lidar ground surface. Since the elevation differences for this type of accuracy does not follow a normal distribution, the percentile method is used and not the 95% confidence level. For the 10 VVA points surveyed, sorting the absolute differences from the smallest to the largest gives the accuracy at the 95th percentile (using the linear interpolation between closest ranks method). For this sample, the 9th and 10st largest difference are respectively 16.8 and 29.4 cm. The small number of points for this sample (10) and the highest difference of 29.4 cm makes the VVA accuracy higher than expected. Nine differences out of ten were 16.8 cm or better.

For this project, the VVA accuracy is therefore 23.7 cm.

This is above the required specifications of 15.0 cm (the QL0 VVA specification).

For a complete listing of the kinematic points and their differences, please refer to the provided spreadsheet named "Cataraqui_VVA_Accuracy_Final_z18.xlsx".

# Conclusion

This is the final report covering the Cataraqui Lidar project. Topics such as field operations, control, data processing and final accuracy results were explained. The project went well, acquired in one season and delivered on schedule. The non-vegetated vertical accuracy meets specifications and the vegetated accuracy is above the QL0 requirement.

# Appendix A - Control Points – Published Coordinates

Cataraqui - Control Station Coordinates

| Station | Easting | Northing | Height | Description |
| --- | --- | --- | --- | --- |
| 2022-71 | 404808.292 | 4932302.816 | 97.707 | Airborne Intermediate Base |
| 883071 | 430947.038 | 4924680.075 | 102.508 | NRCan 3D Passive Control Monument |
| 973001 | 406690.894 | 4910966.377 | 100.042 | NRCan 3D Passive Control Monument |
| BREW | 397949.459 | 4918319.978 | 140.474 | Can-Net CORS Station |
| KNG3 | 374864.450 | 4902364.950 | 118.298 | Can-Net CORS Station |
| PRT2 | 397972.592 | 4975373.480 | 154.199 | Can-Net CORS Station |

Nad83(CSRS), epoch 2010

UTM Zone 18

# Appendix B – RTK Survey & Control Check

| Point | Easting | Northing | Height |  | Point | Easting | Northing | Height |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2001 | 400905.866 | 4933465.562 | 133.954 |  | 3001 | 400906.149 | 4933447.140 | 132.179 |
| 2002 | 397754.417 | 4931204.367 | 144.306 |  | 3002 | 397744.486 | 4931207.507 | 144.126 |
| 2003 | 404243.351 | 4934075.467 | 110.444 |  | 3003 | 404182.213 | 4938612.158 | 132.450 |
| 2004 | 396191.183 | 4929281.405 | 143.540 |  | 3004 | 395188.208 | 4937031.017 | 123.370 |
| 2005 | 404219.006 | 4938612.238 | 133.435 |  | 3005 | 395660.300 | 4945203.834 | 129.686 |
| 2006 | 400640.894 | 4942913.560 | 133.587 |  | 3009 | 412943.172 | 4923481.912 | 97.242 |
| 2007 | 395195.652 | 4937041.156 | 124.548 |  | 3010 | 408560.299 | 4926101.653 | 88.691 |
| 2008 | 395651.660 | 4945189.512 | 129.838 |  | 3011 | 404982.310 | 4926525.586 | 101.404 |
| 2009 | 392271.355 | 4943557.623 | 129.976 |  | 3012 | 405283.192 | 4929003.910 | 116.206 |
| 2010 | 388825.895 | 4940333.783 | 122.698 |  | 3013 | 404807.990 | 4932305.954 | 97.593 |
| 2011 | 388331.850 | 4943986.499 | 142.758 |  |  |  |  |  |
| 2012 | 386267.492 | 4946947.973 | 175.540 |  |  |  |  |  |
| 2018 | 412946.015 | 4923474.227 | 97.928 |  |  |  |  |  |
| 2019 | 408577.503 | 4926108.482 | 88.522 |  |  |  |  |  |
| 2020 | 404501.900 | 4920653.541 | 92.916 |  |  |  |  |  |
| 2021 | 404989.363 | 4926498.358 | 100.920 |  |  |  |  |  |
| 2022 | 405272.514 | 4929012.613 | 116.603 |  |  |  |  |  |

# Appendix C – Published Station Coordinate Sheets