Final Report

For project:

Huron Georgian Bay LiDAR

Prepared for:

Ontario Ministry of Natural Resources

2nd floor, North Tower, 300 Water Street,

Peterborough, ON, K9J 3C7

Prepared by:

Airborne Imaging

2700 – 61 Avenue SE

Calgary, AB T2C 4V2

February 24, 2025

Contents

Introduction	1

Project Schedule & Personnel	2

LiDAR System & Flight Parameters	2

Project Control / Datum	4

Check Points	6

Trajectory Processing	9

LiDAR Data Processing	9

Calibration Procedures	9

Data Tiling and Classification	9

Point Cloud Manipulation	10

Water Classification & Hydro-Flattening	10

Class definitions	11

Deliverables	12

Vertical Accuracy Assessment	13

Conclusion	14

Appendix A - Control Points – Published Coordinates	15

Appendix B – RTK Survey & Control Check	16

Appendix C – Published Station Coordinate Sheets	26

# Introduction

The Ontario Ministry of Natural Resources and Forests contracted Airborne Imaging to acquire and deliver digital elevation data derived from airborne LiDAR (Light Detection and Ranging) to cover various regions in Ontario.

This report covers the Huron Georgian Bay area, collected over three seasons: fall of 2022, spring of 2023 and fall of 2023. The report focuses on LiDAR acquisition details, such as flight parameters, project control, ground truthing results, data processing technique and deliverables.

The contracted area for the Huron Georgian Bay area is 16,944.54 km².

# Project Schedule & Personnel

The ground control and Lidar acquisition occurred during the fall of 2022, spring of 2023 and fall of 2023. The Lidar data was collected in twenty-three missions. See table below for a list of missions. The surveyors collecting ground truth points during this period are Andrew Gruber, Tom Cooper and Alistair Lornie.

A typical Lidar acquisition crew consisted of one Lidar Operator and one Pilot. The project management was done remotely by Allyson Fox and Daniel Arteaga.

# LiDAR System & Flight Parameters

Either a Riegl VQ-1560ii or VQ-1560ii-S was used to collect the Lidar data. The systems were installed in KASI Aviation’s Piper Navajos (call signs C-GMEC, C-GKSV and C-GPBN).

Below is a table of the lifts with their flight parameters.

Table 1: Project Parameters for the Huron Georgian Bay area.

|  | Regular flight plan | Lower flight plan (Meaford area) |
| --- | --- | --- |
| Altitude | 1600m AGL | 900m AGL |
| Field of View (FOV) | 60° | 60° |
| Air Speed | 160 Kts | 160 Kts |
| Pulse Rate | 2100 kHz | 2000 kHz |
| Sidelap | 20% | 20% |
| Point Density | > 8 pts/m2 | > 8 pts/m2 |

The flight lines were designed optimally to maximize time online versus time in turn.

Below is a map of the flight lines including cross-ties.

Below is a map of the flight line footprints colored by season of acquisition.

Fall 2022 (RED):		collection of the majority of the project

Spring 2023 (BLUE):	collection of the Meaford & Borden areas plus a few lines reflown

Fall 2023 (ORANGE):	North Bruce area

# Project Control / Datum

Several baselines were measured by GPS static occupation between the Cannet stations and existing NRCan, cosine points as well as newly established points by Airborne Imaging. The existing control used consisted of thirteen Can-Net Continuous Reference Stations (CORS), five NRCan 3D passive stations, and three Ontario COSINE points.

Appendix A contains a list of Control Points and their published coordinates.

Appendix B shows the checks performed between Cannet stations and passive stations with redundant checks.

Appendix C shows the published station coordinate sheets.

The datum used and therefore all subsequent Lidar data are based on the Nad83 CSRS (epoch 2010.0) horizontal datum and the CGVD2013 vertical datum (CGG2013 undulation model).

Below is an overview map of the control stations.

# Check Points

Control check points were surveyed from different methods to use as vertical check and accuracy assessment. The kinematic method was used for calibration purposes and the RTK method for Non-Vegetated Vertical Accuracy Assessment (NVA) and Vegetated Vertical Accuracy Assessment (VVA).

Kinematic GPS

The first method called kinematic are profiles surveyed on open roads from a truck-mounted GPS receiver while driving. The post-processed kinematic coordinates, collected at an interval of one second are derived from a base station that is part of our control network.

Whenever possible, we try to select roads that are perpendicular to the flight line azimuth so the control points intersect as many Lidar strips as possible.

The output coordinates are analyzed and filtered to ensure only very reliable positions are kept. All points collected while moving slower than 2 meters per second are rejected. This removes all points when the truck was stationary that could skew the statistics.

The following points are also rejected:

- Points with a Quality Level worse than 1.

- Points with a forward/reverse vertical separation worse than 3 cm.

- Points with less than two solutions (forward & reverse).

- Points with a standard deviation worse than 2sigma.

Many sections of road were surveyed throughout the project giving us thousands of surveyed points on the road. We estimate that the vertical accuracy of these points is better than 5 cm.

During the calibration of the Lidar strips, the Lidar strips are matched vertically to these reliable kinematic points. These points are not used for the vertical accuracy assessment of the Lidar point cloud, but rather just for calibration monitoring and shifting. For that reason, the results of the kinematic surveys will not be discussed in this report.

RTK GPS

In order to meet the recommended ASPRS minimum number of points for a 17,000 square kilometer project, RTK points were surveyed in areas distributed throughout the project.

The second method called Real Time Kinematic are individual points surveyed on open terrain. Our surveyor collected GPS data mounted on a pole while receiving real time differential corrections from the GPS receiver positioned over a nearby control point.

248 points were surveyed on the Huron Georgian Bay project. Of these, 151 points were over non-vegetated ground conditions for NVA accuracy reporting and 97 of them were over vegetated areas for VVA accuracy reporting.

Pictures of these RTK points (as well as the existing control) are provided with this report in a folder called “HuronGeorgianBay_Control_Point_Pictures”.

These points are used just for the vertical accuracy assessment of the Lidar point cloud and were not used to determine any vertical shift applied to the point cloud.

See the green triangles on the map below.

Blue = project outline

Green = Fast Static (NVA & VVA)

# Trajectory Processing

The aircraft trajectory positioning for the missions over the Huron Georgian Bay area were processed using the Precise Point Positioning (PPP) technique.

PPP is a positioning technique that removes and models GNSS system errors via precise satellite orbit data and clock corrections that are applied to the trajectory post mission, providing centimeter level positional accuracy without the use of ground base stations.

The TerraPOS software by Terratec was used to postprocess the PPP solutions to produce final kinematic trajectories accurate to 5 centimeters for each flight mission.

The horizontal datum used is Nad83 CSRS UTM zone 17 with the CGVD2013 vertical datum (CGG2013 undulation model).

# LiDAR Data Processing

# Calibration Procedures

After each mission, the field project manager creates the preliminary calibrated point cloud with each sensor’s set boresight values for each mission. Proprietary QC tools using the Lastools software are run on each mission to check for coverage, relative accuracy, point density, etc… If any data is found to be not meeting specifications, or coverage missing, the field crew is immediately notified and the areas affected reflown as soon as possible.

This preliminary calibrated point cloud is inspected in conjunction with the kinematic ground truth points to apply bulk shifts if necessary and further refine the calibration by applying corrections to the preliminary calibrated strips. The refined calibration is done using Bayesmap’s StripAlign software. Intensity values are also normalized by mission to be 16 bits (0 to 65535).

# Data Tiling and Classification

The raw LiDAR data is processed into calibrated point cloud strips corresponding to flight lines. For ease of data manipulation, the strips are combined and saved in tiles of 1 km by 1 km. The format the strips and tiles are saved in is .Laz version 1.4. The tiling scheme is based on the UTM17 projection at the even kilometers.

The tiled point cloud is then processed with a series of algorithms to separate the “most-likely” ground returns from other returns. The point classification is done using a product by the TerraSolid software running on MicroStation Connect called TerraScan and TerraModeler.

In addition to our regular field QC and calibration procedures, we also generate various raster products using the Lastools software to assess coverage and calibration accuracy.

# Point Cloud Manipulation

The TerraScan software uses macros that are set-up to measure the angles and distances between points to determine what classification a point should be: ground, vegetation, noise, other.  The angle and distance values in the macros can be adjusted to be more or less aggressive with the classification of points by varying the incidence angles and estimated distances among neighboring points.  The lower points are generally classified as ground returns, with the points above classified as undefined (class 1). Outliers and noise described in the table below are normally clouds, outliers or atmospheric noise in the data that are separated into above or below the ground level (high point or low points).

After the automated macro is run to determine classes, a systematic visual inspection is performed to fine tune the ground definition.  To better understand areas for improvement for the ground surface, the points that are classified as ground are used to create viewable TINs and grid surfaces.  These surfaces are inspected for areas that appear rough, artificially flattened or truncated, no data areas, or have other viewable errors.

In cleaning up ground points, the focus is concentrated in areas where few ground points have been left in the bare earth model and the ground appears rough or lower and flatter than it may be in reality.  The scarcity of ground points may be a result from no penetration through a dense vegetation layer, water bodies, low reflectivity objects, or too aggressive values with the macro.  A manual/visual inspection of these areas plays a major role in resolving any issues or irregularities with the bare earth model.

Once the first edits are completed, a one-meter grid of the ground surface is re-exported and hill shade images produced to QC the edited ground surface. This QC step is done iteratively as many times as necessary. Once the point cloud data has reached a satisfactory level of classification accuracy, the final deliverables are produced.

# Water Classification & Hydro-Flattening

Once the ground class has reached the final level of classification accuracy, the water delineation process is initiated. The rivers and water bodies are digitized as break lines according to USGS specifications of a minimum of 8000 m² for water bodies and 30 m nominal width for streams and rivers. Ponds and lakes have their elevations set constant to just below ground. Stream and river hydro lines are collected using the downstream method. Once all hydro lines have been collected for an area, the ground points inside the hydro lines are classified to class 9 – Water. The ground points outside of the hydro lines but within 35 cm are classified to class 20 – Ignored ground (near breaklines). Using the TIN of class 2 (ground points), the hydro lines are used as constraints to generate the final 50 cm hydro-flattened DEMs in ESRI Ascii format, then converted to 32bit Geotiff format with the Global Mapper software.

# Class definitions

The classification follows the ASPRS classification codes. The table below shows the classes for the delivered point cloud.

Table 2: ASPRS Point Classes

| Classification Value | Description |
| --- | --- |
| 1 | Unclassified |
| 2 | Ground |
| 6 | Building |
| 7 | Low Points |
| 9 | Water |
| 17 | Bridge Decks |
| 18 | High Points |
| 20 | Ignored Ground (breakline proximity) |
| 21 | Snow |
| 22 | Temporal exclusion |

# Deliverables

Since the project was collected over three seasons, multiple deliverables were made as the data was ready. The majority of the Huron Georgian Bay area was collected in the fall of 2022 and delivered in March 2023. The areas over the Meaford and Borden military bases were collected in the spring of 2023 and delivered in October 2023 (with a few reflights). The North Bruce peninsula was collected in the fall of 2023 and delivered in March 2024.

See below for a table of the folder structure and description of each deliverables.

DTM

Hydro-flattened 50cm Bare Earth DEMs, 1 X 1 km tiles in 32bit Geotiff format

e.g. 1km174390487802022LHURONGEORGIANBAY.tif

METADATA

Metadata files for Breaklines, DEM, LAS and Project in .XML format

e.g. HURONGEORGIANBAY2022_Lidar_metadata_LAS.xml

POINTCLOUD

Point cloud data, 1 X 1 km tiles in .LAZ format 1.4

e.g. 1km174400484302022LHURONGEORGIANBAY.laz

PROJECT_POLYGON

Shape files of: Lift_metadata, Hydro_breaklines, Project_extent & Tile_index

e.g. Mission_5122298a.shp

# Vertical Accuracy Assessment

Non Vegetated Vertical Accuracy (NVA)

Below is a summary of the statistics between the point cloud TIN from the ground class and the NVA points surveyed on flat hard surfaces:

| Number of points: | 151 |  |
| --- | --- | --- |
| Average dz: | -0.007 | m |
| Minimum dz: | -0.144 | m |
| Maximum dz: | +0.122 | m |
| Average magnitude: | 0.035 | m |
| Root mean square: | 0.042 | m |
| Std deviation: | 0.042 | m |
| NVA Vertical Accuracy (95%): | 0.083 | m |

For a complete listing of the RTK points and their differences, please refer to the provided spreadsheet named " HuronGeorgianBay_NVA_Accuracy_Final.xlsx".

As seen in the NVA comparison above, the accuracy observed of 0.083 m meets the project requirement of QL0 NVA vertical accuracy of 9.8 cm (95%).

Vegetated Vertical Accuracy (VVA)

The elevations of the points surveyed in various vegetated land cover types were compared to the Lidar ground surface. Since the elevation differences for this type of accuracy does not follow a normal distribution, the percentile method is used and not the 95% confidence level. For the 97 VVA points surveyed, sorting the absolute differences from the smallest to the largest gives the accuracy at the 95th percentile (using the linear interpolation between closest ranks method). For this sample, the 92nd and 93rd largest difference are respectively 15.9 and 16.0 cm.

For this project, the VVA accuracy is therefore 15.92 cm.

This is slightly above the required specifications of 15.0 cm (the QL0 VVA specification).

For a complete listing of the kinematic points and their differences, please refer to the provided spreadsheet named " HuronGeorgianBay_VVA_Accuracy_Final.xlsx".

# Conclusion

This is the final report covering the Huron Georgian Bay Lidar project. Topics such as field operations, control, data processing and final accuracy results were explained. The acquisition for this leaf off, snow off project was acquired over three seasons due to its size, poor weather and short acquisition windows between leaf off and the accumulation of snow (or vice versa, melting of snow and leaf on status). The non-vegetated vertical accuracy meets the QL0 requirement, and the vegetated vertical accuracy is slightly above the requirement (which is an indication of how well the laser penetrated in vegetated areas and is not an indication of the accuracy of the point cloud).

# Appendix A - Control Points – Published Coordinates

| Station | Easting | Northing | h | H (CGVD2013) | Status | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| STFD | 503007.702 | 4801423.870 | 332.900 | 367.775 | Published | Can-Net |
| LSW2 | 504329.108 | 4843266.883 | 359.647 | 394.487 | Published | Can-Net |
| GLP2 | 556066.788 | 4821156.748 | 315.118 | 350.529 | Published | Can-Net |
| GODE | 442705.012 | 4843795.305 | 194.830 | 229.801 | Published | Can-Net |
| WALK | 486867.066 | 4886684.321 | 264.024 | 299.317 | Published | Can-Net |
| OWEN | 504716.944 | 4935583.079 | 149.744 | 185.391 | Published | Can-Net |
| FLSH | 539535.287 | 4900727.811 | 452.804 | 487.838 | Published | Can-Net |
| PLTG | 470062.390 | 4921788.449 | 177.892 | 213.802 | Published | Can-Net |
| COL2 | 559821.583 | 4927419.998 | 166.252 | 201.720 | Published | Can-Net |
| ORVL | 571296.241 | 4860917.472 | 428.209 | 463.556 | Published | Can-Net |
| BAR2 | 606039.360 | 4910049.139 | 252.634 | 288.262 | Published | Can-Net |
| MDL2 | 588247.742 | 4955539.312 | 169.844 | 205.237 | Published | Can-Net |
| ORIL | 625681.385 | 4940970.863 | 195.767 | 231.065 | Published | Can-Net |
| TOBY | 447993.351 | 5011688.626 | 144.892 | 181.525 | Published | NRCan CACS |
| 81D8270 | 486812.506 | 4851604.699 | 309.522 | 344.434 | Published | NRCan 3D Passive |
| 73D8006 | 516506.445 | 4879258.199 | 339.756 | 374.771 | Published | NRCan 3D Passive |
| 72U219 | 488727.546 | 4956939.366 | 176.581 | 212.559 | Published | NRCan 3D Passive |
| 973007 | 450106.980 | 5008044.382 | 172.744 | 209.304 | Published | NRCan 3D Passive |
| M053009 | 562019.985 | 4928776.088 | 142.915 | 178.417 | Published | NRCan 3D Passive |
| 820000179 | 532410.334 | 4939891.541 | 142.468 | N/A | Published | COSINE |
| 820048009 | 532228.114 | 4903879.148 | 399.165 | 434.190 | Published | COSINE |
| 819720733 | 451547.207 | 4892770.300 | 188.251 | N/A | Published | COSINE |

# Appendix B – RTK Survey & Control Check

| Point # | Easting | Northing | H (Ellip) | H (Geoid) | JD (2022) | Comment | Etie | Ntie | ElevTie |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CSTD_On | 503007.702 | 4801423.870 | 332.900 | 367.775 | 325 | Can-Net |  |  |  |
| 4000 | 501718.557 | 4818901.326 | 335.133 | 369.944 | 325 | Paved road |  |  |  |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 325 | Can-Net |  |  |  |
| 4001 | 498712.991 | 4823065.725 | 328.382 | 363.185 | 325 | Paved road |  |  |  |
| 4002 | 497205.769 | 4829400.738 | 324.233 | 359.039 | 325 | Paved road |  |  |  |
| 4003 | 488168.201 | 4841520.483 | 320.118 | 354.969 | 325 | Paved road |  |  |  |
| 4004 | 482450.226 | 4847044.505 | 306.971 | 341.866 | 325 | Paved road |  |  |  |
| 4005 | 486848.482 | 4851608.069 | 308.457 | 343.369 | 325 | No picture |  |  |  |
| CGLP_On | 556066.788 | 4821156.748 | 315.118 | 350.529 | 325 | Can-Net |  |  |  |
| 1935_chk01 | 535814.728 | 4817824.360 | 301.790 | 336.954 | 325 | Aii 1935-01 | -0.007 | 0.009 | -0.045 |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 326 | Can-Net |  |  |  |
| 3000 | 506332.248 | 4831358.814 | 350.807 | 385.621 | 326 | Low brush |  |  |  |
| 4005A | 506329.525 | 4831351.626 | 351.340 | 386.153 | 326 | Paved road |  |  |  |
| 4006 | 504542.469 | 4841665.219 | 352.544 | 387.377 | 326 | Hard packed |  |  |  |
| 4007 | 505007.316 | 4841265.547 | 355.948 | 390.779 | 326 | Paved road |  |  |  |
| 4008 | 505977.767 | 4855972.395 | 340.223 | 375.120 | 326 | Paved road |  |  |  |
| 3001 | 505967.848 | 4855996.430 | 340.222 | 375.118 | 326 | Short grass |  |  |  |
| 4009 | 519009.686 | 4854635.776 | 385.235 | 420.125 | 326 | Parking lot |  |  |  |
| 2000 | 509741.749 | 4862690.839 | 347.460 | 382.390 | 326 | Paint line |  |  |  |
| 4010 | 498816.772 | 4861645.392 | 329.297 | 364.230 | 326 | Hard packed |  |  |  |
| 4011 | 494474.355 | 4847275.582 | 321.497 | 356.368 | 326 | Parking lot |  |  |  |
| CGLP_On | 556066.788 | 4821156.748 | 315.118 | 350.529 | 326 | Can-Net |  |  |  |
| 1935_Chk02 | 535814.728 | 4817824.340 | 301.804 | 336.968 | 326 | Aii 1935-01 | -0.007 | -0.011 | -0.031 |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 327 | Can-Net |  |  |  |
| 4012 | 477087.533 | 4838375.747 | 320.684 | 355.541 | 327 | Paved road |  |  |  |
| 3002 | 477081.869 | 4838363.525 | 319.880 | 354.737 | 327 | Crop |  |  |  |
| 4013 | 464981.400 | 4842112.588 | 293.253 | 328.142 | 327 | Parking lot |  |  |  |
| 4014 | 459388.811 | 4834891.542 | 259.927 | 294.796 | 327 | Paved road |  |  |  |
| 3003 | 459394.270 | 4834905.633 | 258.978 | 293.846 | 327 | Crop |  |  |  |
| 3004 | 451476.750 | 4832749.417 | 226.177 | 261.046 | 327 | Under canopy |  |  |  |
| 4015 | 451471.487 | 4832767.070 | 226.380 | 261.250 | 327 | Parking lot |  |  |  |
| 4016 | 442898.963 | 4844322.730 | 181.514 | 216.489 | 327 | Parking lot |  |  |  |
| CGOD | 442705.012 | 4843795.305 | 194.83 | 229.801 | 327 | Can_net |  |  |  |
| 3005 | 442890.588 | 4844324.778 | 181.317 | 216.292 | 327 | Short grass |  |  |  |
| 4017 | 457299.372 | 4846545.408 | 260.796 | 295.726 | 327 | Hard packed |  |  |  |
| 4018 | 467627.645 | 4846907.275 | 305.561 | 340.475 | 327 | Paved road |  |  |  |
| CGLP_On | 556066.788 | 4821156.748 | 315.118 | 350.529 | 327 | Can-Net |  |  |  |
| 1935_CHk03 | 535814.731 | 4817824.352 | 301.821 | 336.984 | 327 | Aii 1935-01 | -0.004 | 0.001 | -0.015 |
| CGOD | 442705.012 | 4843795.305 | 194.83 | 229.801 | 328 | Can_net |  |  |  |
| 4018A | 449264.978 | 4851430.041 | 217.448 | 252.451 | 328 | No picture |  |  |  |
| 3006 | 449259.126 | 4851437.988 | 216.873 | 251.876 | 328 | Short grass |  |  |  |
| 3007 | 443753.629 | 4865170.794 | 172.469 | 207.681 | 328 | Tall grass |  |  |  |
| 4019 | 443751.776 | 4865172.554 | 172.482 | 207.694 | 328 | Paved road |  |  |  |
| 4020 | 443141.512 | 4873393.107 | 166.858 | 202.217 | 328 | Hard packed |  |  |  |
| 4021 | 443901.876 | 4885496.871 | 145.165 | 180.733 | 328 | Paved road |  |  |  |
| 4022 | 454317.768 | 4879581.112 | 212.595 | 247.965 | 328 | Paved road |  |  |  |
| 3008 | 454311.062 | 4879587.000 | 212.162 | 247.532 | 328 | Tall grass |  |  |  |
| 2001 | 458250.358 | 4867928.454 | 232.846 | 267.987 | 328 | Paint line |  |  |  |
| 4024 | 457711.304 | 4858846.281 | 260.369 | 295.397 | 328 | Paved road |  |  |  |
| 4025 | 473041.124 | 4856382.023 | 280.034 | 315.016 | 328 | Paved road |  |  |  |
| 4026 | 473040.127 | 4856379.832 | 280.101 | 315.082 | 328 | Paved road |  |  |  |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 328 | Can-Net |  |  |  |
| 4027 | 488255.053 | 4832011.254 | 320.710 | 355.530 | 328 | Hard packed |  |  |  |
| 1935_Chk04 | 535814.738 | 4817824.353 | 301.727 | 336.891 | 328 | Aii 1935-01 | 0.003 | 0.002 | -0.109 |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 329 | Can-Net |  |  |  |
| 81D8270 | 486812.489 | 4851604.741 | 309.517 | 344.428 | 329 | NRCan 81D270 | -0.017 | 0.042 | -0.005 |
| 4029 | 489916.428 | 4857819.905 | 308.380 | 343.316 | 329 | Paved road |  |  |  |
| 3009 | 489922.463 | 4857817.850 | 308.242 | 343.177 | 329 | Tall grass |  |  |  |
| 4030 | 491226.223 | 4868490.220 | 343.444 | 378.449 | 329 | Paved road |  |  |  |
| CWAK_On | 486867.066 | 4886684.321 | 264.024 | 299.317 | 329 | Can-Net |  |  |  |
| 4031 | 491741.945 | 4876925.061 | 285.460 | 320.571 | 329 | Hard packed |  |  |  |
| 3010 | 491735.413 | 4876932.703 | 285.431 | 320.543 | 329 | Short grass |  |  |  |
| 3011 | 479269.599 | 4882643.904 | 253.233 | 288.514 | 329 | Short grass |  |  |  |
| 4032 | 479266.620 | 4882644.701 | 253.309 | 288.590 | 329 | Paved road |  |  |  |
| 4033 | 477146.004 | 4871182.890 | 275.656 | 310.781 | 329 | Parking lot |  |  |  |
| 3012 | 477148.291 | 4871191.218 | 275.370 | 310.496 | 329 | Under canopy |  |  |  |
| 4034 | 463613.662 | 4876682.610 | 248.069 | 283.328 | 329 | Paved road |  |  |  |
| 4035 | 466642.917 | 4865494.940 | 274.340 | 309.421 | 329 | Paved road |  |  |  |
| 4036 | 478131.213 | 4865814.765 | 295.300 | 330.349 | 329 | Paved road |  |  |  |
| 3013 | 478129.086 | 4865802.869 | 295.245 | 330.293 | 329 | Tall grass |  |  |  |
| 3014 | 478129.268 | 4865803.078 | 295.235 | 330.284 | 329 | No picture |  |  |  |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 330 | Can-Net |  |  |  |
| 1935_02 | 510504.989 | 4853638.507 | 364.681 | 399.561 | 330 | Aii 1935-02 | -0.015 | 0.02 | -0.068 |
| 4037 | 510502.550 | 4853639.555 | 364.805 | 399.685 | 330 | Hard packed |  |  |  |
| CWAK_On | 486867.066 | 4886684.321 | 264.024 | 299.317 | 330 | Can-Net |  |  |  |
| 4038 | 499610.869 | 4870154.649 | 335.563 | 370.559 | 330 | Paved road |  |  |  |
| 4039 | 526473.583 | 4863549.403 | 408.898 | 443.826 | 330 | Hard packed |  |  |  |
| 4040 | 520423.972 | 4869460.417 | 378.302 | 413.256 | 330 | Hard packed |  |  |  |
| 3015 | 520456.285 | 4869456.108 | 377.808 | 412.762 | 330 | Short grass |  |  |  |
| 4041 | 510143.530 | 4867702.515 | 341.635 | 376.594 | 330 | Paved road |  |  |  |
| 73D8006 | 516506.428 | 4879258.209 | 339.711 | 374.726 | 330 | NRCan 73D8006 | -0.013 | 0.018 | -0.043 |
| 4042 | 516505.382 | 4879204.900 | 336.875 | 371.890 | 330 | No pic |  |  |  |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 331 | Can-Net |  |  |  |
| 4043 | 515971.251 | 4946963.866 | 195.296 | 230.924 | 331 | Paved road |  |  |  |
| 3016 | 515968.269 | 4946952.304 | 194.860 | 230.488 | 331 | Tall grass |  |  |  |
| 820000179 | 532410.328 | 4939891.562 | 142.488 | 178.010 | 331 | Cosine 820000179 | -0.006 | 0.021 | 0.02 |
| 3017 | 532399.609 | 4939883.193 | 142.658 | 178.180 | 331 | Short grass |  |  |  |
| 2002 | 532363.930 | 4939868.250 | 142.302 | 177.823 | 331 | Paint line |  |  |  |
| 4044 | 534162.051 | 4930085.842 | 387.530 | 422.858 | 331 | Paved road |  |  |  |
| 4045 | 536618.974 | 4915807.125 | 198.840 | 234.021 | 331 | Paved road |  |  |  |
| 4046 | 538016.085 | 4905520.652 | 389.025 | 424.104 | 331 | Paved road |  |  |  |
| CFLS_On | 539535.287 | 4900727.811 | 452.804 | 487.838 | 331 | Can-Net |  |  |  |
| 4047 | 532213.541 | 4903881.342 | 399.398 | 434.464 | 331 | Hard packed |  |  |  |
| 820048009 | 532228.122 | 4903879.142 | 399.130 | 434.197 | 331 | Cosine 820048009 | 0.008 | -0.006 | -0.035 |
| 4048 | 520874.809 | 4911485.255 | 362.655 | 397.816 | 331 | Paved road |  |  |  |
| 4049 | 524964.997 | 4919674.937 | 377.372 | 412.583 | 331 | Hard packed |  |  |  |
| 4050 | 520008.839 | 4932243.373 | 242.877 | 278.308 | 331 | Hard packed |  |  |  |
| 4051 | 508258.048 | 4936139.751 | 198.549 | 234.148 | 331 | Paved road |  |  |  |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 332 | Can-Net |  |  |  |
| 4052 | 482839.107 | 4964035.713 | 161.192 | 197.248 | 332 | Paved road |  |  |  |
| 3018 | 482832.330 | 4964041.291 | 160.890 | 196.947 | 332 | Tall grass |  |  |  |
| 4053 | 488841.782 | 4954107.075 | 150.406 | 186.371 | 332 | Paved road |  |  |  |
| 4054 | 491884.933 | 4963327.404 | 211.899 | 247.861 | 332 | Hard packed |  |  |  |
| 4055 | 495291.467 | 4967992.797 | 181.473 | 217.432 | 332 | Hard packed |  |  |  |
| 4056 | 491410.648 | 4969748.612 | 216.949 | 252.946 | 332 | Hard packed |  |  |  |
| 4057 | 487744.478 | 4968500.478 | 186.658 | 222.680 | 332 | Hard packed |  |  |  |
| 3019 | 487745.868 | 4968509.256 | 186.303 | 222.324 | 332 | Tall grass |  |  |  |
| 72U219 | 488727.555 | 4956939.388 | 176.562 | 212.540 | 332 | NRCan 72U219 | 0.009 | 0.022 | -0.019 |
| 4058 | 489963.191 | 4942152.420 | 183.517 | 219.403 | 332 | Paved road |  |  |  |
| 4059 | 499507.869 | 4945945.938 | 209.486 | 245.274 | 332 | Paved road |  |  |  |
| 4060 | 505446.710 | 4951924.726 | 210.709 | 246.474 | 332 | Paved road |  |  |  |
| 3020 | 505440.831 | 4951928.354 | 210.627 | 246.392 | 332 | Short grass |  |  |  |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 333 | Can-Net |  |  |  |
| 4061 | 507661.905 | 4923499.541 | 255.868 | 291.328 | 333 | Hard packed |  |  |  |
| 4062 | 508187.548 | 4912924.018 | 283.823 | 319.147 | 333 | Paved road |  |  |  |
| 4063 | 511578.528 | 4905716.177 | 313.092 | 348.296 | 333 | Hard packed |  |  |  |
| 4064 | 513397.370 | 4897293.949 | 291.563 | 326.698 | 333 | Hard packed |  |  |  |
| 3021 | 513385.347 | 4897311.172 | 291.373 | 326.508 | 333 | Tall grass |  |  |  |
| 4065 | 513696.282 | 4886668.111 | 311.570 | 346.649 | 333 | Paved road |  |  |  |
| 4066 | 525575.248 | 4888989.389 | 419.918 | 454.921 | 333 | Paved road |  |  |  |
| 4067 | 528543.395 | 4883458.790 | 432.210 | 467.191 | 333 | Hard packed |  |  |  |
| 3022 | 528548.307 | 4883461.387 | 431.846 | 466.826 | 333 | Short grass |  |  |  |
| 4068 | 530244.259 | 4873187.604 | 425.191 | 460.146 | 333 | Hard packed |  |  |  |
| 4069 | 538227.250 | 4873317.973 | 445.883 | 480.839 | 333 | Hard packed |  |  |  |
| 4070 | 540417.420 | 4884105.484 | 446.891 | 481.857 | 333 | Paved road |  |  |  |
| 4071 | 540888.394 | 4894229.347 | 446.101 | 481.095 | 333 | Paved road |  |  |  |
| 3023 | 540875.629 | 4894226.105 | 445.322 | 480.316 | 333 | Tall grass |  |  |  |
| 4072 | 548793.894 | 4900223.145 | 486.996 | 522.019 | 333 | Hard packed |  |  |  |
| 4073 | 547170.705 | 4906150.853 | 441.123 | 476.192 | 333 | Paved road |  |  |  |
| 3024 | 547175.112 | 4906155.191 | 440.976 | 476.045 | 333 | Tall grass |  |  |  |
| 820048009 | 532228.123 | 4903879.122 | 399.153 | 434.220 | 333 | Cosine 820048009 | 0.009 | -0.026 | -0.012 |
| CPTL_On | 470062.39 | 4921788.449 | 177.892 | 213.802 | 334 | Can-Net |  |  |  |
| 819720733 | 4892770.3 | 451547.196 | 188.282 | 223.917 | 334 | Cosine 819720733 | 0 | -0.011 | 0.031 |
| 4074 | 4892762.003 | 451540.467 | 187.552 | 223.187 | 334 | Paved road |  |  |  |
| 4075 | 4901736.086 | 456663.079 | 203.834 | 239.562 | 334 | Paved road |  |  |  |
| COL2_On | 559821.583 | 4927419.998 | 166.252 | 201.72 | 335 | Can-Net |  |  |  |
| 4076 | 542792.249 | 4934266.200 | 161.374 | 196.843 | 335 | Parking lot |  |  |  |
| 3025 | 542788.080 | 4934269.530 | 161.516 | 196.985 | 335 | Short grass |  |  |  |
| 4077 | 543677.956 | 4923599.115 | 276.255 | 311.538 | 335 | Paved road |  |  |  |
| 4078 | 556847.981 | 4915429.761 | 473.451 | 508.653 | 335 | Hard packed |  |  |  |
| 4079 | 556254.530 | 4907064.011 | 484.522 | 519.633 | 335 | Paved road |  |  |  |
| CFLS_On | 539535.287 | 4900727.811 | 452.804 | 487.838 | 335 | Can-Net |  |  |  |
| 3026 | 556251.112 | 4907062.099 | 484.468 | 519.579 | 335 | Short grass |  |  |  |
| 3027 | 564727.747 | 4896781.175 | 471.817 | 506.973 | 335 | Short grass |  |  |  |
| 4080 | 564732.140 | 4896781.749 | 471.916 | 507.072 | 335 | Parking lot |  |  |  |
| 4081 | 558661.656 | 4883779.015 | 472.524 | 507.563 | 335 | Paved road |  |  |  |
| CORL_On | 571296.241 | 4860917.472 | 428.209 | 463.556 | 335 | Can-Net |  |  |  |
| 4082 | 567607.373 | 4866642.667 | 444.209 | 479.453 | 335 | Hard packed |  |  |  |
| 3028 | 567602.727 | 4866657.543 | 443.978 | 479.222 | 335 | Short grass |  |  |  |
| 4083 | 581381.945 | 4873695.888 | 241.996 | 277.527 | 335 | Hard packed |  |  |  |
| 4084 | 581381.965 | 4873695.748 | 242.039 | 277.570 | 335 | Hard packed |  |  |  |
| 4085 | 581276.630 | 4881913.819 | 265.270 | 300.794 | 335 | Hard packed |  |  |  |
| 3029 | 581284.025 | 4881911.802 | 264.166 | 299.691 | 335 | Short brush |  |  |  |
| 2003 | 576865.964 | 4891126.318 | 277.834 | 313.254 | 335 | Paint line |  |  |  |
| CBA2_On | 606039.36 | 4910049.139 | 252.634 | 288.262 | 335 | Can-Net |  |  |  |
| 4086 | 576533.655 | 4900587.732 | 220.037 | 255.499 | 335 | Hard packed |  |  |  |
| 4087 | 564679.413 | 4913307.877 | 374.778 | 410.089 | 335 | Hard packed |  |  |  |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 335 | Can-Net |  |  |  |
| 820000179 | 532410.318 | 4939891.533 | 142.404 | 177.926 | 335 | Cosine 820000179 | -0.016 | -0.008 | -0.064 |
| COL2_On | 559821.583 | 4927419.998 | 166.252 | 201.72 | 336 | Can-Net |  |  |  |
| M053009 | 562027.958 | 4928768.132 | 142.938 | 178.440 | 336 | NRCan M053009 |  |  |  |
| 4088 | 562028.136 | 4928768.274 | 142.940 | 178.442 | 336 | Pad |  |  |  |
| 4089 | 579003.915 | 4924680.726 | 147.941 | 183.508 | 336 | Hard packed |  |  |  |
| 4090 | 591754.639 | 4924947.347 | 152.634 | 188.190 | 336 | Paved road |  |  |  |
| 4091 | 591755.068 | 4924947.434 | 152.665 | 188.221 | 336 | Paved road |  |  |  |
| 4092 | 605723.307 | 4924447.195 | 234.343 | 269.836 | 336 | Paved road |  |  |  |
| 3030 | 605721.531 | 4924450.706 | 234.199 | 269.693 | 336 | Tall grass |  |  |  |
| 4093 | 597821.796 | 4937203.563 | 218.986 | 254.398 | 336 | Paved road |  |  |  |
| 3031 | 597835.091 | 4937208.696 | 219.130 | 254.542 | 336 | Short grass |  |  |  |
| 3032 | 588657.701 | 4938178.795 | 172.559 | 208.041 | 336 | Crop |  |  |  |
| 4094 | 588660.101 | 4938166.777 | 173.134 | 208.616 | 336 | Paved road |  |  |  |
| 4095 | 598533.557 | 4947040.282 | 260.859 | 296.204 | 336 | Parking lot |  |  |  |
| CMD2_On | 588247.742 | 4955539.312 | 169.844 | 205.237 | 336 | Can-Net |  |  |  |
| 4096 | 598533.684 | 4947040.360 | 260.829 | 296.174 | 336 | Parking lot |  |  |  |
| 4097 | 597348.022 | 4953616.591 | 188.770 | 224.108 | 336 | Paved road |  |  |  |
| CMD2_On | 588247.742 | 4955539.312 | 169.844 | 205.237 | 336 | Can-Net |  |  |  |
| 1935_chk_01 | 532410.326 | 4939891.541 | 142.405 | 177.927 | 336 | Cosine 820000179 | -0.008 | 0 | -0.063 |
| 1935_chk02 | 532410.414 | 4939891.605 | 142.398 | 177.921 | 336 | Cosine 820000179 | 0.08 | 0.064 | -0.063 |
| 1935_chk03 | 532410.387 | 4939891.620 | 142.418 | 177.940 | 336 | Cosine 820000179 | 0.053 | 0.079 | -0.05 |
| CORI_On | 625681.385 | 4940970.863 | 195.767 | 231.065 | 337 | Can-Net |  |  |  |
| 4098 | 617994.899 | 4942035.418 | 234.354 | 269.648 | 337 | Hard packed |  |  |  |
| 3033 | 617997.645 | 4942038.822 | 234.247 | 269.540 | 337 | Short grass |  |  |  |
| 4099 | 627030.570 | 4950646.884 | 186.820 | 222.024 | 337 | Parking lot |  |  |  |
| 3034 | 627029.713 | 4950654.083 | 186.873 | 222.077 | 337 | Short grass |  |  |  |
| 4100 | 621195.935 | 4955080.831 | 177.503 | 212.699 | 337 | Hard packed |  |  |  |
| 3035 | 612429.832 | 4955845.455 | 153.828 | 189.063 | 337 | Short grass |  |  |  |
| 4101 | 612423.543 | 4955843.286 | 154.135 | 189.370 | 337 | Paved road |  |  |  |
| CMD2_On | 588247.742 | 4955539.312 | 169.844 | 205.237 | 337 | Can-Net |  |  |  |
| 4102 | 609959.510 | 4963983.537 | 162.333 | 197.520 | 337 | Hard packed |  |  |  |
| 4103 | 608360.931 | 4971186.871 | 176.947 | 212.076 | 337 | Hard packed |  |  |  |
| 3036 | 608353.252 | 4971192.024 | 176.663 | 211.793 | 337 | Short brush |  |  |  |
| 4104 | 599956.967 | 4970168.347 | 162.672 | 197.877 | 337 | Paved road |  |  |  |
| 3037 | 599948.662 | 4970165.699 | 162.199 | 197.404 | 337 | Tall brush |  |  |  |
| 3038 | 581537.295 | 4950848.626 | 199.834 | 235.296 | 337 | Short grass |  |  |  |
| 4105 | 581523.099 | 4950847.804 | 199.786 | 235.248 | 337 | Parking lot |  |  |  |
| 3039 | 581907.789 | 4938335.457 | 182.476 | 217.998 | 337 | Tall grass |  |  |  |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 337 | Can-Net |  |  |  |
| 820000179 | 532410.332 | 4939891.557 | 142.386 | 177.908 | 337 | Cosine 820000179 | -0.002 | 0.016 | -0.082 |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 338 | Can-Net |  |  |  |
| 4106 | 487032.649 | 4931629.056 | 192.172 | 228.012 | 338 | Parking lot |  |  |  |
| 3040 | 487038.141 | 4931615.388 | 191.945 | 227.785 | 338 | Tall grass |  |  |  |
| 3041 | 492671.939 | 4920365.210 | 202.410 | 238.064 | 338 | Short grass |  |  |  |
| 4107 | 492669.113 | 4920374.180 | 202.888 | 238.541 | 338 | Hard packed |  |  |  |
| CWAK_On | 486867.066 | 4886684.321 | 264.024 | 299.317 | 338 | Can-Net |  |  |  |
| 4108 | 492876.625 | 4909739.076 | 223.256 | 258.776 | 338 | Paved road |  |  |  |
| 3042 | 492875.859 | 4909751.738 | 222.671 | 258.191 | 338 | Crop |  |  |  |
| 3043 | 497398.305 | 4899122.970 | 265.120 | 300.459 | 338 | Tall grass |  |  |  |
| 4109 | 497403.779 | 4899127.279 | 265.258 | 300.598 | 338 | Paved road |  |  |  |
| 4110 | 495159.519 | 4891142.156 | 257.151 | 292.434 | 338 | Paved road |  |  |  |
| 3044 | 495162.481 | 4891139.842 | 257.012 | 292.295 | 338 | Tall grass |  |  |  |
| 3045 | 501059.069 | 4895777.730 | 264.454 | 299.716 | 338 | Short grass |  |  |  |
| 4111 | 501060.562 | 4895780.223 | 264.566 | 299.828 | 338 | Paved road |  |  |  |
| 4112 | 506808.568 | 4885833.870 | 279.407 | 314.536 | 338 | Hard packed |  |  |  |
| 3046 | 506814.870 | 4885850.812 | 278.975 | 314.104 | 338 | Tall grass |  |  |  |
| 3047 | 498136.194 | 4884424.215 | 238.945 | 274.125 | 338 | Short brush |  |  |  |
| 4113 | 498137.182 | 4884418.510 | 239.480 | 274.660 | 338 | Paved road |  |  |  |
| 4114 | 481488.730 | 4895985.891 | 225.905 | 261.385 | 338 | Hard packed |  |  |  |
| 3048 | 481488.651 | 4895995.189 | 224.771 | 260.251 | 338 | Crop |  |  |  |
| 3049 | 478518.708 | 4904900.855 | 198.404 | 234.033 | 338 | Tall grass |  |  |  |
| CPTL_On | 470062.39 | 4921788.45 | 177.892 | 213.802 | 338 | Can-Net |  |  |  |
| 4115 | 478526.666 | 4904902.766 | 198.149 | 233.778 | 338 | Hard packed |  |  |  |
| 4116 | 475624.518 | 4916636.812 | 182.535 | 218.339 | 338 | Hard packed |  |  |  |
| 4117 | 484940.258 | 4921680.125 | 192.923 | 228.686 | 338 | Hard packed |  |  |  |
| 3050 | 484935.087 | 4921677.103 | 192.385 | 228.148 | 338 | Crop |  |  |  |
| 820000179 | 532410.323 | 4939891.526 | 142.410 | 177.932 | 338 | Cosine 820000179 | -0.011 | -0.015 | -0.058 |
| COWE_On | 504716.944 | 4935583.079 | 149.744 | 185.391 | 339 | Can-Net |  |  |  |
| 3051 | 505702.151 | 4935186.035 | 189.447 | 225.075 | 339 | Short grass |  |  |  |
| 3052 | 519554.296 | 4930152.229 | 209.984 | 245.395 | 339 | Tall grass |  |  |  |
| COL2_On | 559821.583 | 4927419.998 | 166.252 | 201.72 | 339 | Can-Net |  |  |  |
| 3053 | 534129.806 | 4930964.437 | 344.623 | 379.966 | 339 | Tall grass |  |  |  |
| 3054 | 539182.977 | 4923852.247 | 197.022 | 232.311 | 339 | Short grass |  |  |  |
| 3055 | 525485.256 | 4921757.330 | 392.422 | 427.654 | 339 | Tall grass |  |  |  |
| 3056 | 508183.449 | 4922604.532 | 257.137 | 292.576 | 339 | Tall grass |  |  |  |
| 3057 | 522614.980 | 4913562.014 | 379.972 | 415.139 | 339 | Crop |  |  |  |
| CFLS_On | 539535.287 | 4900727.811 | 452.804 | 487.838 | 339 | Can-Net |  |  |  |
| 3058 | 530654.121 | 4907265.501 | 396.943 | 432.034 | 339 | Crop |  |  |  |
| 820048009 | 532228.136 | 4903879.153 | 399.121 | 434.187 | 339 | Cosine 820048009 | 0.022 | 0.005 | -0.044 |
| 3059 | 536408.570 | 4871240.738 | 445.235 | 480.187 | 339 | Crop |  |  |  |
| CLS2_On | 504329.108 | 4843266.883 | 359.647 | 394.487 | 340 | Can-Net |  |  |  |
| 3060 | 503902.066 | 4823147.535 | 334.423 | 369.233 | 340 | Crop |  |  |  |
| 3061 | 497878.851 | 4833675.061 | 328.265 | 363.078 | 340 | Crop |  |  |  |
| 3062 | 475177.629 | 4848867.246 | 301.120 | 336.042 | 340 | Crop |  |  |  |
| 3063 | 463759.751 | 4858539.660 | 292.446 | 327.455 | 340 | Tall grass |  |  |  |
| CWAK_On | 486867.066 | 4886684.321 | 264.024 | 299.317 | 340 | Can-Net |  |  |  |
| 3064 | 474556.654 | 4858791.113 | 272.494 | 307.491 | 340 | Short grass |  |  |  |
| 3065 | 461136.994 | 4873397.710 | 262.925 | 298.140 | 340 | Tall grass |  |  |  |
| 3066 | 486841.286 | 4851579.485 | 309.299 | 344.211 | 340 | Tall grass |  |  |  |
| 81D270 | 486812.507 | 4851604.694 | 309.520 | 344.432 | 340 | NRCan 81D270 | 0.001 | -0.005 | -0.002 |
|  |  |  |  |  | JD (2023) |  |  |  |  |
| 1939-20 | 477221.910 | 4979837.811 | 152.853 | 189.014 | 348 | Base Station |  |  |  |
| 2200 | 479991.829 | 4972182.430 | 156.480 | 192.583 | 348 | Paved Road |  |  |  |
| 2201 | 474302.090 | 4970156.753 | 150.047 | 186.181 | 348 | Paved Road |  |  |  |
| 2202 | 472901.443 | 4973956.490 | 146.406 | 182.563 | 348 | Paved Road |  |  |  |
| 2203 | 475388.138 | 4979141.564 | 150.383 | 186.550 | 348 | Paved Road |  |  |  |
| 2204 | 478632.680 | 4976001.125 | 155.884 | 192.016 | 348 | Paved Road |  |  |  |
| 2205 | 482170.913 | 4978559.297 | 154.216 | 190.340 | 348 | Paved Road |  |  |  |
| 2206 | 477638.747 | 4985165.759 | 157.874 | 194.071 | 348 | Gravel Road |  |  |  |
| 2207 | 471746.940 | 4983477.651 | 153.046 | 189.257 | 348 | Paved Road |  |  |  |
| 2208 | 473047.354 | 4987326.136 | 154.433 | 190.662 | 348 | Paved Road |  |  |  |
| 2209 | 469057.967 | 4981314.006 | 141.494 | 177.707 | 348 | Paved Road |  |  |  |
| 2210 | 467451.251 | 4990565.842 | 165.382 | 201.658 | 348 | Paved Road |  |  |  |
| 2211 | 463599.242 | 4988508.792 | 143.324 | 179.609 | 348 | Paved Road |  |  |  |
| 2212 | 460166.854 | 4990740.751 | 141.906 | 178.227 | 348 | Paved Road |  |  |  |
| 2213 | 473541.139 | 4992364.469 | 161.938 | 198.203 | 348 | Paved Road |  |  |  |
| 2214 | 474875.597 | 4996650.268 | 167.847 | 204.150 | 348 | Gravel Road |  |  |  |
| 3000 | 479959.594 | 4972179.395 | 155.548 | 191.651 | 348 | Short grass |  |  |  |
| 3001 | 474325.934 | 4970136.518 | 149.296 | 185.430 | 348 | Canopy |  |  |  |
| 3002 | 472913.669 | 4973973.960 | 145.913 | 182.069 | 348 | Short grass |  |  |  |
| 3003 | 475393.092 | 4979143.241 | 150.255 | 186.422 | 348 | Short grass |  |  |  |
| 3004 | 478615.449 | 4976001.527 | 155.266 | 191.397 | 348 | Short grass |  |  |  |
| 3005 | 482180.075 | 4978563.423 | 153.900 | 190.025 | 348 | Tall grass |  |  |  |
| 3006 | 477631.386 | 4985172.180 | 157.738 | 193.935 | 348 | Tall grass |  |  |  |
| 3007 | 471747.973 | 4983485.157 | 152.965 | 189.175 | 348 | Short grass |  |  |  |
| 3008 | 473049.148 | 4987316.765 | 153.808 | 190.037 | 348 | Tall grass |  |  |  |
| 3009 | 469053.428 | 4981323.239 | 141.449 | 177.663 | 348 | Tall grass |  |  |  |
| 3010 | 467442.443 | 4990567.265 | 165.037 | 201.312 | 348 | Tall grass |  |  |  |
| 3011 | 463583.558 | 4988507.527 | 143.022 | 179.307 | 348 | Short grass |  |  |  |
| 3012 | 460175.872 | 4990736.475 | 141.079 | 177.400 | 348 | Brush |  |  |  |
| 3013 | 473562.381 | 4992363.175 | 161.436 | 197.700 | 348 | Tall grass |  |  |  |
| 3014 | 474870.937 | 4996646.834 | 167.506 | 203.809 | 348 | Tall grass |  |  |  |
| 90R1 | 477193.918 | 4979824.382 | 152.813 | 188.974 | 348 | BM | -0.009 | -0.003 | 0.014 |
| CHK1 | 477209.442 | 4979829.439 | 152.332 | 188.493 | 348 | Check In |  |  |  |
| CHK2 | 477209.431 | 4979829.430 | 152.337 | 188.498 | 348 | Check Out | -0.011 | -0.009 | 0.005 |
| 1935-21 | 465658.954 | 4997114.203 | 172.084 | 208.416 | 349 | Base Station |  |  |  |
| 2215 | 470488.948 | 4997103.786 | 170.390 | 206.703 | 349 | Paved Road |  |  |  |
| 2216 | 469203.539 | 5002838.511 | 163.772 | 200.142 | 349 | Hard Packed |  |  |  |
| 2217 | 473798.887 | 5004846.653 | 188.022 | 224.401 | 349 | Hard Packed |  |  |  |
| 2218 | 462831.172 | 4999642.455 | 156.423 | 192.790 | 349 | Paved Road |  |  |  |
| 2219 | 456657.891 | 4996160.774 | 141.111 | 177.498 | 349 | Gravel Road |  |  |  |
| 2220 | 454580.760 | 5000715.378 | 149.149 | 185.592 | 349 | Paved Road |  |  |  |
| 2221 | 454591.681 | 5005804.678 | 164.150 | 200.642 | 349 | Paved Road |  |  |  |
| 2222 | 458814.443 | 5008106.662 | 154.232 | 190.721 | 349 | Paved Lot |  |  |  |
| 2223 | 449387.004 | 5007141.429 | 165.782 | 202.338 | 349 | Paved Road |  |  |  |
| 2224 | 447911.097 | 5003789.765 | 140.899 | 177.439 | 349 | Gravel Road |  |  |  |
| 2225 | 451717.320 | 5010579.121 | 163.282 | 199.862 | 349 | Paved Lot |  |  |  |
| 2226 | 445266.916 | 5009154.501 | 142.405 | 179.037 | 349 | Paved Road |  |  |  |
| 2227 | 448496.391 | 5011700.236 | 152.982 | 189.609 | 349 | Paved Road |  |  |  |
| 2228 | 449384.262 | 5009655.554 | 161.989 | 198.579 | 349 | Hard Packed |  |  |  |
| 2229 | 461112.986 | 5006313.872 | 176.797 | 213.246 | 349 | Hard Packed |  |  |  |
| 973007 | 450106.973 | 5008044.383 | 172.742 | 209.301 | 349 | Control Monument Check | -0.007 | 0.001 | -0.002 |
| 3015 | 470488.553 | 4997087.286 | 168.966 | 205.278 | 349 | Tall Grass |  |  |  |
| 3016 | 469206.544 | 5002839.836 | 163.779 | 200.149 | 349 | Tall Grass |  |  |  |
| 3017 | 473778.723 | 5004845.189 | 187.958 | 224.337 | 349 | Canopy |  |  |  |
| 3018 | 462827.031 | 4999654.254 | 155.986 | 192.353 | 349 | Short Grass |  |  |  |
| 3019 | 456662.955 | 4996152.428 | 140.032 | 176.420 | 349 | Tall Grass |  |  |  |
| 3020 | 454573.002 | 5000717.084 | 148.750 | 185.193 | 349 | Brush |  |  |  |
| 3021 | 454590.909 | 5005813.442 | 163.776 | 200.268 | 349 | Tall Grass |  |  |  |
| 3022 | 458832.485 | 5008087.630 | 153.762 | 190.251 | 349 | Canopy |  |  |  |
| 3023 | 449378.656 | 5007139.954 | 165.539 | 202.095 | 349 | Tall Grass |  |  |  |
| 3024 | 447909.126 | 5003795.193 | 140.876 | 177.416 | 349 | Short Grass |  |  |  |
| 3025 | 451721.290 | 5010570.262 | 163.530 | 200.110 | 349 | Canopy |  |  |  |
| 3026 | 445265.691 | 5009168.768 | 141.972 | 178.604 | 349 | Short Grass |  |  |  |
| 3027 | 448485.757 | 5011715.868 | 153.150 | 189.777 | 349 | Brush |  |  |  |
| 3028 | 449388.884 | 5009651.105 | 161.125 | 197.715 | 349 | Short Grass |  |  |  |
| 3029 | 461116.191 | 5006311.593 | 176.472 | 212.922 | 349 | Canopy |  |  |  |
| CHK1 | 465673.228 | 4997130.404 | 171.956 | 208.288 | 349 | Check In |  |  |  |
| CHK2 | 465673.229 | 4997130.396 | 171.952 | 208.284 | 349 | Check Out | 0.001 | -0.008 | -0.004 |

# Appendix C – Published Station Coordinate Sheets