Final Report

For project:

Kawartha Lakes LiDAR

Prepared for:

Ontario Ministry of Natural Resources

2nd floor, North Tower, 300 Water Street,

Peterborough, ON, K9J 3C7

Prepared by:

Airborne Imaging

2700 – 61 Avenue SE

Calgary, AB T2C 4V2

February 24, 2025

Contents

Introduction	1

Project Schedule & Personnel	2

LiDAR System & Flight Parameters	2

Project Control / Datum	5

Check Points	6

Trajectory Processing	8

LiDAR Data Processing	8

Calibration Procedures	8

Data Tiling and Classification	8

Point Cloud Manipulation	9

Water Classification & Hydro-Flattening	9

Class definitions	10

Deliverables	10

Vertical Accuracy Assessment	11

Conclusion	12

Appendix A - Control Points – Published Coordinates	13

Appendix B – RTK Survey & Control Check	14

Appendix C – Published Station Coordinate Sheets	28

# Introduction

The Ontario Ministry of Natural Resources and Forests contracted Airborne Imaging to acquire and deliver digital elevation data derived from airborne LiDAR (Light Detection and Ranging) to cover various regions in Ontario.

This report covers the Kawartha Lakes area, collected in the spring of 2023. The report focuses on LiDAR acquisition details, such as flight parameters, project control, ground truthing results, data processing technique and deliverables.

The contracted area for the Kawartha Lakes area is 3,375 km².

# Project Schedule & Personnel

The ground control occurred during the spring of 2023. The surveyor collecting ground truth points during this period was Ryan Gemmel. The Lidar data was collected in three missions during the spring of 2023. See table below for a list of missions.

A typical Lidar acquisition crew consisted of one Lidar Operator and one Pilot.

# LiDAR System & Flight Parameters

A Riegl VQ-1560ii-S was used to collect the Lidar data. Two systems were installed in KASI Aviation’s Piper Navajo call signs C-GKSX and C-KMEC.

Below is a table of the lifts with their flight parameters.

Table 1: Project Parameters for the Kawartha Lakes area.

|  | Flight plan |
| --- | --- |
| Altitude | 165m AGL |
| Field of View (FOV) | 60° |
| Air Speed | 160 Kts |
| Pulse Rate | 2100 kHz |
| Sidelap | 20% |
| Point Density | > 8 pts/m2 |

The flight lines were designed optimally to maximize time online versus time in turn.

Below is a map of the flight lines including cross-ties.

Below is a map of the flight line footprints colored by season of acquisition.

Spring 2023 (BLUE)

# Project Control / Datum

Several baselines were measured by GPS static occupation between Can-Net Continuously Operating Stations (CORS), existing NRCan 3D Passive Control Monuments, and Ontario COSINE control points, as well as newly established points by Airborne Imaging. The existing control used consisted of four Can-Net stations, one NRCan passive station, and one Cosine point.

Appendix A contains a list of Control Points and their published coordinates.

Appendix B shows the coordinate list of deliverable check points.

Appendix C shows the published station coordinate sheets.

The datum used for processing the Ground Control and all subsequent Lidar data are based on the Nad83 CSRS (epoch 2010.0) horizontal datum and the CGVD2013 vertical datum (CGG2013 undulation model).

Below is an overview map of the control stations.

Figure 1 Map of Control Points for MNRF Kawartha Lakes

# Check Points

RTK GPS

In order to meet the recommended ASPRS minimum number of points for a 3375.1 square kilometer project, RTK points were surveyed in areas distributed throughout the project. A minimum of 60 NVA and 49 VVA points were required to meet ASPRS standards. Real Time Kinematic points are individual points surveyed on open terrain. Our surveyor collected GPS data mounted on a pole while receiving real time differential corrections from the GPS receiver positioned over a nearby control point.

344 points were surveyed on the Kawartha Lakes project. Of these, 171 points were over non-vegetated ground conditions for NVA accuracy reporting and 173 of them were over vegetated areas for VVA accuracy reporting. Pictures of these RTK points (as well as the existing control) are provided with this report in a folder called “Kawartha_Control_Point_Pictures”. These points are used just for the vertical accuracy assessment of the Lidar point cloud and were not used to determine any vertical shift applied to the point cloud.

Figure 2 NVA (Yellow) and VVA (Green) Check Points

# Trajectory Processing

The aircraft trajectory positioning for the missions over the Kawartha Lakes area were processed using the Precise Point Positioning (PPP) technique.

PPP is a positioning technique that removes and models GNSS system errors via precise satellite orbit data and clock corrections that are applied to the trajectory post mission, providing centimeter level positional accuracy without the use of ground base stations.

The TerraPOS software by Terratec was used to postprocess the PPP solutions to produce final kinematic trajectories accurate to 5 centimeters for each flight mission.

The horizontal datum used is Nad83 CSRS UTM zone 17 with the CGVD2013 vertical datum (CGG2013 undulation model).

# LiDAR Data Processing

# Calibration Procedures

After each mission, the field project manager creates the preliminary calibrated point cloud with each sensor’s set boresight values for each mission. Proprietary QC tools using the Lastools software are run on each mission to check for coverage, relative accuracy, point density, etc… If any data is found to be not meeting specifications, or coverage missing, the field crew is immediately notified and the areas affected reflown as soon as possible.

This preliminary calibrated point cloud is inspected in conjunction with the kinematic ground truth points to apply bulk shifts if necessary and further refine the calibration by applying corrections to the preliminary calibrated strips. The refined calibration is done using Bayesmap’s StripAlign software. Intensity values are also normalized by mission to be 16 bits (0 to 65535).

# Data Tiling and Classification

The raw LiDAR data is processed into calibrated point cloud strips corresponding to flight lines. For ease of data manipulation, the strips are combined and saved in tiles of 1 km by 1 km. The format the strips and tiles are saved in is .Laz version 1.4. The tiling scheme is based on the UTM17 projection at the even kilometers.

The tiled point cloud is then processed with a series of algorithms to separate the “most-likely” ground returns from other returns. The point classification is done using a product by the TerraSolid software running on MicroStation Connect called TerraScan and TerraModeler.

In addition to our regular field QC and calibration procedures, we also generate various raster products using the Lastools software to assess coverage and calibration accuracy.

# Point Cloud Manipulation

The TerraScan software uses macros that are set-up to measure the angles and distances between points to determine what classification a point should be: ground, vegetation, noise, other.  The angle and distance values in the macros can be adjusted to be more or less aggressive with the classification of points by varying the incidence angles and estimated distances among neighboring points.  The lower points are generally classified as ground returns, with the points above classified as undefined (class 1). Outliers and noise described in the table below are normally clouds, outliers or atmospheric noise in the data that are separated into above or below the ground level (high point or low points).

After the automated macro is run to determine classes, a systematic visual inspection is performed to fine tune the ground definition.  To better understand areas for improvement for the ground surface, the points that are classified as ground are used to create viewable TINs and grid surfaces.  These surfaces are inspected for areas that appear rough, artificially flattened or truncated, no data areas, or have other viewable errors.

In cleaning up ground points, the focus is concentrated in areas where few ground points have been left in the bare earth model and the ground appears rough or lower and flatter than it may be in reality.  The scarcity of ground points may be a result from no penetration through a dense vegetation layer, water bodies, low reflectivity objects, or too aggressive values with the macro.  A manual/visual inspection of these areas plays a major role in resolving any issues or irregularities with the bare earth model.

Once the first edits are completed, a one-meter grid of the ground surface is re-exported and hill shade images produced to QC the edited ground surface. This QC step is done iteratively as many times as necessary. Once the point cloud data has reached a satisfactory level of classification accuracy, the final deliverables are produced.

# Water Classification & Hydro-Flattening

Once the ground class has reached the final level of classification accuracy, the water delineation process is initiated. The rivers and water bodies are digitized as break lines according to USGS specifications of a minimum of 8000 m² for water bodies and 30 m nominal width for streams and rivers. Ponds and lakes have their elevations set constant to just below ground. Stream and river hydro lines are collected using the downstream method. Once all hydro lines have been collected for an area, the ground points inside the hydro lines are classified to class 9 – Water. The ground points outside of the hydro lines but within 35 cm are classified to class 20 – Ignored ground (near breaklines). Using the TIN of class 2 (ground points), the hydro lines are used as constraints to generate the final 50 cm hydro-flattened DEMs in ESRI Ascii format, then converted to 32bit Geotiff format with the Global Mapper software.

# Class definitions

The classification follows the ASPRS classification codes. The table below shows the classes for the delivered point cloud.

Table 2: ASPRS Point Classes

| Classification Value | Description |
| --- | --- |
| 1 | Unclassified |
| 2 | Ground |
| 6 | Building |
| 7 | Low Points |
| 9 | Water |
| 17 | Bridge Decks |
| 18 | High Points |
| 20 | Ignored Ground (breakline proximity) |
| 21 | Snow |
| 22 | Temporal exclusion |

# Deliverables

The project was collected in the spring of 2023 and delivered in October 2023.

See below for a table of the folder structure and description of each deliverable.

DIFF

50cm rasters of flight lines differences, 1 X 1 km tiles in 32bit GeoTIFF format

e.g. 1km176410496102023LKAWARTHALAKES_diff.tif

DTM

Hydro-flattened 50cm Bare Earth DEMs, 1 X 1 km tiles in 32bit GeoTIFF format

e.g. 1km176410496102023LKAWARTHALAKES.tif

METADATA

Metadata files for Breaklines, DEM, LAS and Project in .XML format

e.g. KAWATHALAKES2023_Lidar_metadata_LAS.xml

POINTCLOUD

Point cloud data, 1 X 1 km tiles in .LAZ format 1.4

e.g. 1km176410496102023LKAWARTHALAKES.laz

PROJECT_POLYGON

Shape files of: Lift_metadata, Hydro_breaklines, Project_extent & Tile_index

e.g. Mission_9523129a.shp

# Vertical Accuracy Assessment

Non-Vegetated Vertical Accuracy (NVA)

Below is a summary of the statistics between the point cloud TIN from the ground class and the NVA points surveyed on flat hard surfaces:

| Number of points: | 173 |  |
| --- | --- | --- |
| Average dz: | 0.009 | m |
| Minimum dz: | -0.107 | m |
| Maximum dz: | +0.091 | m |
| Average magnitude: | 0.029 | m |
| Root mean square: | 0.037 | m |
| Std deviation: | 0.036 | m |
| NVA Vertical Accuracy (95%): | 0.072 | m |

For a complete listing of the RTK points and their differences, please refer to the provided spreadsheet named "Kawartha_NVA_Final.xlsx".

As seen in the NVA comparison above, the accuracy observed of 0.072 m meets the project requirement of QL0 NVA vertical accuracy of 9.8 cm (95%).

Vegetated Vertical Accuracy (VVA)

The elevations of the points surveyed in various vegetated land cover types were compared to the Lidar ground surface. Since the elevation differences for this type of accuracy does not follow a normal distribution, the percentile method is used and not the 95% confidence level. For the 171 VVA points surveyed, sorting the absolute differences from the smallest to the largest gives the accuracy at the 95th percentile (using the linear interpolation between closest ranks method). For this sample, the 162nd and 163rd largest difference are respectively 14.8 and 15.1 cm.

For this project, the VVA accuracy is therefore 14.95 cm.

This is below the required specifications of 15.0 cm (the QL0 VVA specification).

For a complete listing of the kinematic points and their differences, please refer to the provided spreadsheet named "Kawartha_VVA_Final.xlsx".

# Conclusion

This is the final report covering the Kawartha Lakes Lidar project. Topics such as field operations, control, data processing and final accuracy results were explained. The acquisition was done in one season and data processing went well. Both the vegetated and non-vegetated vertical accuracies meet the QL0 requirement.

# Appendix A - Control Points – Published Coordinates

Kawartha Lakes - Control Station Coordinates

| Station | Easting | Northing | h | H (CGVD2013) | Status | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1939-01 | 684430.165 | 4900550.079 | 248.317 | 283.621 | Temporary | Airborne Intermediate Control |
| 1939-02 | 664789.976 | 4884259.072 | 219.913 | 255.431 | Temporary | Airborne Intermediate Control |
| 1939-03 | 669108.314 | 4911053.459 | 239.807 | 275.097 | Temporary | Airborne Intermediate Control |
| 1939-04 | 660546.249 | 4936043.511 | 259.671 | 294.887 | Temporary | Airborne Intermediate Control |
| 1939-05 | 673615.230 | 4947691.605 | 241.897 | 276.889 | Temporary | Airborne Intermediate Control |
| 1939-06 | 694737.876 | 4918619.147 | 235.934 | 271.040 | Temporary | Airborne Intermediate Control |
| 1939-07 | 657156.021 | 4957244.824 | 240.110 | 275.073 | Temporary | Airborne Intermediate Control |
| 1939-08 | 685327.617 | 4934603.303 | 268.302 | 303.315 | Temporary | Airborne Intermediate Control |
| 963023 | 677082.175 | 4914661.868 | 233.427 | 268.657 | Published | NRCan 3D Passive |
| BOV2 | 687546.684 | 4863734.431 | 59.640 | 95.516 | Published | CanNet CORS |
| Cos3005 | 660387.786 | 4908117.616 | 271.459 | 306.806 | Published | Ontario COSINE ‘00820130005’ |
| LIND | 679915.777 | 4914311.893 | 239.627 | 274.846 | Published | CanNet CORS |
| ORIL | 625681.379 | 4940970.854 | 195.769 | 231.067 | Published | CanNet CORS |
| UXBG | 649365.172 | 4885492.470 | 259.722 | 295.268 | Published | CanNet CORS |

Nad83(CSRS), epoch 2010

# Appendix B – RTK Survey & Control Check

| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1939-01 | 684430.165 | 4900550.078 | 248.317 | 283.621 | 137 | Aii Control - Base |
| 2001 | 684473.261 | 4900556.609 | 246.713 | 282.018 | 137 | Paint line |
| 3001 | 684539.012 | 4900560.794 | 242.816 | 278.12 | 137 | Under canopy |
| 2002 | 692563.357 | 4907190.085 | 222.381 | 257.605 | 137 | Paint line |
| 3002 | 692543.839 | 4907219.549 | 221.458 | 256.681 | 137 | Under canopy |
| 2003 | 693886.107 | 4899436.275 | 249.365 | 284.649 | 137 | Paved road |
| 3003 | 693925.823 | 4899458.975 | 249.306 | 284.59 | 137 | Under canopy |
| 2004 | 703880.901 | 4911428.956 | 234.428 | 269.571 | 137 | Paint line - H Park |
| 3004 | 703975.653 | 4911471.687 | 234.458 | 269.6 | 137 | Under canopy |
| 2005 | 703566.347 | 4906640.404 | 219.215 | 254.415 | 137 | Paved road - UC |
| 3005 | 699709.533 | 4905255.446 | 275.587 | 310.806 | 137 | Low grass |
| 3006 | 696090.323 | 4904047.967 | 258.472 | 293.708 | 137 | Under canopy |
| 2006 | 696112.293 | 4903947.529 | 265.32 | 300.557 | 137 | Paved road |
| 3007 | 692956.073 | 4902805.351 | 229.656 | 264.914 | 137 | Under canopy |
| 2007 | 692984.65 | 4902816.717 | 230.22 | 265.477 | 137 | Paved road |
| 2008 | 694439.708 | 4894911.87 | 226.921 | 262.255 | 137 | Paved road |
| 3008 | 694209.817 | 4894824.256 | 221.882 | 257.216 | 137 | Under canopy |
| 2009 | 695257.231 | 4887024.782 | 271.623 | 307.046 | 137 | Paint line - H Park |
| 3009 | 695264.634 | 4887094.808 | 267.875 | 303.297 | 137 | Under canopy |
| 3010 | 695522.798 | 4886121.165 | 283.328 | 318.765 | 137 | Under canopy |
| 2010 | 695515.162 | 4886158.379 | 283.315 | 318.751 | 137 | Hard surface |
| 2011 | 689655.283 | 4885562.967 | 290.416 | 325.841 | 137 | Hard surface |
| 3011 | 689635.901 | 4885541.547 | 290.16 | 325.584 | 137 | Under canopy |
| 3012 | 689457.192 | 4885660.886 | 303.916 | 339.339 | 137 | Under canopy |
| 2012 | 689428.65 | 4885622.112 | 300.084 | 335.506 | 137 | Paint line |
| 2013 | 685641.079 | 4887276.966 | 278.51 | 313.915 | 137 | Paved road |
| 3013 | 685630.003 | 4887262.587 | 278.905 | 314.31 | 137 | Under canopy |
| 2014 | 681622.892 | 4885542.203 | 272.786 | 308.226 | 137 | Hard surface |
| 3014 | 681664.385 | 4885510.037 | 271.659 | 307.1 | 137 | Under canopy |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-02 | 664789.976 | 4884259.071 | 219.913 | 255.431 | 138 | Aii Control - Base |
| 2015 | 664721.554 | 4884236.311 | 222.131 | 257.649 | 138 | Pavement |
| 3015 | 664737.915 | 4884184.421 | 223.037 | 258.557 | 138 | Under canopy |
| 2016 | 664410.607 | 4885474.096 | 219.287 | 254.793 | 138 | Corner Sidewalk |
| 3016 | 664471.995 | 4885364.891 | 223.58 | 259.087 | 138 | Low grass |
| 3017 | 664534.684 | 4885405.933 | 222.968 | 258.474 | 138 | Under canopy |
| 2018 | 664595.105 | 4885443.189 | 222.773 | 258.279 | 138 | Paint line |
| 2017 | 664809.77 | 4885521.998 | 215.282 | 250.787 | 138 | Pavement |
| 3019 | 664739.576 | 4885587.77 | 216.328 | 251.832 | 138 | Low grass |
| 2019 | 664751.917 | 4885688.987 | 214.583 | 250.086 | 138 | Corner Sidewalk |
| 3018 | 664731.824 | 4885809.336 | 214.798 | 250.3 | 138 | Low grass - UC |
| 3020 | 664665.459 | 4885911.337 | 214.776 | 250.277 | 138 | Low grass |
| 2020 | 664648.585 | 4885794.463 | 217.57 | 253.072 | 138 | Paint line - H Park |
| 2021 | 664461.027 | 4885625.942 | 217.749 | 253.253 | 138 | Manhole cover |
| 2022 | 663890.08 | 4887354.368 | 215.655 | 251.142 | 138 | Hard surface - UC |
| 3021 | 664205.238 | 4887441.466 | 220.076 | 255.562 | 138 | Low grass - UC |
| 3022 | 663947.609 | 4887333.288 | 220.4 | 255.888 | 138 | Tall brush |
| 3023 | 663661.461 | 4889306.532 | 237.294 | 272.761 | 138 | Low grass |
| 2023 | 663648.364 | 4889197.971 | 236.197 | 271.665 | 138 | Pavement |
| 2024 | 665132.231 | 4894816.224 | 218.113 | 253.527 | 138 | Pavement |
| 3024 | 665190.402 | 4894840.517 | 215.132 | 250.545 | 138 | Low grass |
| 3025 | 663452.816 | 4896324.134 | 223.717 | 259.12 | 138 | Low grass |
| 2025 | 663399.457 | 4896380.623 | 223.798 | 259.201 | 138 | Paint line |
| 3026 | 663107.942 | 4899716.123 | 247.673 | 283.051 | 138 | Low brush |
| 2026 | 663135.813 | 4899707.603 | 248.641 | 284.019 | 138 | Pavement |
| 2027 | 661418.402 | 4885552.679 | 219.036 | 254.546 | 138 | Pavement |
| 2028 | 661276.942 | 4885510.062 | 219.486 | 254.997 | 138 | Paint line |
| 3027 | 661256.237 | 4885526.026 | 217.939 | 253.449 | 138 | Tall brush |
| 3028 | 663399.618 | 4886198.494 | 228.895 | 264.395 | 138 | Low grass - UC |
| 2029 | 663361.43 | 4886168.012 | 230.384 | 265.884 | 138 | Paint line |
| 2030 | 662531.441 | 4884987.391 | 232.534 | 268.049 | 138 | Hard surface |
| 3029 | 662540.712 | 4884999.953 | 232.794 | 268.308 | 138 | Low grass |
| 2031 | 661850.017 | 4885682.085 | 219.758 | 255.266 | 138 | Paint line |
| 2032 | 662952.559 | 4882889.2 | 244.512 | 280.047 | 138 | Hard surface |
| 3030 | 662537.932 | 4882815.231 | 251.328 | 286.865 | 138 | Low grass - UC |
| 3031 | 658084.357 | 4881537.7 | 252.285 | 287.84 | 138 | Under canopy |
| 2033 | 658059.855 | 4881550.221 | 252.727 | 288.282 | 138 | Paved road |
| 2034 | 666348.85 | 4879565.217 | 236.705 | 272.28 | 138 | Paint line |
| 3032 | 666320.506 | 4879473.713 | 240.002 | 275.579 | 138 | Tall grass - UC |
| 3033 | 672894.208 | 4882460.616 | 267.757 | 303.27 | 138 | Tall grass - UC |
| 2035 | 672893.994 | 4882438.471 | 268.367 | 303.88 | 138 | Paint line |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-04 | 660546.247 | 4936043.511 | 259.666 | 294.882 | 140 | Aii Control - Base |
| 3034 | 660549.762 | 4936087.248 | 260.609 | 295.825 | 140 | Low grass - UC |
| 2036 | 660572.953 | 4936072.021 | 261.224 | 296.439 | 140 | Paved road |
| 3035 | 675033.58 | 4937587.814 | 230.869 | 265.951 | 140 | Tall grass - UC |
| 2037 | 675086.98 | 4937575.589 | 230.28 | 265.361 | 140 | Paint line |
| 2038 | 668230.321 | 4927548.433 | 248.183 | 283.389 | 140 | Paved road |
| 3036 | 668181.933 | 4927551.052 | 248.078 | 283.284 | 140 | Low grass - UC |
| 2039 | 664127.804 | 4926311.8 | 235.647 | 270.895 | 140 | Paint line |
| 2040 | 664771.519 | 4924542.278 | 249.188 | 284.436 | 140 | Paved road |
| 3037 | 664800.082 | 4924548.145 | 248.845 | 284.093 | 140 | Tall grass |
| 3038 | 662839.414 | 4925670.624 | 248.083 | 283.345 | 140 | Tall grass |
| 2041 | 662857.178 | 4925607.454 | 249.649 | 284.911 | 140 | Paved road |
| 2042 | 663332.439 | 4928427.254 | 248.358 | 283.604 | 140 | Hard surface |
| 3039 | 663310.265 | 4928528.225 | 247.46 | 282.706 | 140 | Tall grass |
| 3040 | 662003.234 | 4932115.646 | 222.431 | 257.668 | 140 | Tall grass |
| 2043 | 661945.147 | 4932104.774 | 222.718 | 257.955 | 140 | Paint line |
| 2044 | 660788.621 | 4936117.393 | 263.288 | 298.502 | 140 | Hard surface |
| 3041 | 660791.96 | 4936102.558 | 264.046 | 299.259 | 140 | Low grass - UC |
| 3042 | 656634.441 | 4935051.618 | 239.761 | 275.012 | 140 | Under canopy |
| 2045 | 656621.579 | 4935061.086 | 239.759 | 275.01 | 140 | Paint line |
| 2046 | 655438.386 | 4934321.615 | 239.064 | 274.33 | 140 | Paint line |
| 3043 | 655433.361 | 4934369.786 | 237.716 | 272.982 | 140 | Tall brush |
| 3044 | 655242.436 | 4937720.104 | 207.149 | 242.383 | 140 | Tall grass |
| 2047 | 655249.867 | 4937749.549 | 207.463 | 242.696 | 140 | Paved road |
| 2048 | 654926.566 | 4940284.825 | 211.305 | 246.513 | 140 | Paint line |
| 3045 | 654930.143 | 4940267.172 | 209.85 | 245.058 | 140 | Tall brush |
| 3046 | 653659.593 | 4941019.378 | 211.877 | 247.083 | 140 | Tall brush |
| 2049 | 653604.816 | 4941070.972 | 213.551 | 248.756 | 140 | Hard surface |
| 2050 | 650678.077 | 4947083.847 | 202.327 | 237.477 | 140 | Paved road |
| 3047 | 650684.857 | 4947053.163 | 202.502 | 237.652 | 140 | Low grass |
| 3048 | 650660.487 | 4947124.298 | 200.316 | 235.465 | 140 | Low grass - UC |
| 3049 | 651411.679 | 4948670.489 | 217.469 | 252.593 | 140 | Under canopy |
| 2051 | 651395.144 | 4948680.564 | 216.744 | 251.868 | 140 | Hard surface - UC |
| 2052 | 653690.357 | 4949537.777 | 224.826 | 259.921 | 140 | Hard surface |
| 3050 | 653705.778 | 4949551.89 | 225.318 | 260.412 | 140 | Under canopy |
| 3051 | 654964.877 | 4949993.606 | 232.989 | 268.068 | 140 | Under canopy |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-03 | 669108.314 | 4911053.459 | 239.807 | 275.098 | 141 | Aii Control - Base |
| 2054 | 669100.34 | 4911076.037 | 239.549 | 274.84 | 141 | Paint line |
| 3052 | 669133.927 | 4911043.58 | 240.575 | 275.866 | 141 | Low grass - UC |
| 3053 | 669117.738 | 4911002.41 | 238.535 | 273.826 | 141 | Low grass - UC |
| 3054 | 666001.502 | 4910059.26 | 245.578 | 280.887 | 141 | Tall grass - UC |
| 2055 | 666031.981 | 4910071.287 | 246.372 | 281.681 | 141 | Manhole cover |
| 3055 | 666094.246 | 4910101.481 | 246.3 | 281.608 | 141 | Tall grass |
| 2056 | 664682.439 | 4909573.618 | 241.26 | 276.577 | 141 | Hard surface |
| 3057 | 665038.154 | 4912617.554 | 236.695 | 271.995 | 141 | Tall grass - UC |
| 2057 | 665039.346 | 4912611.509 | 237.408 | 272.709 | 141 | Hard surface |
| 2058 | 666250.217 | 4917420.319 | 251.616 | 286.885 | 141 | Paved road |
| 3058 | 666247.697 | 4917426.219 | 251.211 | 286.48 | 141 | Low grass |
| 2059 | 666470.869 | 4918414.778 | 247.953 | 283.216 | 141 | Paint line |
| 2060 | 664757.555 | 4919749.945 | 253.433 | 288.703 | 141 | Paint line |
| 3060 | 664775.064 | 4919744.726 | 251.465 | 286.734 | 141 | Tall grass - UC |
| 3059 | 666458.986 | 4918415.645 | 246.769 | 282.032 | 141 | Low grass - UC |
| 3061 | 665223.876 | 4923426.391 | 254.515 | 289.763 | 141 | Tall brush |
| 2061 | 665198.366 | 4923410.225 | 254.853 | 290.103 | 141 | Paint line |
| 2062 | 667246.888 | 4923895.213 | 255.464 | 290.694 | 141 | Paved road |
| 3062 | 667136.846 | 4923892.14 | 256.226 | 291.458 | 141 | Low grass - UC |
| 3063 | 667794.579 | 4922161.859 | 258.916 | 294.152 | 141 | Low grass |
| 2063 | 667811.824 | 4922147.942 | 259.743 | 294.978 | 141 | Paved road |
| 2064 | 668927.328 | 4922483.603 | 265.444 | 300.669 | 141 | Hard surface |
| 3064 | 668933.547 | 4922477.527 | 264.775 | 300.001 | 141 | Low grass |
| 3065 | 670904.562 | 4922494.925 | 240.989 | 276.199 | 141 | Low grass - UC |
| 2065 | 670912.7 | 4922498.543 | 241.397 | 276.608 | 141 | Hard surface |
| 2066 | 672017.056 | 4922675.043 | 232.393 | 267.595 | 141 | Hard surface |
| 3066 | 672021.869 | 4922682.841 | 232.007 | 267.209 | 141 | Tall grass - UC |
| 3067 | 676761.187 | 4924308.48 | 241.76 | 276.923 | 141 | Low grass |
| 2067 | 676635.288 | 4924250.528 | 239.22 | 274.385 | 141 | Paint line |
| 2068 | 676931.632 | 4923538.113 | 243.653 | 278.821 | 141 | Paint line - H Park |
| 3068 | 676923.694 | 4923525.141 | 243.439 | 278.607 | 141 | Low grass - UC |
| 3069 | 672604.878 | 4919628.2 | 236.552 | 271.771 | 141 | Low grass |
| 2069 | 672640.983 | 4919638.159 | 236.226 | 271.445 | 141 | Paint line |
| 2070 | 669474.956 | 4919156.139 | 246.338 | 281.578 | 141 | Paint line |
| 3070 | 669461.197 | 4919169.131 | 246.557 | 281.797 | 141 | Low grass - UC |
| 3071 | 670630.3 | 4915958.96 | 240.514 | 275.768 | 141 | Low grass |
| 2071 | 670617.537 | 4915938.272 | 240.182 | 275.436 | 141 | Paved road |
| 2072 | 673712.227 | 4912641.395 | 238.023 | 273.283 | 141 | Paint line |
| 3072 | 673684.475 | 4912694.942 | 237.058 | 272.318 | 141 | Tall grass |
| 3073 | 676895.418 | 4912817.717 | 235.477 | 270.724 | 141 | Tall grass - UC |
| 2073 | 676998.719 | 4912713.636 | 233.492 | 268.739 | 141 | Paint line - H Park |
| 2074 | 677226.423 | 4914757.602 | 232.985 | 268.215 | 141 | Paint line - H Park |
| 3074 | 677227.302 | 4914813.114 | 231.721 | 266.951 | 141 | Crop |
| 2075 | 677158.462 | 4914714.042 | 231.861 | 267.092 | 141 | Paint line |
| 3075 | 677204.861 | 4914789.847 | 232.462 | 267.692 | 141 | Low grass |
| 3076 | 678069.684 | 4913642.032 | 232.897 | 268.132 | 141 | Low grass |
| 2076 | 678061.904 | 4913648.247 | 232.729 | 267.964 | 141 | Paint line - H Park |
| 2077 | 678903.398 | 4913967.032 | 238.818 | 274.046 | 141 | Paint line |
| 3077 | 678875.42 | 4913955.116 | 239.542 | 274.771 | 141 | Low grass - UC |
| 3078 | 681539.884 | 4914912.259 | 234.506 | 269.712 | 141 | Low grass |
| 2078 | 681482.54 | 4914894.284 | 232.085 | 267.291 | 141 | Paint line - H Park |
| 2079 | 681614.848 | 4923001.098 | 221.463 | 256.603 | 141 | Paved road |
| 3079 | 681609.614 | 4923013.502 | 220.79 | 255.93 | 141 | Low grass |
| 2080 | 681533.951 | 4919910.498 | 239.802 | 274.966 | 141 | Paved road |
| 2081 | 670983.951 | 4905315.246 | 225.866 | 261.189 | 141 | Paint line |
| 3081 | 670969.503 | 4905273.59 | 227.76 | 263.082 | 141 | Low grass |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-05 | 673615.23 | 4947691.604 | 241.897 | 276.89 | 142 | Aii Control - Base |
| 2082 | 674325.838 | 4947814.392 | 223.653 | 258.639 | 142 | Paint Line |
| 3082 | 674386.99 | 4947822.598 | 223.51 | 258.496 | 142 | Tall grass - UC |
| 3083 | 673824.233 | 4949818.684 | 228.84 | 263.806 | 142 | Low grass |
| 2083 | 673812.146 | 4949861.376 | 228.679 | 263.644 | 142 | Hard surface |
| 2084 | 672991.831 | 4955177.614 | 231.381 | 266.289 | 142 | Paint line |
| 3084 | 673025.368 | 4955251.505 | 230.571 | 265.478 | 142 | Under canopy |
| 3085 | 671944.893 | 4957850.508 | 248.193 | 283.069 | 142 | Under canopy |
| 2085 | 671996.487 | 4957835.261 | 248.587 | 283.464 | 142 | Paved road |
| 3086 | 673026.505 | 4960007.332 | 253.84 | 288.681 | 142 | Tall grass |
| 2086 | 672998.259 | 4959998.983 | 253.42 | 288.261 | 142 | Paint line |
| 2087 | 671801.107 | 4957330.566 | 243.118 | 278.002 | 142 | Paint line |
| 3087 | 668264.55 | 4955032.137 | 249.6 | 284.531 | 142 | Tall grass |
| 3088 | 668246.251 | 4955020.456 | 249.803 | 284.733 | 142 | Under canopy |
| 2088 | 668256.702 | 4955034.449 | 250.002 | 284.933 | 142 | Hard surface |
| 2089 | 667479.041 | 4957076.635 | 256.706 | 291.613 | 142 | Hard surface - UC |
| 3089 | 667471.588 | 4957063.801 | 256.52 | 291.427 | 142 | Under canopy |
| 3090 | 667807.113 | 4958074.003 | 257.173 | 292.065 | 142 | Tall brush |
| 2090 | 667812.55 | 4958074.565 | 257.162 | 292.054 | 142 | Hard surface |
| 2091 | 667975.455 | 4955786.149 | 254.223 | 289.146 | 142 | Hard surface |
| 3091 | 667984.872 | 4955779.154 | 254.551 | 289.474 | 142 | Short grass |
| 3092 | 665797.853 | 4953915.492 | 237.731 | 272.688 | 142 | Short grass |
| 2092 | 665774.826 | 4953905.719 | 238.144 | 273.102 | 142 | Hard surface |
| 2093 | 663782.488 | 4954949.553 | 237.727 | 272.681 | 142 | Hard surface |
| 3093 | 663798.404 | 4955012.375 | 237.765 | 272.717 | 142 | Short grass |
| 3094 | 660024.507 | 4957663.88 | 234.841 | 269.781 | 142 | Tall grass |
| 2094 | 660018.191 | 4957668.425 | 235.179 | 270.119 | 142 | Hard surface |
| 2095 | 664696.851 | 4953650.997 | 243.32 | 278.287 | 142 | Paint line |
| 3095 | 664716.985 | 4953644.534 | 242.629 | 277.596 | 142 | Short grass |
| 3096 | 665817.848 | 4949885.982 | 245.56 | 280.573 | 142 | Tall grass |
| 2096 | 665820.261 | 4949902.435 | 246.382 | 281.395 | 142 | Paved road |
| 2097 | 664457.813 | 4945819.006 | 229.101 | 264.178 | 142 | Hard surface |
| 3097 | 664471.639 | 4945829.488 | 229.001 | 264.078 | 142 | Short grass - UC |
| 3098 | 665109.966 | 4944004.387 | 225.485 | 260.583 | 142 | Crop |
| 2098 | 665091.432 | 4943991.595 | 225.328 | 260.426 | 142 | Hard surface |
| 2099 | 662719.208 | 4942484.611 | 219.082 | 254.215 | 142 | Paved road |
| 3099 | 662742.441 | 4942500.248 | 218.186 | 253.319 | 142 | Tall grass |
| 2100 | 663088.718 | 4941462.269 | 222.523 | 257.667 | 142 | Paint line |
| 2101 | 666158.966 | 4933233.467 | 232.696 | 267.891 | 142 | Paved road |
| 3101 | 666150.84 | 4933232.03 | 232.622 | 267.818 | 142 | Short grass |
| 3102 | 669651.764 | 4936031.418 | 225.797 | 260.94 | 142 | Tall grass |
| 2102 | 669670.075 | 4936044.538 | 225.679 | 260.821 | 142 | Hard surface |
| 2103 | 668416.444 | 4931193.349 | 222.076 | 257.263 | 142 | Hard surface |
| 3103 | 668399.677 | 4931201.317 | 221.357 | 256.544 | 142 | Under canopy |
| 3104 | 669483.647 | 4928324.077 | 241.655 | 276.845 | 142 | Tall grass |
| 2104 | 669497.88 | 4928320.623 | 243.675 | 278.865 | 142 | Paint line |
| 2105 | 671623.857 | 4933942.823 | 231.444 | 266.582 | 142 | Paint line |
| 3105 | 671626.557 | 4933915.29 | 231.357 | 266.496 | 142 | Tall brush |
| 3106 | 676014.12 | 4932453.218 | 232.409 | 267.519 | 142 | Crop |
| 2106 | 675958.321 | 4932425.112 | 234.467 | 269.577 | 142 | Hard surface |
| 2107 | 675048.064 | 4936962.483 | 241.108 | 276.194 | 142 | Paint line - H Park |
| 3107 | 675027.48 | 4936959.587 | 241.283 | 276.369 | 142 | Short grass |
| 3108 | 675124.067 | 4944390.384 | 226.448 | 261.465 | 142 | Tall grass |
| 2108 | 675088.961 | 4944372.174 | 227.422 | 262.439 | 142 | Paint line |
| 2109 | 682860.72 | 4948100.45 | 225.458 | 260.38 | 142 | Paint line |
| 3109 | 682859.329 | 4948088.37 | 224.062 | 258.983 | 142 | Tall grass |
| 3110 | 686189.962 | 4960760.609 | 271.728 | 306.502 | 142 | Under canopy |
| 2110 | 686172.291 | 4960741.399 | 272.564 | 307.338 | 142 | Hard surface |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 963023 | 677082.176 | 4914661.867 | 233.427 | 268.658 | 143 | NRCAN BM - Base |
| 2111 | 677233.332 | 4914771.116 | 232.753 | 267.983 | 143 | Paint line |
| 3111 | 677267.567 | 4914679.802 | 230.703 | 265.933 | 143 | Low grass |
| 2112 | 687799.79 | 4925034.539 | 236.338 | 271.421 | 143 | Paved road |
| 3112 | 687802.021 | 4925017.461 | 236.118 | 271.202 | 143 | Low grass - UC |
| 3113 | 690344.856 | 4920027.877 | 232.274 | 267.39 | 143 | Tall grass |
| 2113 | 690341.919 | 4920037.693 | 232.824 | 267.939 | 143 | Paved road |
| 2114 | 689905.707 | 4910863.846 | 228.207 | 263.408 | 143 | Hard surface |
| 3114 | 689893.363 | 4910856.1 | 227.713 | 262.914 | 143 | Tall grass |
| 3115 | 687819.078 | 4909805.541 | 222.453 | 257.673 | 143 | Tall grass |
| 2115 | 687839.411 | 4909796.002 | 223.264 | 258.484 | 143 | Paint line |
| 2116 | 683934.251 | 4911832.698 | 235.797 | 271.018 | 143 | Paint line - H Park |
| 3116 | 683928.542 | 4911818.333 | 235.593 | 270.814 | 143 | Short grass - UC |
| 3117 | 681723.578 | 4910220.428 | 220.817 | 256.063 | 143 | Short grass |
| 2117 | 681752.388 | 4910261.718 | 220.81 | 256.056 | 143 | Paved road |
| 2118 | 681441.675 | 4907021.313 | 220.3 | 255.573 | 143 | Paved road |
| 3118 | 681450.182 | 4907026.471 | 219.509 | 254.782 | 143 | Short grass |
| 3119 | 680508.677 | 4906441.228 | 215.003 | 250.284 | 143 | Tall grass |
| 2119 | 680499.785 | 4906442.462 | 215.204 | 250.485 | 143 | Hard surface |
| 3120 | 678380.194 | 4898717.422 | 216.891 | 252.235 | 143 | Tall grass |
| 2120 | 678388.449 | 4898700.51 | 218.706 | 254.051 | 143 | Paved road |
| 2121 | 680635.489 | 4900831.472 | 224.059 | 259.379 | 143 | Paved road |
| 3121 | 680626.013 | 4900845.892 | 223.054 | 258.374 | 143 | Tall grass |
| 3122 | 677328.69 | 4906210.856 | 216.751 | 252.047 | 143 | Short grass |
| 2122 | 677337.552 | 4906204.285 | 217.429 | 252.725 | 143 | Paved road |
| 2123 | 678052.414 | 4901952.385 | 223.519 | 258.842 | 143 | Paved road |
| 3123 | 678060.066 | 4901983.476 | 222.366 | 257.688 | 143 | Short grass - UC |
| 2124 | 675365.169 | 4901606.219 | 233.99 | 269.324 | 143 | Paved road |
| 3124 | 675266.038 | 4901577.169 | 230.803 | 266.138 | 143 | Tall brush |
| 3125 | 670477.487 | 4906462.043 | 226.986 | 262.302 | 143 | Short grass |
| 2125 | 670505.136 | 4906461.546 | 227.325 | 262.64 | 143 | Paint line |
| 2126 | 678478.756 | 4913076.978 | 235.255 | 270.492 | 143 | Paint line - H Park |
| 3126 | 678383.975 | 4913123.925 | 235.022 | 270.259 | 143 | Short grass |
| 2127 | 680353.515 | 4913587.98 | 224.221 | 259.445 | 143 | Paint line - H Park |
| 3127 | 680327.984 | 4913604.062 | 224.306 | 259.53 | 143 | Short grass |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-02 | 664789.976 | 4884259.071 | 219.913 | 255.431 | 144 | Aii Control - Base |
| 2128 | 666691.551 | 4888552.595 | 234.491 | 269.96 | 144 | Stop Line |
| 3128 | 666708.577 | 4888566.095 | 232.958 | 268.427 | 144 | Tall Grass |
| 3129 | 668309.781 | 4893004.564 | 236.18 | 271.603 | 144 | Brush - UC |
| 3130 | 668355.891 | 4892982.333 | 236.527 | 271.95 | 144 | Short Grass |
| 2129 | 668334.127 | 4892925.114 | 237.343 | 272.767 | 144 | Stop Line |
| 2130 | 668311.047 | 4892945.818 | 236.877 | 272.301 | 144 | Paved Lot |
| 2131 | 670881.503 | 4895510.996 | 215.661 | 251.058 | 144 | Paved Road |
| 3131 | 670867.319 | 4895526.076 | 215.536 | 250.932 | 144 | Short Grass |
| 3132 | 670068.05 | 4890500.142 | 234.123 | 269.564 | 144 | Tall Grass |
| 2132 | 670065.732 | 4890509.146 | 234.763 | 270.204 | 144 | Paved Road |
| 2133 | 666863.666 | 4885718.206 | 242.196 | 277.695 | 144 | Paved Lot |
| 3133 | 666864.788 | 4885745.373 | 241.853 | 277.352 | 144 | Short Grass |
| 3134 | 673704.378 | 4888798.314 | 225.39 | 260.833 | 144 | Brush |
| 2134 | 673678.428 | 4888794.131 | 225.636 | 261.08 | 144 | Stop Line |
| 2135 | 673254 | 4892012.911 | 215.134 | 250.551 | 144 | Paved Road |
| 3135 | 673244.247 | 4892016.527 | 214.744 | 250.162 | 144 | Short Grass |
| 3136 | 674362.472 | 4892844.349 | 216.431 | 251.838 | 144 | Short grass - UC |
| 2136 | 674382.46 | 4892843.739 | 217.099 | 252.506 | 144 | Paved Road |
| 2137 | 676030.489 | 4891174.058 | 254.836 | 290.247 | 144 | Gravel Road |
| 3137 | 676044.537 | 4891181.461 | 255.05 | 290.461 | 144 | Short Grass |
| 3138 | 677398.163 | 4887102.211 | 267.904 | 303.347 | 144 | Short Grass |
| 2138 | 677391.173 | 4887108.949 | 268.11 | 303.552 | 144 | Paved Road |
| 2139 | 674556.895 | 4886107.746 | 260.012 | 295.477 | 144 | Paved Road |
| 3139 | 674571.647 | 4886163.114 | 258.461 | 293.925 | 144 | Short Grass |
| 3140 | 675932.897 | 4882003.932 | 268.54 | 304.047 | 144 | Tall Grass |
| 2140 | 675955.859 | 4882023.891 | 269.991 | 305.498 | 144 | Paved Road |
| 2141 | 670011.652 | 4881412.98 | 246.639 | 282.177 | 144 | Gravel Pull-Out |
| 3141 | 669986.734 | 4881384.484 | 246.178 | 281.716 | 144 | Short Grass |
| 3142 | 664795.111 | 4884240.095 | 219.564 | 255.082 | 144 | Short Grass |
| 2142 | 664833.259 | 4884229.939 | 218.736 | 254.254 | 144 | Paved Lot |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-01 | 684430.165 | 4900550.078 | 248.317 | 283.621 | 144 | Aii Control - Base |
| 2143 | 685300.591 | 4898785.621 | 218.373 | 253.685 | 144 | Paved Road |
| 3143 | 685308.529 | 4898770.065 | 217.496 | 252.808 | 144 | Short Grass |
| 3144 | 686097.843 | 4896590.975 | 249.473 | 284.798 | 144 | Tall Grass |
| 2144 | 686118.077 | 4896573.236 | 249.503 | 284.828 | 144 | Stop Line |
| 2145 | 687123.149 | 4892875.123 | 267.42 | 302.769 | 144 | Stop Line |
| 3145 | 687115.945 | 4892899.002 | 267.528 | 302.877 | 144 | Tall Grass |
| 3146 | 687996.571 | 4889654.817 | 298.825 | 334.201 | 144 | Tall Grass |
| 2146 | 688002.569 | 4889664.592 | 299.211 | 334.587 | 144 | Paved Road |
| 3147 | 690120.377 | 4890432.342 | 319.539 | 354.906 | 144 | Tall Grass |
| 2147 | 690123.123 | 4890427.112 | 319.728 | 355.096 | 144 | Gravel Road |
| 2148 | 690328.653 | 4887490.491 | 310.388 | 345.79 | 144 | Paved Road |
| 3148 | 690303.773 | 4887492.913 | 310.305 | 345.706 | 144 | Brush |
| 3149 | 693056.171 | 4888378.804 | 282.215 | 317.615 | 144 | Tall Grass |
| 2149 | 693050.621 | 4888384.763 | 282.793 | 318.193 | 144 | Paved Road |
| 2150 | 693328.947 | 4892307.145 | 236.896 | 272.254 | 144 | Paved Road |
| 3150 | 693266.688 | 4892297.438 | 235.523 | 270.881 | 144 | Tall Grass |
| 3151 | 692731.494 | 4894325.984 | 226.794 | 262.128 | 144 | Tall Grass |
| 2151 | 692708.537 | 4894322.74 | 227.3 | 262.634 | 144 | Paved Lot |
| 2152 | 691780.125 | 4896823.216 | 233.404 | 268.712 | 144 | Paved Road |
| 3152 | 691768.523 | 4896830.216 | 233.249 | 268.556 | 144 | Brush |
| 3153 | 689446.277 | 4896288.707 | 285.875 | 321.188 | 144 | Short Grass |
| 2153 | 689449.588 | 4896269.408 | 287.776 | 323.09 | 144 | Paved Road |
| 2154 | 686821.656 | 4904159.367 | 249.459 | 284.73 | 144 | Paved Road |
| 3154 | 686807.632 | 4904144.952 | 250.106 | 285.376 | 144 | Short Grass |
| 3155 | 687628.569 | 4906124.222 | 224.579 | 259.833 | 144 | Short Grass |
| 2155 | 687633.983 | 4906135.5 | 224.386 | 259.64 | 144 | Gravel Road |
| 2156 | 689465.833 | 4905212.875 | 218.262 | 253.515 | 144 | Gravel Road |
| 3156 | 689477.352 | 4905195.678 | 217.652 | 252.904 | 144 | Short Grass |
| 3157 | 681260.665 | 4899433.257 | 241.239 | 276.563 | 144 | Tall Grass |
| 2157 | 681273.493 | 4899441.09 | 242.752 | 278.077 | 144 | Stop Line |
| 2158 | 683355.98 | 4893705.176 | 234.285 | 269.643 | 144 | No Picture |
| 3158 | 683346.242 | 4893703.254 | 234.288 | 269.646 | 144 | No Picture |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-08 | 685327.617 | 4934603.302 | 268.302 | 303.315 | 145 | Aii Control - Base |
| 2159 | 685347.085 | 4934652.200 | 267.130 | 302.142 | 145 | No Picture |
| 3159 | 685324.456 | 4934672.693 | 265.749 | 300.761 | 145 | No Picture |
| 3160 | 691351.122 | 4934430.880 | 230.440 | 265.418 | 145 | No Picture |
| 2160 | 691338.808 | 4934418.187 | 230.741 | 265.719 | 145 | No Picture |
| 2161 | 695254.124 | 4935394.856 | 219.554 | 254.496 | 145 | No Picture |
| 3161 | 695246.260 | 4935427.076 | 220.049 | 254.991 | 145 | No Picture |
| 3162 | 693344.684 | 4940488.921 | 278.795 | 313.693 | 145 | No Picture |
| 2162 | 693344.565 | 4940499.512 | 277.897 | 312.795 | 145 | No Picture |
| 2163 | 691011.995 | 4947162.151 | 270.237 | 305.092 | 145 | No Picture |
| 3163 | 690989.402 | 4947226.101 | 269.109 | 303.964 | 145 | No Picture |
| 3170 | 690741.952 | 4947920.719 | 260.412 | 295.263 | 145 | No Picture |
| 2170 | 690756.165 | 4947850.768 | 260.796 | 295.648 | 145 | No Picture |
| 2171 | 690202.725 | 4943991.961 | 266.128 | 301.016 | 145 | No Picture |
| 3171 | 690190.595 | 4944001.236 | 263.58 | 298.468 | 145 | No Picture |
| 3172 | 683584.001 | 4941753.756 | 264.482 | 299.446 | 145 | No Picture |
| 2172 | 683583.99 | 4941747.437 | 264.784 | 299.748 | 145 | No Picture |
| 3173 | 683338.642 | 4939917.258 | 261.114 | 296.094 | 145 | No Picture |
| 2173 | 683344.045 | 4939919.397 | 261.448 | 296.428 | 145 | No Picture |
| 3174 | 684258.188 | 4937525.599 | 258.337 | 293.329 | 145 | No Picture |
| 2174 | 684263.531 | 4937525.203 | 259.346 | 294.338 | 145 | No Picture |
| Point | Easting | Northing | Ellips | H (CGVD2013) | Julian day | Comment |
| 1939-07 | 657156.021 | 4957244.824 | 240.11 | 275.073 | 146 | Aii Control - Base |
| 3164 | 657118.622 | 4957343.578 | 227.715 | 262.678 | 146 | Tall grass |
| 2164 | 657112.531 | 4957341.092 | 228.109 | 263.072 | 146 | Hard surface |
| 2165 | 657549.620 | 4957848.251 | 237.195 | 272.148 | 146 | Hard surface |
| 3165 | 657548.934 | 4957840.418 | 236.679 | 271.632 | 146 | Tall grass |
| 3166 | 657676.314 | 4956108.512 | 240.085 | 275.061 | 146 | Short grass |
| 2166 | 657628.373 | 4956083.612 | 240.468 | 275.445 | 146 | Paved road |
| 2167 | 645739.457 | 4950556.161 | 208.006 | 243.138 | 146 | Paved road |
| 3167 | 645704.777 | 4950584.422 | 208.145 | 243.277 | 146 | Short grass |
| 3168 | 645634.844 | 4957424.363 | 207.017 | 242.059 | 146 | Short grass |
| 2168 | 645644.924 | 4957404.553 | 206.773 | 241.815 | 146 | Paved road |
| 3169 | 649949.240 | 4962954.327 | 203.22 | 238.151 | 146 | Tall grass |
| 2169 | 649936.243 | 4962963.512 | 203.193 | 238.124 | 146 | Hard surface |

# Appendix C – Published Station Coordinate Sheets