User Guide

for

Ontario Hydro Network

Land Information Ontario Data Class

Provincial Mapping

Mapping and Information Resources Branch

Corporate Management and Information Division

Ministry of Natural Resources and Forestry

2024-05-24

Disclaimer

This technical documentation has been prepared by His Majesty the King in right of Ontario as represented by the Ministry of Natural Resources and Forestry (the “Ministry”). No warranties or representations, express or implied, statutory or otherwise shall apply or are being made by the Ministry with respect to the documentation, its accuracy or its completeness. In no event will the Ministry be liable or responsible for any lost profits, loss of revenue or earnings, claims by third parties or for any economic, indirect, special, incidental, consequential or exemplary damage resulting from any errors, inaccuracies or omissions in this documentation; and in no event will the Ministry’s liability for any such errors, inaccuracies or omissions on any particular claim, proceeding or action, exceed the actual consideration paid by the claimant involved to the Ministry for the materials to which this instructional documentation relates. Save and except for the liability expressly provided for above, the Ministry shall have no obligation, duty or liability whatsoever in contract, tort or otherwise, including any liability or negligence. The limitations, exclusions and disclaimers expressed above shall apply irrespective of the nature of any cause of action, demand or action, including but not limited to breach of contract, negligence, strict liability, tort or any other legal theory, and shall survive any fundamental breach or breaches.

Cette publication spécialisée n’est disponible qu’en anglais.

Additional Information

This document does not comply with all the applicable guidelines for accessible digital documents. For an alternative format please email Provincial Mapping at pmu@ontario.ca.

For more information about this document, please contact Provincial Mapping at pmu@ontario.ca.

Executive Summary

Key Words

Canals, Creeks, Drainage, Hydrography, Hydrology, Lakes, OHN, Ontario Hydro Network, Rivers, Shorelines, Streams, Water, Water Flow, Water Management, Waterbody, Watercourse

Abstract

This user guide details the specifications for the Ontario Hydro Network (OHN) Geospatial Ontario (GEO) Data Class. The OHN consists of 6 provincial data classes and 21 small scale cartographic data classes.  The 6 OHN provincial data classes, which have their own distinct metadata record, are:

•	OHN Waterbody

•	OHN Watercourse

•	OHN Shoreline

•	OHN Hydrographic Point

•	OHN Hydrographic Line

•	OHN Hydrographic Poly

The OHN small scale cartographic products consist of 3 distinct data classes (Waterbody, Watercourse and Shoreline) and corresponding 3 metadata records for each of the following 7 scales:

•	1:100,000 (100K)

•	1:200,000 (200K)

•	1:500,000 (500K)

•	1:1,000,000 (1M)

•	1:2,000,000 (2M)

•	1:5,000,000 (5M)

•	1:10,000,000 (10M)

Table of Contents

Disclaimer	2

Additional Information	2

Executive Summary	3

Key Words	3

Abstract	3

Table of Contents	4

List of Figures	7

List of Tables	7

List of Acronyms	8

1. Introduction	10

2. Objectives	11

3. Data Classes:	11

OHN – Waterbody	11

OHN – Watercourse	12

OHN – Shoreline	17

OHN – Hydrographic Point, Line, Polygon	18

OHN – Common Attributes	19

OHN – Small Scale Cartographic Products	19

4. Model Diagrams:	22

OHN	22

OHN – Waterbody	22

OHN – Watercourse	23

OHN – Shoreline	24

OHN – Hydrographic Poly	25

OHN – Hydrographic Line	26

OHN – Hydrographic Point	27

OHN – Small Scale Cartographic Products	28

OHN – Small Scale Waterbody	28

OHN – Small Scale Watercourse	29

OHN – Small Scale Shoreline	30

5. Data Description	31

OHN	31

OHN – Waterbody	31

OHN – Watercourse	32

OHN – Shoreline	35

OHN – Hydrographic Poly	35

OHN – Hydrographic Line	36

OHN – Hydrographic Point	37

OHN – Small Scale Cartographic Products	38

OHN – Small Scale Waterbody	38

OHN – Small Scale Watercourse	39

OHN – Small Scale Shoreline	40

6. Links to Additional Information	42

APPENDIX A: GEO Lookup Tables and Values	43

OHN	43

OHN – Waterbody	43

OHN – Watercourse	47

OHN – Hydrographic Poly	48

OHN – Hydrographic Line	50

OHN – Hydrographic Point	52

OHN – Small Scale Cartographic Products	53

OHN – Small Scale Waterbody	53

OHN – Small Scale Watercourse	58

OHN – Small Scale Shoreline	58

APPENDIX B: Table Key Dependency and Relationship Index	59

APPENDIX C: Frequently Asked Questions	61

How can I extract Islands from the OHN?	61

How are Wetland data related to the OHN?	62

How are Constructed Drains data related to the OHN?	62

Does the OHN support network analysis?	63

Where did the names come from?	63

How authoritative is the OHN Official Name Label?	63

How is the Official Name and Official Alternate Name attribute used for labeling?	63

Why are all water names not included in the OHN?	64

Why is the OHN geometry different from the GEL?	64

Are there other dam layers?	64

List of Figures

Figure 1: Examples of watercourse line symbology.	13

Figure 2: Virtual Flow line symbology.	14

Figure 3: Flow Classifications – Primary and Flow Gap.	16

Figure 4: Virtual Flow gap and Flow gap examples.	16

Figure 5: Example of shoreline symbology that demonstrates the differences between OHN-Waterbody polygon outlines and OHN-Shoreline.	18

Figure 6: Primary Key example	60

Figure 7: Foreign Key example	60

Figure 8: Alternate Key example	60

Figure 9: Example of an Identifying Relationship	61

Figure 10: Example of a Non-identifying Relationship	61

Figure 11: Model Diagram Relationship Symbology Table	61

Figure 12: OHN Waterbody with holes that represent islands	62

Figure 13: Polygon around the lake and the resulting island polygons after erasing the lake	63

List of Tables

Table 1: Feature types found as a point, line or polygon marked by ‘X’.	18

Table 2: List of the 21 OHN small scale cartographic data classes	21

List of Acronyms

CGNDB: Canadian Geographical Names Data Base

GIS: Geographic Information System

GEO: Geospatial Ontario

GNO: Geographic Names Ontario

LUT: Lookup Table

MIRB: Mapping and Information Resources Branch

MNR: Ontario Ministry of Natural Resources

NHN: National Hydro Network

NRCan: Natural Resources Canada

OBM: Ontario Base Maps

ODI: Ontario Dam Inventory

OGF: Ontario Geospatial Feature

OGNB: Ontario Geographic Names Board

OHN: Ontario Hydro Network

OHNHLN: OHN – Hydrographic Line

OHNHPOLY: OHN – Hydrographic Poly

OHNHPT: Hydrographic Point

OHNSLIN: OHN - Shoreline

OHNWBDY: OHN – Waterbody

OHNWCRS: OHN - Watercourse

OIH: Ontario Integrated Hydrology

OPS: Ontario Public Service

OWES: Ontario Wetland Evaluation System

PM: Provincial Mapping

ROD: Regional Operations Division

1. Introduction

The User Guide for the Ontario Hydro Network (OHN) contains an overview of all the OHN data classes and highlights details of which users of hydro data should be aware. Hydro data is defined as surface water information including lakes, rivers, streams, barriers, and obstacles. The OHN data is distributed through Geospatial Ontario (GEO).  The OHN is continually updated.

The OHN is a provincial data product that is maintained following a specification for feature capture.  As the OHN is updated with improved source data, along with improvements to its currency and accuracy, feature-capture is also improving.  Many areas that were originally captured from sources at 1:20,000 or 1:50,000 scales are now being represented with feature dimensions criteria equivalent to 1:10,000, or better.

The OHN consists of the following six data classes:

OHN - Watercourse – Line features that represent flowing surface water (natural or constructed).

OHN - Waterbody – Polygon features that represent bodies of surface water (natural or constructed).

OHN - Shoreline – Lines that represent the shorelines of waterbodies. The shoreline is derived from the OHN - Waterbody data.

OHN - Hydrographic Point – Point features (natural or constructed) that occur on waterbodies or watercourses which may pose hazards or impediments to water flow or navigation.

OHN - Hydrographic Line – Line features (natural or constructed) that occur on waterbodies or watercourses which may pose hazards or impediments to water flow or navigation.

OHN - Hydrographic Poly – Polygon features (natural or constructed) that occur on waterbodies or watercourses which may pose hazards or impediments to water flow or navigation.

The OHN data is provided to Natural Resources Canada (NRCan) for incorporation into the National Hydro Network (NHN). The NHN specifications differ from the OHN and therefore the NHN geometry and attributes may differ from the OHN.

Geographic names on the NHN data posted to Geogratis for Ontario are sourced from the Canadian Geographical Names Data Base (CGNDB). As a result, geographic names on the NHN product for Ontario may differ from the Geographic Names Ontario (Geonames or GNO).

The official names of geographic features (for example, lakes, bays, rivers, creeks, islands, etc.) and unincorporated places (for example, villages, hamlets, etc.) in Ontario as authorized through the Ontario Geographic Names Board (OGNB) Act, can be obtained from the Geographic Names Ontario (Geonames or GNO) data by searching the Ontario GeoHub for “Geonames”.

The OHN data class layers are the authoritative source for water geometry for Ontario.

2. Objectives

This guide is intended for users with a general interest in the OHN GEO Data Class. The remainder of this document describes the extent and context of the information collected for the data class.

3. Data Classes:

OHN – Waterbody

Waterbody Type

Mandatory attribute. The nature of a body of water defined according to its water velocity and usage. Available values are:

Lake (default value) - A natural, usually flat, open body of water, which excludes wetlands, islands, surface rocks or other hazards to water flow and/or navigation.

River - A natural body of water usually narrower than a lake through which water may flow.

Canal - An artificial watercourse serving as a navigable waterway or to channel water.

Reservoir - A wholly or partially human-made body of water for storing and/ or regulating and controlling water.

Kettle Lake - A shallow, sediment-filled lake formed by retreating glaciers or draining floodwaters.

Pond - A body of standing water, usually smaller than a lake.  This feature type is used to differentiate non-lake features from lakes, including: irrigation ponds, reservoirs, flooded aggregate pits and quarries.

Beaver Pond - A pond that is created by a beaver dam.

Ocean - A coastal waterbody (Hudson Bay and James Bay).

Permanency

An indication of the permanency of the water feature.

Permanent (default value) - A waterbody is considered permanent if it contains water at least 9 months of the year.

Intermittent - A waterbody is considered intermittent if it contains water less than 9 months of the year.

OHN – Watercourse

Watercourse features are line features (natural or constructed) that represent the location of flowing water. The OHN Watercourse features may be used for cartographic or network analysis purposes. The bullets below explain some options to consider depending on the use of the data.

Flow direction – The watercourse lines are oriented in the direction that water normally flows across the landscape. The direction is usually governed by changes in ground elevation – water flows downhill. However, dams and other structures may redirect the flow.

- Cartographic products typically symbolize streams as blue lines and are not concerned with the flow direction (Figure 1: a).

- For network analysis the flow direction is crucial for tracing up or down stream, stream ordering, creating watersheds, etc. The watercourse lines may be symbolized as lines with arrows to display the direction of water flow (Figure 1: b).

Figure 1: Examples of watercourse line symbology.

Connectivity – Connectivity is crucial for network analysis. Streams entering and exiting waterbodies are connected with virtual flow lines that occur on the waterbody polygons.

- Cartographic products typically ignore the virtual flow lines so that they are not displayed in the waterbodies (Figure 2: a). Alternatively, the virtual flow line may be symbolized as a dashed line or as a different colour (Figure 2: b).

Figure 2: Virtual Flow line symbology.

Watercourse Type

Watercourses are line features (natural or constructed) that can be defined as flowing water within a defined channel with beds or banks and usually discharges itself into some other watercourse or waterbody. A digitized watercourse can also be virtual, inferring water flow within a waterbody or inferring water flow over land where a defined channel is absent.

Stream (default) - A natural body of water (such as a river, stream or creek) through which water flows.

Ditch - A small, open constructed channel for the purpose of conveying water.  Open conduits, constructed drains, irrigation channels, aqueducts, penstocks, flumes and sluices are included.

Virtual Flow - An inferred watercourse feature inside a waterbody needed to maintain the continuity of water flow between adjacent water features.  These will be lines through a waterbody that connect all adjacent water features.

Virtual Connector - An inferred watercourse feature outside of a waterbody needed to maintain the continuity of water flow between adjacent water features.  These will be lines where flow is known to exist but is not directly mapped, such as underground conduits.

Flow Direction Verified

An indicator of known correct geometry direction. This is a Yes/No field with a default value of ‘No’.

Permanency

An indication of the permanency of the water feature.

Permanent – (default) A watercourse is considered permanent if it contains flowing water at least 9 months of the year.

Intermittent – A watercourse is considered intermittent if it contains flowing water less than 9 months of the year.

Note: Virtual Flow segments must be assigned the permanency value of ‘Permanent’.

Flow Classification

The Flow Classification attribute is used to identify the different pieces of a loop (sometimes referred to as a ‘cycle’). A loop is created when one watercourse line diverges into two lines and further downstream converges back into one line (Figure 3: a). Loops may occur inside (with virtual flow lines) or outside of waterbodies. The following bullets explain the three pieces of a loop.

Primary (default) – All watercourse lines are initially considered to be primary. Only inside of a loop are some of the lines classified otherwise.

Flow Gap – Loops need to have a piece removed (to break the loop) for some network analysis processes. The Flow Gap value is assigned to the first watercourse line on the secondary segment of the divergence at no less than 43 m measured in a straight line from the divergence (Figure 3: c). The Flow Gap value may also be assigned to virtual flows where there is a divergence. If the Flow Gap intersects the shoreline, it will continue on land for no less than 43 m measured in a straight line (Figure 4). If required, the Flow Gap lines can be removed before performing network analysis.

Figure 3: Flow Classifications – Primary and Flow Gap.

Figure 4: Virtual Flow gap and Flow gap examples.

Note: Watercourse segments that are isolated from the flow network may form incomplete loops. They may diverge into two or more streams and not converge back together. In these cases, the same rules already explained will apply for primary and flow gap.

This attribute is typically used for network analysis purposes and may be ignored for most cartographic purposes.

Secondary – currently there are watercourse segments classified as secondary flow downstream from flow gaps. This is in the process of being discontinued.

OHN – Shoreline

The OHN Shoreline data class is derived from the OHN Waterbody data class. It can be used to cartographically display the line where water meets the land.

The shoreline can differ from the outline of waterbody polygons. For example, a large lake may be divided into several polygons which would result in outlines passing through the lake. Another example would be where a lake polygon is adjacent to a river polygon. The adjacent outlines would appear to run through the water (Figure 4: a).

The cartographic solution is to symbolize the OHN - Shoreline as a dark line and leave the OHN - Waterbody outlines transparent (Figure 4: b).

Figure 5: Example of shoreline symbology that demonstrates the differences between OHN-Waterbody polygon outlines and OHN-Shoreline.

OHN – Hydrographic Point, Line, Polygon

The natural or constructed features that occur on waterbodies and/or watercourses and may pose hazards or impediments to water flow and/or navigation. See Table 1 for details.

| Feature Type | Point | Line | Polygon |
| --- | --- | --- | --- |
| Falls | X | X | Not Applicable |
| Rapids | X | X | X |
| Rocks | X | Not Applicable | Not Applicable |
| Dam | Not Applicable | X | X |
| Hydro Wall | Not Applicable | X | X |
| Lock-Gate | Not Applicable | X | Not Applicable |
| Sea Lamprey Barrier | X | X | Not Applicable |

Table 1: Feature types found as a point, line or polygon marked by ‘X’.

Hydrographic Types

Falls - A perpendicular or steep drop in a body of water over which water flows.

Rapids - A fast-flowing, often turbulent, section of a body of water, generally containing exposed rocks or boulders.

Rocks - A rock or earthen formation always visible above the water surface.

Dam - A feature representing an obstacle that disturbs or impedes the flow of surface water, excluding beaver dams, water-crossings and culverts.

Hydro Wall - A human-made structure built in a waterbody and possibly extending onshore, or built along a waterbody forming the shoreline, that does not allow water to pass under it, but allows water to pass around it.  Hydro walls may extend along and/or may be contained within a waterbody, and may include wharfs, docks, piers, jetties, headwalls, groynes, locks, berms or embankments.

Lock-Gate - A gate on a navigable canal used to raise or lower the water level so that boats may pass from one level to another.

Sea Lamprey Barrier - A feature on a watercourse or waterbody that forms a barrier (either electrical or physical) to the migration of Sea Lamprey.

OHN – Common Attributes

The attributes listed in this section are found on all OHN data classes except for Shoreline. Attributes that are specific to data classes can be found in earlier sections.

OGF ID – A unique numeric provincial identifier assigned to each object.

Location Accuracy - The degree of conformity or closeness of a measurement within the database to its true value in the world.

Verification Status - An indication as to whether a qualified employee has verified the existence of the feature.

Verification Status Date - The date that the geographic unit was verified/validated.

General Comments - A 2000-character field for general comments about the feature.

Source Detail - A 254-character field that contains what part of the source pertains to the feature. Examples: Summary data from a database, pages in a book or atlas, figure number and page from a publication, a section of a map, or record in a database.

- Effective Date/Time - Date/time that the record was created in the GEO database.

OHN – Small Scale Cartographic Products

The OHN small scale consists of 3 distinct data classes (Waterbody, Watercourse and Shoreline) and 3 corresponding metadata records for each of the following 7 scales:

1:100,000 (100K)

1:200,000 (200K)

1:500,000 (500K)

1:1,000,000 (1M)

1:2,000,000 (2M)

1:5,000,000 (5M)

1:10,000,000 (10M)

| Watercourse | Waterbody | Shoreline |
| --- | --- | --- |
| OHN_100K_Watercourse | OHN_100K_Waterbody | OHN_100K_Shoreline |
| OHN_200K_Watercourse | OHN_200K_Waterbody | OHN_200K_Shoreline |
| OHN_500K_Watercourse | OHN_500K_Waterbody | OHN_500K_Shoreline |
| OHN_1M_Watercourse | OHN_1M_Waterbody | OHN_1M_Shoreline |
| OHN_2M_Watercourse | OHN_2M_Waterbody | OHN_2M_Shoreline |
| OHN_5M_Watercourse | OHN_5M_Waterbody | OHN_5M_Shoreline |
| OHN_10M_Watercourse | OHN_10M_Waterbody | OHN_10M_Shoreline |

Table 2: List of the 21 OHN small scale cartographic data classes

The small scale OHN products are intended to support cartographic and web mapping services.  This data is not designed for analytical purposes.  Users that require OHN data for analytical purposes should use the provincial specification OHN data classes.

It should be noted that the OHN small scale cartographic products contain data that extends beyond the Ontario provincial boundary and includes additional data sourced from NRCan (for adjacent provinces and territories) and from the United States Geologic Service (for selected US states).  Please refer to the OHN small scale metadata records for further details on these sources.

4. Model Diagrams:

OHN

OHN – Waterbody

Physical Model Diagram for OHN - Waterbody (OHNWBDY)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Watercourse

Physical Model Diagram for OHN - Watercourse (OHNWCRS)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Shoreline

Physical Model Diagram for OHN – Shoreline (OHNSLIN)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

OHN – Hydrographic Poly

Physical Model Diagram for OHN – Hydrographic Poly (OHNHPOLY)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Hydrographic Line

Physical Model Diagram for OHN – Hydrographic Line (OHNHLN)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Hydrographic Point

Physical Model Diagram for OHN – Hydrographic Point (OHNHPT)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Small Scale Cartographic Products

OHN – Small Scale Waterbody

Physical Model Diagram for OHN Small Scale Class – Waterbody

(OHN100WB, OHN200WB, OHN500WB, OHN1MWB, OHN2MWB, OHN5MWB, OHN10MWB)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Small Scale Watercourse

Physical Model Diagram for OHN Small Scale Class – Watercourse

(OHN100WC, OHN200WC, OHN500WC, OHN1MWC, OHN2MWC, OHN5MWC, OHN10MWC)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

OHN – Small Scale Shoreline

Physical Model Diagram for OHN Small Scale Class – Shoreline

(OHN100SL, OHN200SL, OHN500SL, OHN1MSL, OHN2MSL, OHN5MSL, OHN10MSL)

Model Effective Date 2012-JUN-14

Note: Where applicable to this model:

Tables with a “_LIST” extension (shaded yellow) = Lookup Table (LUT)

Additional feature-level metadata may be stored in other ‘Shared’ tables.  Please refer to the Class Common Tables diagram for more information.

For table key dependency and relationships, see Appendix B

5. Data Description

The main business area table(s) for this model can be found below.  Associated lookup tables will be listed in Appendix A of this document and are hyperlinked where referenced in this section.

OHN

OHN – Waterbody

OHN_WATERBODY_FT

Waterbodies are polygon features (natural and constructed) that describe various realizations of surface water.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | System generated identifier, unique at the application level. |
| WATERBODY_TYPE | VARCHAR2(20) | Yes | WBDY_TYPE | The nature of a body of water defined according to its water velocity and usage. Default = Lake |
| OFFICIAL_NAME_LABEL | VARCHAR2(100) | No | OFF_NAME | This field is a concatenation of the Geographic Name Extent Fields "Official Name" and "Alternate Name". |
| GEL_NAME_IDENT | VARCHAR2(32) | No | GEL_IDENT | A unique, 32 length, alpha-numeric identifier used to distinguish an object. |
| PERMANENCY | VARCHAR2(20) | Yes | PERMANENCY | An indication of the permanency of a water feature. Default = Permanent Permanent - Permanent water features are known to exist for at least 9 months a year. Intermittent - Intermittent water features are known to exist for less than 9 months a year. VALID VALUES: Permanent, Intermittent |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| VERIFICATION_STATUS _FLG | VARCHAR2(10) | No | VERISTT_FL | An indication as to whether a qualified employee has verified the existence of the geographic unit. VALID VALUES: Verified, Unverified |
| VERIFICATION_STATUS _DATE | DATE | No | VERISTT_DT | Date that the geographic unit was verified/validated. |
| GENERAL_COMMENTS | VARCHAR2(2000) | No | COMMENTS | General comments. |
| SYSTEM_CALCULATED _AREA | NUMBER(16,3) | No | SYS_AREA | The area of a polygon measured in square metres. |
| SYSTEM_CALCULATED _PERIMETER | NUMBER(16,3) | No | SYS_PERIM | The perimeter of a polygon measured in metres. |
| GEOMETRY_UPDATE_DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

OHN – Watercourse

OHN_WATERCOURSE_FT

Watercourses are line features (natural and constructed) that describe various realizations of surface water.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| WATERCOURSE_TYPE | VARCHAR2(20) | Yes | WCRS_TYPE | The type of watercourse. Default = Stream |
| OFFICIAL_NAME_LABEL | VARCHAR2(100) | No | OFF_NAME | This field is a concatenation of the Geographic Name Extent Fields "Official Name" and "Alternate Name". |
| GEL_NAME_IDENT | VARCHAR2(32) | No | GEL_IDENT | A unique, 32 length, alpha-numeric identifier used to distinguish an object. |
| FLOW_DIRECTION _VERIFIED_IND | VARCHAR2(3) | Yes | FLOW_VERIF | An indicator of known correct geometry direction. Default = No VALID VALUES: Yes, No |
| PERMANENCY | VARCHAR2(20) | Yes | PERMANENCY | An indication of the permanency of a water feature. Default = Permanent Permanent - Permanent water features are known to exist for at least 9 months a year. Intermittent - Intermittent water features are known to exist for less than 9 months a year. VALID VALUES: Permanent, Intermittent |
| FLOW_CLASSIFICATION | VARCHAR2(20) | Yes | FLOW_CLASS | This classification defines the primary (main) and secondary (alternate) routes of a water flow network. Secondary streams are indicative of divergent flow where there are multiple routes traversing through a system (eg. a braided stream network or delta). Default = Primary Primary - The main route of a water flow network. Secondary - The alternate route of a water flow network. Flow Gap - A secondary flow segment which exists to represent proper flow routing but can be removed to break loops in divergent flow. VALID VALUES: Primary, Secondary, Flow Gap |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| VERIFICATION_STATUS _FLG | VARCHAR2(10) | No | VERISTT_FL | An indication as to whether a qualified employee has verified the existence of the geographic unit. VALID VALUES: Verified, Unverified |
| VERIFICATION_STATUS _DATE | DATE | No | VERISTT_DT | Date that the geographic unit was verified/validated. |
| GENERAL_COMMENTS | VARCHAR2(2000) | No | COMMENTS | General comments. |
| SYSTEM_CALCULATED _LENGTH | NUMBER(16,3) | No | SYS_LENGTH | The length of a line measured in metres. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

OHN – Shoreline

OHN_SHORELINE_FT

The derived line between water and land.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SHAPE | SDO_GEOMETRY | No | SHAPE | Geometry attribute. |

Model Subject Area does not have any business area tables.

OHN – Hydrographic Poly

OHN_HYDROGRAPHIC_POLY_FT

Natural and constructed features which occur on waterbodies and/or watercourses and may pose hazards or impediments to water flow and/or navigation.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| HYDROGRAPHIC_POLY _TYPE | VARCHAR2(20) | Yes | POLY_TYPE | The type of natural or constructed features that occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| VERIFICATION_STATUS _FLG | VARCHAR2(10) | No | VERISTT_FL | An indication as to whether a qualified employee has verified the existence of the geographic unit. VALID VALUES: Verified, Unverified |
| VERIFICATION_STATUS _DATE | DATE | No | VERISTT_DT | Date that the geographic unit was verified/validated. |
| GENERAL_COMMENTS | VARCHAR2(2000) | No | COMMENTS | General comments. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

Model Subject Area does not have any business area tables.

OHN – Hydrographic Line

OHN_HYDROGRAPHIC_LINE_FT

Natural and constructed features which occur on waterbodies and/or watercourses and may pose hazards or impediments to water flow and/or navigation.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| HYDROGRAPHIC_LINE _TYPE | VARCHAR2(20) | Yes | LINE_TYPE | The type of natural or constructed features that occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| VERIFICATION_STATUS _FLG | VARCHAR2(10) | No | VERISTT_FL | An indication as to whether a qualified employee has verified the existence of the geographic unit. VALID VALUES: Verified, Unverified |
| VERIFICATION_STATUS _DATE | DATE | No | VERISTT_DT | Date that the geographic unit was verified/validated. |
| GENERAL_COMMENTS | VARCHAR2(2000) | No | COMMENTS | General comments. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

Model Subject Area does not have any business area tables.

OHN – Hydrographic Point

OHN_HYDROGRAPHIC_POINT_FT

Natural and constructed features which occur on waterbodies and/or watercourses and may pose hazards or impediments to water flow and/or navigation.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| HYDROGRAPHIC_POINT _TYPE | VARCHAR2(20) | Yes | POINT_TYPE | The type of natural or constructed features that occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| VERIFICATION_STATUS _FLG | VARCHAR2(10) | No | VERISTT_FL | An indication as to whether a qualified employee has verified the existence of the geographic unit. VALID VALUES: Verified, Unverified |
| VERIFICATION_STATUS _DATE | DATE | No | VERISTT_DT | Date that the geographic unit was verified/validated. |
| GENERAL_COMMENTS | VARCHAR2(2000) | No | COMMENTS | General comments. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

Model Subject Area does not have any business area tables.

OHN – Small Scale Cartographic Products

OHN – Small Scale Waterbody

OHN_100K_WATERBODY_FT, OHN_200K_WATERBODY_FT, OHN_500K_WATERBODY_FT, OHN_1M_WATERBODY_FT, OHN_2M_WATERBODY_FT, OHN_5M_WATERBODY_FT, OHN_10M_WATERBODY_FT

Waterbodies are polygon features (natural and constructed) that describe various realizations of surface water.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| WATERBODY_TYPE | VARCHAR2(20) | Yes | WBDY_TYPE | The nature of a body of water defined according to its water velocity and usage. |
| OFFICIAL_NAME_LABEL | VARCHAR2(100) | No | OFF_NAME | This field is a concatenation of the Geographic Names Ontario Fields "Official Name" and "Alternate Name". |
| GEL_NAME_IDENT | VARCHAR2(32) | No | GEL_IDENT | An identifier from the Geographic Names Ontario (Geonames or GNO). This is a unique, 32 length, alpha-numeric identifier used to distinguish an object. |
| POLITICAL_SUBDIVISION _CODE | VARCHAR2(5) | No | POL_SUBDIV | ISO codes for the representation of countries and their sub-divisions. |
| LABEL_IND | VARCHAR2(3) | Yes | LABEL_IND | Used to identify if the feature is to be labelled Default = No VALID VALUES: Yes, No |
| SYSTEM_CALCULATED _AREA | NUMBER(16,3) | Yes | SYS_AREA | The area of a polygon measured in square metres. |
| SYSTEM_CALCULATED _PERIMETER | NUMBER(16,3) | Yes | SYS_PERIM | The perimeter of a polygon or length of a line measured in metres. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | No | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | EXP_DATE | Date/time the record was loaded into or last modified in the GEO database. |

Model Subject Area does not have any business area tables.

OHN – Small Scale Watercourse

OHN_100K_WATERCOURSE_FT, OHN_200K_WATERCOURSE_FT, OHN_500K_WATERCOURSE_FT, OHN_1M_WATERCOURSE_FT, OHN_2M_WATERCOURSE_FT, OHN_5M_WATERCOURSE_FT, OHN_10M_WATERCOURSE_FT

Watercourses are line features (natural and constructed) that describe various realizations of flowing water.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| WATERCOURSE_TYPE | VARCHAR2(20) | Yes | WCRS_TYPE | The type of watercourse. |
| OFFICIAL_NAME_LABEL | VARCHAR2(100) | No | OFF_NAME | This field is a concatenation of the Geographic Names Ontario Fields "Official Name" and "Alternate Name". |
| GEL_NAME_IDENT | VARCHAR2(32) | No | GEL_IDENT | An identifier from the Geographic Names Ontario (Geonames or GNO). This is a unique, 32 length, alpha-numeric identifier used to distinguish an object. |
| POLITICAL_SUBDIVISION _CODE | VARCHAR2(5) | No | POL_SUBDIV | ISO codes for the representation of countries and their sub-divisions. |
| LABEL_IND | VARCHAR2(3) | Yes | LABEL_IND | Used to identify if the feature is to be labelled Default = No VALID VALUES: Yes, No |
| SYSTEM_CALCULATED _LENGTH | NUMBER(16,3) | Yes | SYS_LENGTH | The perimeter of a polygon or length of a line measured in metres. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | No | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

Model Subject Area does not have any business area tables.

OHN – Small Scale Shoreline

OHN_100K_SHORELINE_FT, OHN_200K_SHORELINE_FT, OHN_500K_SHORELINE_FT, OHN_1M_SHORELINE_FT, OHN_2M_SHORELINE_FT, OHN_5M_SHORELINE_FT, OHN_10M_SHORELINE_FT

The derived line between water and land.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| OGF_ID | NUMBER(13) | Yes | OGF_ID | A unique numeric provincial identifier assigned to each object. |
| POLITICAL_SUBDIVISION_CODE | VARCHAR2(5) | No | POL_SUBDIV | ISO codes for the representation of countries and their sub-divisions. |
| GEOMETRY_UPDATE _DATETIME | DATE | No | GEO_UPD_DT | Date/time the geometry was created or last modified in the source database. |
| EFFECTIVE_DATETIME | DATE | No | EFF_DATE | Date/time the record was created or last modified in the source database. |
| SYSTEM_DATETIME | DATE | Yes | SYS_DATE | Date/time the record was loaded into or last modified in the GEO database. |

6. Links to Additional Information

Access the official Metadata Record by searching Ontario GeoHub for “Ontario Hydro Network, OHN, Waterbody, Watercourse, Shoreline, Hydrographic Poly, Hydrographic Line, Hydrographic Point, Hydrographic, Small Scale Cartographic Products, Small Scale”

APPENDIX A: GEO Lookup Tables and Values

OHN

OHN – Waterbody

LOCATION_ACCURACY_LIST

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| LOCATION_ACCURACY | VARCHAR2(25) | Yes | ACCURACY | The accuracy of the location of the feature at an OBM scale. The degree of conformity or closeness of a measurement to the true value. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

LOCATION_ACCURACY_LIST Permissible Values

| LOCATION ACCURACY | EXPIRY DATETIME |
| --- | --- |
| Not Applicable | Not applicable |
| Over 10,000 metres | Not applicable |
| Within 1 metre | Not applicable |
| Within 10 metres | Not applicable |
| Within 10,000 metres | Not applicable |
| Within 100 metres | Not applicable |
| Within 1000 metres | Not applicable |
| Within 2 metres | Not applicable |
| Within 20 metres | Not applicable |
| Within 200 metres | Not applicable |
| Within 2000 metres | Not applicable |
| Within 5 metres | Not applicable |
| Within 50 metres | Not applicable |
| Within 500 metres | Not applicable |
| Within 5000 metres | Not applicable |
| AC Accurate (to 10m) | 01/12/2007 |
| AP Approximate (to 500m) | 01/12/2007 |
| GE General (to 10,000m) | 01/12/2007 |
| MO Moderate (to 1000m) | 01/12/2007 |
| RE Reliable (to 100m) | 01/12/2007 |
| VA Very Accurate (to 2m) | 01/12/2007 |
| VG Vague (to 100,000m) | 01/12/2007 |
| ^ Data Load | 01/12/2007 |

OHN_WATERBODY_TYPE_LIST

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| WATERBODY_TYPE | VARCHAR2(20) | Yes | WBDY_TYPE | The nature of a body of water defined according to its water velocity and usage. |
| DESCRIPTION | VARCHAR2(254) | No | TYPE_DESCR | Describes the Waterbody Type |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_WATERBODY_TYPE_LIST Permissible Values

| WATERBODY TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Beaver Pond | A pond that is created by a beaver dam. | Not applicable |
| Canal | An artificial watercourse serving as a navigable waterway or to channel water. | Not applicable |
| Kettle Lake | A shallow, sediment-filled lake formed by retreating glaciers or draining floodwaters. | Not applicable |
| Lake | A natural, usually flat, open body of water, which excludes wetlands, islands, surface rocks or other hazards to waterflow and/or navigation. | Not applicable |
| Ocean | A coastal waterbody (Hudson Bay and James Bay). | Not applicable |
| Pond | A body of standing water, usually smaller than a lake. This feature type is used to differentiate non-lake features from lakes, including: irrigation ponds, reservoirs, flooded gravel pits and quarries. | Not applicable |
| Reservoir | A wholly or partially human-made body of water for storing and/ or regulating and controlling water. | Not applicable |
| River | A natural body of water (such as a river, stream or creek) through which water may flow. | Not applicable |

OHN – Watercourse

LOCATION_ACCURACY_LIST

See table under OHN Waterbody

OHN_WATERCOURSE_TYPE_LIST

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| WATERCOURSE_TYPE | VARCHAR2(20) | Yes | WCRS_TYPE | The type of watercourse. |
| DESCRIPTION | VARCHAR2(225) | No | TYPE_DESCR | Describes the Watercourse Type |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_WATERCOURSE_TYPE_LIST Permissible Values

| WATERCOURSE TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Ditch | A small, open constructed channel for the purpose of conveying water. Open conduits, constructed drains, irrigation channels, aqueducts, penstocks, flumes and sluices are included. | Not applicable |
| Stream | A natural body of water (such as a river, stream or creek) through which water flows. | Not applicable |
| Virtual Connector | An inferred watercourse feature needed to maintain the continuity of water flow between adjacent water features. These will be lines where flow is known to exist but is not directly mapped, such as underground conduits. | Not applicable |
| Virtual Flow | An inferred watercourse feature needed to maintain the continuity of water flow between adjacent water features. These will be lines through a waterbody that connect all adjacent water features. | Not applicable |

OHN – Hydrographic Poly

LOCATION_ACCURACY_LIST

See table under OHN Waterbody

OHN_POLY_TYPE_LIST

Describes the Hydrographic Poly Type.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| HYDROGRAPHIC_POLY_TYPE | VARCHAR2(20) | Yes | POLY_TYPE | The type of natural or constructed features that may occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| DESCRIPTION | VARCHAR2(400) | No | DESCR | Describes the Hydrographic Poly Type. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_POLY_TYPE_LIST Permissible Values

| HYDROGRAPHIC POLY TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Dam | A feature representing an obstacle that disturbs or impedes the flow of surface water, excluding beaver dams, water-crossings and culverts. | Not applicable |
| Hydro Wall | A human-made structure built in a waterbody and possibly extending onshore, or built along a waterbody forming the shoreline, that does not allow water to pass under it, but allows water to pass around it. Hydro walls may extend along and/or may be contained within a waterbody, and may include wharfs, docks, piers, jetties, headwalls, groynes, locks, berms or embankments. | Not applicable |
| Rapids | A fast-flowing, often turbulent, section of a body of water, generally containing exposed rocks or boulders. | Not applicable |

OHN – Hydrographic Line

LOCATION_ACCURACY_LIST

See table under OHN Waterbody

OHN_LINE_TYPE_LIST

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| HYDROGRAPHIC_LINE_TYPE | VARCHAR2(20) | Yes | LINE_TYPE | The type of natural or constructed features that may occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| DESCRIPTION | VARCHAR2(400) | No | TYPE_DESCR | Describes the Hydrographic Line Type. |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_LINE_TYPE_LIST Permissible Values

| HYDROGRAPHIC LINE TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Dam | A feature representing an obstacle that disturbs or impedes the flow of surface water, excluding beaver dams, water-crossings and culverts. | Not applicable |
| Falls | A perpendicular or steep drop in a body of water over which water flows. | Not applicable |
| Hydro Wall | A human-made structure built in a waterbody and possibly extending onshore, or built along a waterbody forming the shoreline, that does not allow water to pass under it, but allows water to pass around it. Hydro walls may extend along and/or may be contained within a waterbody, and may include wharfs, docks, piers, jetties, headwalls, groynes, locks, berms or embankments. | Not applicable |
| Lock-Gate | A gate on a navigable canal used to raise or lower the water level so that boats may pass from one level to another. | Not applicable |
| Rapids | A fast-flowing, often turbulent, section of a body of water, generally containing exposed rocks or boulders. | Not applicable |
| Sea Lamprey Barrier | A feature on a watercourse or waterbody that forms a barrier (either electrical or physical) to the migration of Sea Lamprey. | Not applicable |

OHN – Hydrographic Point

LOCATION_ACCURACY_LIST

See table under OHN Waterbody

OHN_POINT_TYPE_LIST

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| HYDROGRAPHIC_POINT_TYPE | VARCHAR2(20) | Yes | POINT_TYPE | The type of natural or constructed features that may occur on waterbodies or watercourses which may pose hazards or impediments to water flow and/or navigation. |
| DESCRIPTION | VARCHAR2(160) | No | TYPE_DESCR | Describes the Hydrographic Point Type |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_POINT_TYPE_LIST Permissible Values

| HYDROGRAPHIC POINT TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Falls | A perpendicular or steep drop in a body of water over which water flows. | Not applicable |
| Rapids | A fast-flowing, often turbulent, section of a body of water, generally containing exposed rocks or boulders. | Not applicable |
| Rocks | A rock or earthen formation always visible above the water surface. | Not applicable |
| Sea Lamprey Barrier | A feature on a watercourse or waterbody that forms a barrier (either electrical or physical) to the migration of Sea Lamprey. | Not applicable |

OHN – Small Scale Cartographic Products

OHN – Small Scale Waterbody

OHN_SS_WATERBODY_TYPE_LIST

The nature of a body of water defined according to its water velocity and usage.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| WATERBODY_TYPE | VARCHAR2(20) | Yes | WBDY_TYPE | The nature of a body of water defined according to its water velocity and usage. |
| DESCRIPTION | VARCHAR2(100) | Yes | DESCR | Describes the Waterbody Type |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN_SS_WATERBODY_TYPE_LIST Permissible. Values

| WATERBODY TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Lake | A natural body of water that is localized to the bottom of basin and moves slowly if it moves at all. | Not applicable |
| Ocean | A coastal waterbody (Hudson Bay and James Bay). | Not applicable |
| River | A natural body of water (such as a river, stream or creek) through which water may flow. | Not applicable |

POLITICAL_SUBDIVISION_LIST

ISO code list for the countries and their sub-divisions.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| POLITICAL_SUBDIVISION_CODE | VARCHAR2(5) | Yes | POL_SUBDIV | ISO codes for the representation of countries and their sub-divisions. |
| DESCRIPTION | VARCHAR2(100) | Yes | DESCR | Describes the Subdivision Code |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

POLITICAL_SUBDIVISION_LIST Permissible Values

| POLITICAL SUBDIVISION CODE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| CA-AB | Canada - Alberta | Not applicable |
| CA-BC | Canada - British Columbia | Not applicable |
| CA-MB | Canada - Manitoba | Not applicable |
| CA-NB | Canada - New Brunswick | Not applicable |
| CA-NL | Canada - Newfoundland and Labrador | Not applicable |
| CA-NS | Canada - Nova Scotia | Not applicable |
| CA-NT | Canada - Northwest Territories | Not applicable |
| CA-NU | Canada - Nunavut | Not applicable |
| CA-ON | Canada - Ontario | Not applicable |
| CA-PE | Canada - Prince Edward Island | Not applicable |
| CA-QC | Canada - Quebec | Not applicable |
| CA-SK | Canada - Saskatchewan | Not applicable |
| CA-YT | Canada - Yukon Territory | Not applicable |
| US-AK | United States - Alaska | Not applicable |
| US-AL | United States - Alabama | Not applicable |
| US-AR | United States - Arkansas | Not applicable |
| US-AZ | United States - Arizona | Not applicable |
| US-CA | United States - California | Not applicable |
| US-CO | United States - Colorado | Not applicable |
| US-CT | United States - Connecticut | Not applicable |
| US-DE | United States - Delaware | Not applicable |
| US-FL | United States - Florida | Not applicable |
| US-GA | United States - Georgia | Not applicable |
| US-HI | United States - Hawaii | Not applicable |
| US-IA | United States - Iowa | Not applicable |
| US-ID | United States - Idaho | Not applicable |
| US-IL | United States - Illinois | Not applicable |
| US-IN | United States - Indiana | Not applicable |
| US-KS | United States - Kansas | Not applicable |
| US-KY | United States - Kentucky | Not applicable |
| US-LA | United States - Louisiana | Not applicable |
| US-MA | United States - Massachusetts | Not applicable |
| US-MD | United States - Maryland | Not applicable |
| US-ME | United States - Maine | Not applicable |
| US-MI | United States - Michigan | Not applicable |
| US-MN | United States - Minnesota | Not applicable |
| US-MO | United States - Missouri | Not applicable |
| US-MS | United States - Mississippi | Not applicable |
| US-MT | United States - Montana | Not applicable |
| US-NC | United States - North Carolina | Not applicable |
| US-ND | United States - North Dakota | Not applicable |
| US-NE | United States - Nebraska | Not applicable |
| US-NH | United States - New Hampshire | Not applicable |
| US-NJ | United States - New Jersey | Not applicable |
| US-NM | United States - New Mexico | Not applicable |
| US-NV | United States - Nevada | Not applicable |
| US-NY | United States - New York | Not applicable |
| US-OH | United States - Ohio | Not applicable |
| US-OK | United States - Oklahoma | Not applicable |
| US-OR | United States - Oregon | Not applicable |
| US-PA | United States - Pennsylvania | Not applicable |
| US-RI | United States - Rhode Island | Not applicable |
| US-SC | United States - South Carolina | Not applicable |
| US-SD | United States - South Dakota | Not applicable |
| US-TN | United States - Tennessee | Not applicable |
| US-TX | United States - Texas | Not applicable |
| US-UT | United States - Utah | Not applicable |
| US-VA | United States - Virginia | Not applicable |
| US-VT | United States - Vermont | Not applicable |
| US-WA | United States - Washington | Not applicable |
| US-WI | United States - Wisconsin | Not applicable |
| US-WV | United States - West Virginia | Not applicable |
| US-WY | United States - Wyoming | Not applicable |

OHN – Small Scale Watercourse

OHN_SS_WATERCOURSE_TYPE_LIST

The type of watercourse.

| Column Name | Column Type | Mandatory | Short Name | Description |
| --- | --- | --- | --- | --- |
| WATERCOURSE_TYPE | VARCHAR2(20) | Yes | WCRS_TYPE | The type of watercourse. |
| DESCRIPTION | VARCHAR2(100) | Yes | DESCR | Describes the Watercourse Type |
| EFFECTIVE_DATETIME | DATE | Yes | EFF_DATE | Date/time the record was created or last modified in the source database. DEFAULT: SYSDATE |
| EXPIRY_DATETIME | DATE | No | EXP_DATE | Date/time that the record was expired from use. |

OHN SS_WATERCOURSE_TYPE_LIST Permissible Values

| WATERCOURSE TYPE | DESCRIPTION | EXPIRY DATETIME |
| --- | --- | --- |
| Stream | A natural body of water (such as a river, stream or creek) through which water flows. | Not applicable |
| Virtual Flow | An inferred watercourse feature needed to maintain the continuity of water flow. | Not applicable |

POLITICAL_SUBDIVISION_LIST

See table under OHN Small Scale Waterbody

OHN – Small Scale Shoreline

POLITICAL_SUBDIVISION_LIST

See table under OHN Small Scale Waterbody

APPENDIX B: Table Key Dependency and Relationship Index

Keys are used to link or make relationships between tables through one or more columns. A Parent Table is the source of unique key columns used by a Child Table. The main key types are described below.

Primary Key (PK):

A Primary Key (PK) is a column that uniquely identifies each record in a table. There can be more than one PK defined for a table when additional rules for record uniqueness are needed. Primary Keys cannot contain null values.

Figure 6: Primary Key example

Foreign Key (FK):

When a Primary Key from a Parent Table appears as a column in the Child Table it is referred to as a Foreign Key (FK). The FK may retain the same name of the Parent Table column, or it may be renamed.

Figure 7: Foreign Key example

Alternate Key (AK):

Alternate Keys (AK) are secondary unique identifiers in a Parent Table. A Child Table can link to a Parent Table AK and use it as a Foreign Key (FK). The FK may retain the same name of the Parent Table column, or it may be renamed. Alternate Keys allow null values.

Figure 8: Alternate Key example

There are two relationship type options, identifying and non-identifying, which are described below.

An Identifying Relationship occurs when a column from the Parent Table is added to the key section of the Child Table. Though technically a Foreign Key (FK) in the child table, it now forms part of the Primary Key (PK). Identifying Relationships are symbolized with a solid line.

Figure 9: Example of an Identifying Relationship

A Non-identifying Relationship occurs when a column from the Parent Table is added to the main body of the Child Table where it is treated as a Foreign Key (FK). Non-identifying Relationships are symbolized with a dashed line.

Figure 10: Example of a Non-identifying Relationship

The relationships between tables are set up with cardinality rules describing if the feature record in the Parent Table can relate to zero, one or more records in the associated Child Table and vice versa. These rules are symbolized in the model diagram as follows.

Figure 11: Model Diagram Relationship Symbology Table

APPENDIX C: Frequently Asked Questions

How can I extract Islands from the OHN?

The OHN does not contain island polygons. Islands are represented as holes in waterbodies.

Figure 12: OHN Waterbody with holes that represent islands

The following steps are an example of how to create island polygons from waterbody polygons that have holes.

- Create a polygon larger than the extent of the lakes being used

- Erase the lakes from the large polygon

- Convert the result from a multi-part feature to single-part features

- Delete the outer feature which was the outer part of the large polygon and that will leave the island polygons

Figure 13: Polygon around the lake and the resulting island polygons after erasing the lake

How are Wetland data related to the OHN?

The Wetland data are managed independently from other hydro data classes and may overlap waterbodies or other hydro data.

Wetland data is not part of the OHN. The Wetland data class is available from GEO.

The Wetland data class provides the spatial representation and basic information for wetlands in Ontario. The attributes identify which wetlands have been evaluated through the Ontario Wetland Evaluation System (OWES), and of those evaluated wetlands which ones have been designated as Provincially Significant.

How are Constructed Drains data related to the OHN?

Constructed Drains are watercourses in the form of ditches, natural watercourses that have been modified to improve drainage, or buried tile systems.

The Constructed Drains data class is not part of the OHN. It is disseminated from the GEO warehouse and is maintained by the Ontario Ministry of Agriculture, Food and Rural Affairs for monitoring modifications and new construction.

Some Constructed Drain features will overlap or duplicate OHN Watercourse features. It cannot be incorporated into the OHN Watercourse layer because their data model contains different attributes and their data specifications are for a larger scale product.

Does the OHN support network analysis?

Yes, the OHN Watercourse data class contains directionality and connectivity to support a geometric network for hydrological analysis.

For details on how to create a geometric network, see your GIS software documentation or contact support for your GIS software.

The Ontario Integrated Hydrology‘s Enhanced Watercourse layer is an example of a geometric network derived from a dated copy of the OHN Watercourse. Search Ontario GeoHub for “Ontario Integrated Hydrology Data”.

Where did the names come from?

Within the OHN_Waterbody and OHN_Watercourse data classes the official names were extracted from the Geographic Names Ontario (Geonames or GNO) data class.

The small scale OHN layers source names from several contributing datasets.

Ontario: OHN (GNO)

Manitoba, Nunavut, Quebec:  National Hydro Network (NHN) – Names in the NHN come from the Canadian Geographic Names Data Base (CGNDB)

USA: The names and geometry come from the National Hydrography Dataset (NHD)

How authoritative is the OHN Official Name Label?

The source of OFFICIAL_NAME_LABEL and GEL_NAME_IDENT attributes on the OHN - Waterbody and OHN - Watercourse features is GNO. Users should not use the OHN as the authoritative source for official water names. Refer to the GNO data class to verify official names.

Note: Due to duplicate OFFICIAL_NAME_LABEL values, to select an entire named feature, select all waterbodies and watercourse features that share the same GEL_NAME_IDENT.  In the Geographic Names Ontario data class, the named extent will be one feature but the OHN segmentation may require the name to span multiple features in multiple layers.  A single name may be attached to more than one waterbody as well as multiple watercourse features.

How is the Official Name and Official Alternate Name attribute used for labeling?

If an OHN feature has an official name and an official alternate name, then the OFFICIAL_NAME_LABEL will contain the official name and the official alternate in parenthesis, e.g.  Lake Ontario (lac Ontario).  Very few named extents will have an official alternate name.

Why are all water names not included in the OHN?

The OHN only supports one name per water feature. For more detailed water name information see the GNO.

A group of OHN features such as a group of lakes may be considered as one extent in the GNO and would then be represented as one multi-part feature in the GNO data class (e.g. Kawartha Lakes). Some multi-part GNO features (e.g. a group of lake polygons with a single name, like “Kawartha Lakes”) will have individual GNO features for each part (e.g. Pigeon Lake, Stony Lake, etc.). In these cases the OHN feature will only store one official name in the Official Name Label field. The more detailed name (e.g. Pigeon Lake) will be stored on the OHN feature. However, channels, bays and other similar more detailed GNO feature names will NOT be stored on the OHN feature.

Why is the OHN geometry different from the GNO?

In many instances, the geometry found in the GEL was copied from the OHN data classes. When the OHN is updated with new geometry, the geometry in GNO remains unchanged. Staff in the Parcel Mapping and Georeferencing Unit must review the changes in the OHN before adopting any changes to the GNO.

Are there other dam layers?

The Ontario Dam Inventory (ODI) is a point layer with basic attributes such as dam name and ownership.  Search Ontario GeoHub for “Ontario Dam Inventory”.