User Guide

for

Ontario Digital Surface Model (Lidar-Derived)

Geospatial Ontario Dataset

Geospatial Ontario

Digital, Data, Information and Business Services Branch

Corporate Management and Information Division

Ministry of Natural Resources

2025-06-08

Disclaimer

This technical documentation has been prepared by His Majesty the King in right of Ontario as represented by the Ministry of Natural Resources (the “Ministry”). No warranties or representations, express or implied, statutory or otherwise shall apply or are being made by the Ministry with respect to the documentation, its accuracy or its completeness. In no event will the Ministry be liable or responsible for any lost profits, loss of revenue or earnings, claims by third parties or for any economic, indirect, special, incidental, consequential or exemplary damage resulting from any errors, inaccuracies or omissions in this documentation; and in no event will the Ministry’s liability for any such errors, inaccuracies or omissions on any particular claim, proceeding or action, exceed the actual consideration paid by the claimant involved to the Ministry for the materials to which this instructional documentation relates. Save and except for the liability expressly provided for above, the Ministry shall have no obligation, duty or liability whatsoever in contract, tort or otherwise, including any liability or negligence. The limitations, exclusions and disclaimers expressed above shall apply irrespective of the nature of any cause of action, demand or action, including but not limited to breach of contract, negligence, strict liability, tort or any other legal theory, and shall survive any fundamental breach or breaches.

Additional Information

This document does not comply with all the applicable guidelines for accessible digital documents. For an alternative format please email Geospatial Ontario at geospatial@ontario.ca.

For more information about this document, please contact Geospatial Ontario at geospatial@ontario.ca.

Executive Summary

Key Words

OMAFRA Lidar, Lidar Eastern Acquisition Project, LEAP, Airborne Topographic Lidar, Digital Elevation Model, DEM, Digital Surface Model, DSM, Elevation, Light Detection and Ranging, Lidar, terrain, and topography.

Abstract

This user guide is intended to go beyond metadata and give data users a sense of the purpose for which the data was collected, the technical processes, software, and applicable standards involved, suggested applications for the data (and general approaches to how to use it), and other details that will help data users. As such, this user guide details the specifications for the Ontario Digital Surface Model (Lidar-Derived) Geospatial Ontario (GEO) Dataset.

The Ontario Digital Surface Model (Lidar-Derived) is a compilation of raster digital elevation data from multiple acquisition projects within the Province of Ontario.

The Digital Surface Model is a gridded raster data product representing both surface features (e.g. trees and buildings) and exposed terrain (e.g. bare fields and paved surfaces), that has been generated from a classified lidar point cloud. The datasets incorporated into the Ontario Digital Surface Model (Lidar-Derived) have not been hydro-flattened using water body breaklines in the interest of preserving overhanging features such as vegetation and power lines. The product was generated by Ontario Ministry of Natural Resources (MNR) staff from the Ontario Classified Point Cloud (Lidar-Derived).

This data is intended for GIS and remote sensing applications that require a high resolution, high accuracy 3D elevation model which includes all surface features. These elevation models are invaluable for agricultural soil mapping, infrastructure assessment and development, forest modelling and management, land hazard/erosion mapping and flood control amongst other applications.

Document History

| Date | Description |
| --- | --- |
| 2020-08-12 | First release |
| 2022-01-31 | Lake Nipissing, Ottawa River, and York Region – Lake Simcoe acquisitions added |
| 2022-07-28 | Ottawa-Gatineau acquisitions added |
| 2022-08-19 | Hamilton-Niagara acquisition added |
| 2022-01-19 | OMAFRA 2022 acquisitions added |
| 2023-07-25 | Eastern Ontario and Belleville acquisitions added. |
| 2023-09-22 | DEDSFM Huron-Georgian Bay 2022 project area added. |
| 2023-12-01 | DEDSFM Huron-Georgian Bay 2023 project area added. |
| 2023-12-20 | DEDSFM Kawartha Lakes 2023 project area added. |
| 2024-01-17 | Huron Shores 2021 project added. |
| 2024-05-06 | DEDSFM Sudbury & Sault Ste Marie 2023 and Muskoka projects added. |
| 2024-09-03 | Added the final part of the Huron-Georgian Bay project covering the northern Bruce Peninsula and missing flight lines from the Sudbury project. |
| 2025-01-17 | Added the Thunder Bay 2023-24 project, Timmins 2024 and the 2024 part of the Sault Ste Marie project. |
| 2025-02-25 | Added the GTA 2023 project |
| 2025-09-09 | Added the DEDSFM Fall 2024 projects: Atikokan, Cataraqui, Chapleau, Dryden, Ignace, Northeastern Ontario and Sioux Lookout. |
| 2025-12-01 | Added the Manitoulin 2024 project. |
| 2026-03-24 | Added the DEDSFM Spring 2025 projects: Central East, Kenora, Northeastern Ontario, Rainy River, and Upper Thames-Grand River. |
| 2026-06-08 | Added the Haliburton 2019 and DEDSFM Fall 2025 projects: Lower Thames and Wasauksing-Parry Sound |

Table of Contents

Disclaimer	2

Additional Information	2

Executive Summary	3

Key Words	3

Abstract	3

Document History	4

Table of Contents	5

List of Figures	7

List of Tables	7

List of Acronyms	8

List of Definitions	10

Mass Points	10

Digital Elevation Model (DEM)	10

Digital Terrain Model (DTM)	10

Digital Surface Model (DSM)	11

1. Product Description	12

1.1 Ontario Digital Surface Model (Lidar-Derived)	12

1.2 Lidar Acquisitions	14

1.2.1 Acquisition Projects	14

1.2.2 Acquisition Details and Parameters	18

1.3 Geographic Extent*	23

1.4 Reference System	25

1.4.1 Horizontal Reference System	25

1.4.2 Vertical Reference System	26

1.5 Accuracy Assessment	27

1.5.1 Vertical Accuracy	27

1.5.2 Calculated Horizontal Accuracy	29

1.6 Spatial Resolution	29

2. Product Details	30

2.1 Data Delivery Format	30

2.2 Use Restrictions	31

3. Links to Additional Information	32

4. References	33

Appendix A: Getting Started – Display and use of DSM Data	37

Opening and viewing the DSM	37

Assembly, viewing and analysis of multiple DSM tiles: new dataset vs virtual	39

Appendix B: Common DEM Analysis Functions	40

Slope	40

Aspect	40

Curvature	41

Appendix C: Important Considerations and Product Limitations	42

Vertical datum difference	42

Scan line artefacts	43

Tile boundaries extend beyond the collection area boundary	45

Hydro-flattening	46

List of Figures

Figure 1: DSM viewed as a hillshade in the City of Peterborough, ON	13

Figure 2: Extent of Lidar Derived data for Southern Ontario	23

Figure 3: Extent of Lidar Derived data for Northern Ontario	24

Figure 4: Tile names reflect tile size, UTM zone, easting, northing, year, and project	30

Figure 5: Unzipped DSM package contents	37

Figure 6: DSM displayed by elevation in greyscale	37

Figure 7: DSM tile displayed as a hillshade and colour-shaded relief	38

Figure 8: Examples of Slope and Aspect	40

Figure 9: Vertical profile graphs along a 50 metre transect showing the difference between data in the CGVD 1928 and CGVD 2013 vertical datums	42

Figure 10: Scan line artefacts with profile location and profile elevation values	43

Figure 11: Chart of elevation values along the profile	44

Figure 12: Detail comparing raw data collection, project, and package extents	45

Figure 13: Comparison of DTM (left) to DSM (right), with hydrographic breaklines displayed as blue lines. In the image on the right overhanging vegetation and overhead wires are visible, as well as elevation noise from the water surface.	46

List of Tables

Table 1: Acquisition details for the Ontario DSM (Lidar-Derived) source projects	18

Table 2: Lidar system specifications and flight parameters by sensor system	20

Table 3: Lidar project areas by UTM zone	25

Table 4: Vertical datums by projects	26

Table 5: Airborne lidar non-vegetated vertical accuracy	27

Table 6: Airborne lidar horizontal accuracy	29

List of Acronyms

AGL: Above Ground Level

ASPRS: American Society for Photogrammetry and Remote Sensing

ATL: Airborne Topographic LIDAR

CGVD: Canadian Geodetic Vertical Datum

CLOCA: Central Lake Ontario Conservation Authority

CSRS: Canadian Spatial Reference System

DEDSFM: Digital Elevation Data to Support Flood Mapping

DEM: Digital Elevation Model

DSM: Digital Surface Model

DTM: Digital Terrain Model

FHIMP: Flood Hazard Identification and Mapping Program

GEO: Geospatial Ontario

GIS: Geographic Information System

GTA: Greater Toronto Area

LEAP: Lidar Eastern Acquisition Project

LIDAR: Light Detection and Ranging

MNR: Ontario Ministry of Natural Resources

MTO: Ontario Ministry of Transportation

NAD: North American Datum

NDMP: National Disaster Mitigation Program

NRCan: Natural Resources Canada

OMAFRA: Ontario Ministry of Agriculture, Food and Rural Affairs

SNC: South Nation Conservation

URL: Uniform Resource Locator

USGS: United States Geological Survey

UTM: Universal Transverse Mercator

List of Definitions

Mass Points

Mass points are irregularly spaced points, each with x/y location coordinates and z-values, typically (but not always) used to form a TIN. When generated manually, mass points are ideally chosen to depict the most significant variations in the slope or aspect of TIN triangles. However, when generated automatically, e.g., by lidar or IfSAR scanner, mass point spacing and patterns depend upon the characteristics of the technologies used to acquire the data.

Digital Elevation Model (DEM)

DEM is a generic term for digital topographic and/or bathymetric data that is comprised of x/y coordinates and z-values to represent an elevation surface.

The term ‘DTM’ and ‘DSM’ should be used over the term ‘DEM’ to more specifically reference ‘bare-earth’ or ‘surface elevation’ model products when possible.

The term ‘DEM’ is to be used as a broader term when referencing a generic elevation data product. The Provincial DEM is an example of a generic elevation product given that it has been constructed using a combination of both ‘DTM’ and ‘DSM’ elevation datasets to achieve Provincial coverage.

Digital Terrain Model (DTM)

The bare earth surface (lowest surface, last reflective surface, or lidar last-return) represents the surface of the "bare-earth" terrain, after removal of vegetation and constructed features.

Photogrammetry has traditionally generated DTMs when elevations are generated by manual compilation techniques. Unless specified to the contrary, the bare-earth surface includes the top surface of water bodies, rather than the submerged surface of underwater terrain.

Like a DSM, a DTM can be structured either as a vector dataset (comprised of mass points and optionally 3D breaklines) to model bare-earth elevations or a raster dataset that is interpolated from the vector elevation data to model bare-earth terrain elevations.

Using modern elevation point cloud classification algorithms and file formats, such as LAS, a DTM can represent a mass point dataset that has been classified for ‘bare-earth’ terrain elevations.

Digital Surface Model (DSM)

A DSM is the highest reflective surface of ground features captured by the sensor. This surface may also be referred to as the first reflective surface or lidar first-return. The DSM may include treetops, rooftops, and tops of towers, telephone poles, and other natural or manmade features; or it may include the ground surface if there is no vegetative ground cover. Photogrammetry, IFSAR, lidar and sonar can all provide this type of surface, yet characteristics such as accuracy and degree of detail (ability to resolve desired surface features) may vary significantly across technologies and even within the same technology. With sonar, the DSM may include sunken vessels and other artifacts, whereas the bathymetric surface reflects the natural underwater terrain. Similarly, with photogrammetry, lidar, and IFSAR the reflective surface may include any artifact present when the sensor mapped the area, including passing cars and trucks and similar features not normally considered to be part of a digital terrain model.

Like a DTM, a DSM can be structured either as a vector dataset (comprised of mass points and optionally 3D breaklines) to model surface elevations or a raster dataset that is interpolated from the vector elevation data to model surface elevations.

Using modern elevation point cloud classification algorithms and file formats, such as LAS, a DSM can represent a mass point dataset that has been classified for ‘surface’ elevation features.

1. Product Description

1.1 Ontario Digital Surface Model (Lidar-Derived)

The purpose of each Airborne Topographic Lidar acquisition was to acquire classified lidar digital elevation data and derivative products for selected geographic areas in Ontario. This data is intended for GIS and remote sensing applications that require a high resolution, high accuracy elevation model. These elevation models, known as Digital Elevation Models (DEMs), are invaluable for agricultural soil mapping, infrastructure assessment and development, forest modelling and management, land hazard/erosion mapping and flood control amongst other applications.

The Ontario Digital Surface Model (Lidar-Derived) is a gridded raster data product that provides a generalized representation of both surface features (e.g. trees and buildings) and exposed terrain (e bare fields and paved surfaces). It has been generated from a classified point cloud by interpolating from the maximum (or first-return) lidar return in each cell. The datasets incorporated into the Ontario Digital Surface Model (Lidar-Derived) have not been hydro-flattened using water body breaklines in the interest of preserving overhanging features such as vegetation and power lines. See Appendix C for more details. Resolution depends on the acquisition project. Figure 1 shows a sample hillshade of the DSM.

Figure 1: DSM viewed as a hillshade in the City of Peterborough, ON

The following products which contributed to the production of the DSM were delivered by the lidar project contractors:

Classified Point Cloud: Available as the Ontario Classified Point Cloud (Lidar-Derived) (https://geohub.lio.gov.on.ca/datasets/adf19376eecd4440a4579a73 abe490f5).

Digital Terrain Model: Available as the Ontario Digital Terrain Model (Lidar-Derived) (https://geohub.lio.gov.on.ca/search?q=ontario%20digital%20terrain%20 model%20lidar-derived).

1.2 Lidar Acquisitions

1.2.1 Acquisition Projects

The Ontario DSM (Lidar-Derived) is a compilation of lidar data from multiple acquisition projects, as such, specifications, parameters, accuracy, and sensors may vary by project. See Table 1 and Table 2 for a summary of acquisition details.

The Digital Elevation Data to Support Flood Mapping (DEDSFM) 2022-26 project was initiated by the Ontario Ministry of Natural Resources’ (MNR) Ontario Elevation Mapping Program in response to Ontario’s 2020 Flooding Strategy. The project is acquiring lidar data to support flood mapping and other uses across targeted areas of Ontario from 2022 to 2025.

The Greater Toronto Area (GTA) 2023 lidar project was acquired by Airborne Imaging Inc. for a partnership of Greater Toronto Area (GTA) conservation authorities, Halton Region Conservation Authority, Lake Simcoe Region Conservation Authority, and Toronto and Region Conservation Authority. The project is supported by Natural Resources Canada (NRCan) through FHIMP.

The Muskoka 2018, 2021 and 2023 lidar projects were acquired by Hatch Ltd. for the District of Muskoka to support the Muskoka Floodline Mapping Project. See Muskoka’s Lidar Discovery story map for more details. These projects were delivered to the Muskoka in the CGVD28 vertical datum. They have been transformed to CGVD2013 by MNR. Both versions have been made available for download.

The Belleville 2022 lidar project was acquired by Airborne Imaging Inc. for the Government of Canada as part of the FHIMP.

The Eastern Ontario 2021-22 lidar project was acquired by Airborne Imaging Inc. for a partnership of Eastern Ontario Conservation Authorities, Municipalities, Counties and Conservation Ontario with NRCan under FHIMP. The full list of partners includes:

Cataraqui Region Conservation Authority

Mississippi Valley Conservation

Quinte Conservation

South Nation Conservation

Raisin Region Conservation Authority

Rideau Valley Conservation Authority

City of Cornwall

City of Kingston

Municipality of South Dundas

Township of Loyalist

Township of North Dundas

Township of North Glengarry

Township of North Stormont

Township of South Glengarry

Township of South Stormont

County of Frontenac

County of Lanark

United Counties of Leeds and Grenville

United Counties of Prescott and Russell

United Counties of Stormont, Dundas and Glengarry

Natural Resources Canada

Conservation Ontario

The OMAFRA 2022 lidar project was acquired by Airborne Imaging Inc. for the Ministry of Agriculture, Food and Rural Affairs (OMAFRA) to cover two project areas, the southern portion of the Lake Huron watershed and the northern part of the Lake Simcoe watershed. Funding for the project was provided through the Canadian Agricultural Partnership, a five-year, $3 billion investment by federal-provincial and territorial governments to encourage innovation, competitiveness and sustainability in Canada’s agriculture industry. Review and quality control support of the data deliveries were provided by MNR as in-kind contributions.

The Huron Shores 2021 lidar was acquired by Tulloch to support a flood plain mapping project by Hatch Ltd for the Municipality of Huron Shores. Matching funding was provided by Public Safety Canada through the National Disaster Mitigation Program (NDMP).

The Hamilton-Niagara Region lidar project was acquired by Leading Edge Geomatics for NRCan with emergency management funding from Public Safety Canada.

The Ottawa-Gatineau lidar project was acquired by Perron Hudon Belanger Inc. as a partnership between the City of Gatineau, the City of Ottawa, and the National Capital Commission. Review and quality control support of the data deliveries were provided by MNR as in-kind contributions. The horizontal projection of the data was originally in NAD83 CSRS MTM Zone 09 and was reprojected to NAD83 CSRS UTM Zone 18 by MNR.

The North Bay-Mattawa and Lake Nipissing lidar project was acquired by Eagle Mapping Ltd for Natural Resources Canada (NRCan) with emergency management funding from Public Safety Canada.

The Ottawa River lidar project was acquired by Groupe Info Consult Inc. for NRCan with emergency management funding from Public Safety Canada.

The Haliburton 2019 project was acquired by Airborne Imaging Inc. for the County of Haliburton to support flood plain mapping studies. Matching funding was provided by Public Safety Canada through the NDMP.

The York Region – Lake Simcoe Watershed project was acquired from Airborne Imaging Inc. by OMAFRA with funding through the Lake Simcoe Protection Plan delivery program. Review and quality control support of the data deliveries were provided by MNR as in-kind contributions.

The South Nation Conservation (SNC) and Central Lake Ontario Conservation Authority (CLOCA) airborne topographic lidar acquisitions were flown by Airborne Imaging Inc. under a project initiated by the respective Conservation Authorities with matching funding from the NDMP. Review and quality control support of the data deliveries were provided by Ontario MNR as in-kind contributions.

The GTA 2014-18 lidar acquisitions were flown as six separate projects by Airborne Imaging Inc. For extents of each of the acquisition projects see the figures in Section 1.3. The data was purchased under license by NRCan and made available as open data through the High Resolution Digital Elevation Model (HRDEM). The HRDEM is at 1 metre resolution, whereas the GTA lidar data has been interpolated to 0.5 metre resolution for the Ontario DSM (Lidar-Derived).

The OMAFRA airborne topographic lidar acquisition was collected through a collaborative partnership with OMAFRA, MNR and a private contractor; it covers selected areas in Southern Ontario and portions of Northern Ontario. A contract was awarded to Airborne Imaging Incorporated for the collection of lidar for the three project areas: Cochrane, Peterborough, and the Lake Erie watershed. This project was funded in part through Growing Forward 2 (GF2), a federal-provincial-territorial initiative.

The LEAP Airborne Topographic Lidar data was funded through a collaborative partnership consisting of the MNR, Ministry of Transportation (MTO), City of Kingston, Cataraqui Region Conservation Authority, Prince Edward County, and the Municipality of Quinte West. A contract was awarded to Géomatique EMCO Inc. for the acquisition of lidar across the project area.

An extensive technical review of the LEAP deliverables conducted by MNR’s Southern Region identified several quality issues in the data based on the project’s Technical Guidelines and specifications. It was determined that the lidar deliverables did not meet the specifications within the Agreement. MNR staff subsequently made extensive quality control improvements using the deliverables to produce a fully modified, edited, and reformatted set of derivative products.

Refer to the project metadata documents for more details. To see the exact acquisition date for a particular location and the sensor system used refer to the Lift Metadata shapefile attached to the LIO metadata record for the Ontario Classified Point Cloud (Lidar-Derived).

1.2.2 Acquisition Details and Parameters

Table 1: Acquisition details for the Ontario DSM (Lidar-Derived) source projects

| Project Area | Acquisition Period | Project Area (km2) | Vertical Accuracy Class | Pulse Density (Pulses/m2) | Flight Altitude AGL (m) | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
| LEAP | May to Dec 2009 | 4,245 | 15 cm | 1.3 | 1900 |  |
| GTA (2014) | Apr to May 2014 | 1,251 | 10 cm | 10.3 | 1300 |  |
| GTA (2015) | Apr 2015 | 1,475 | 10 cm | 8.96 | 1300 |  |
| GTA (Brampton) | Nov 2015 | 96 | 10 cm | 15.1 | 1100 |  |
| GTA (Peel) | May 2016 | 556 | 10 cm | 15.1 | 950 |  |
| OMAFRA Lidar Cochrane | Oct to Nov 2016, May to Jun and Sep 2017 | 6,124 | 5 cm | 8 | 1000 | See Airborne Imaging (2018a) for more details |
| OMAFRA Lidar Peterborough | Nov to Dec 2016, Apr to May and Oct 2017 | 6,836 | 5 cm | 8 | 1000 | See Airborne Imaging (2018b) for more details |
| GTA (Milton) | Nov to Dec 2017 | 217 | 10 cm | 15.8 | 950 |  |
| OMAFRA Lidar Lake Erie | Mar to May 2017, Oct to Dec 2017 and Mar to May 2018 | 23,029 | 5 cm | 8 | 1000 (Leica) 700 (Riegl) | See Airborne Imaging (2019a) for more details |
| GTA (Halton) | Mar to Jun 2018 | 1,080 | 10 cm | 12.8 | 1200 |  |
| Muskoka 2018 | May 18-24, 2018 | 1,508 | 10 / 15 cm | 2.5 / 10.4 | 1250 / 1550 |  |
| CLOCA Lidar | Dec. 9–10, 2018 | 386 | 5 cm | 8 | 1000 | See Airborne Imaging (2019b) for more details |
| SNC Lidar | Nov. 11-14, 2018 & Apr. 4-22, 2019 | 798 | 5 cm | 8 |  | See Airborne Imaging (2019c) for more details |
| York Region-Lake Simcoe | Aug 2 to Sep 20, 2019 | 990 | 10 cm | 10 | 1000 |  |
| Haliburton 2019 | Sep 21 to Oct 9, 2019 | 503 | 10 cm | 5.4 | 1400 |  |
| Ottawa River | Oct 5-25, 2019 & Apr 24 to May 22, 2020 | 5,969 | 10 cm | 6 | 800 |  |
| Ottawa-Gatineau | May 3-22 & Oct 9-26, 2019 & Apr 18 - May 2, 2020 | 4,353 | 5 cm | 8 | 1150 |  |
| North Bay & Lake Nipissing | Jun 1-19, 2020 | 5,975 | 10 cm | 11 | 1650 |  |
| Hamilton-Niagara | Apr 5 to May 25, 2021 | 2,983 | 5 cm | 26.4 | 1300 |  |
| Huron Shores | Oct 28, 2021 | 539 | 5 cm | 7.43 | 1800 |  |
| Muskoka 2021 | Nov 9-10, 2021 | 1,205 | 10 cm | 4 | 1900 / 950 |  |
| Eastern Ontario | Nov 5-27, 2021 & Apr 18-30, 2022 | 14,825 | 5 cm | 8 | 1400 / 1600 |  |
| OMAFRA 2022 Lake Huron | Apr 12-24, 2022 | 4,373 | 5 cm | 8 | 1600 | See Airborne Imaging (2022) for more details |
| OMAFRA 2022 Lake Simcoe | May 5-7, 2022 | 2,088 | 5 cm | 8 | 1600 | See Airborne Imaging (2022b) for more details |
| Belleville 2022 | Oct 1-25, 2022 | 7,517 | 5 cm | ≥ 8 | 1600 |  |
| DEDSFM Huron-Georgian Bay | Oct 24 to Nov 10, 2022, Apr 27 to Jun 10 & Nov 15-16, 2023 | 16,968 | 5 cm | 13.55 | 900 / 1650 / 2050 | See Airborne Imaging (2024a) for more details |
| GTA 2023 | Apr 9 to May 11, 2023 | 5,253 | 5 cm | 16 | 1800 |  |
| DEDSFM Kawartha Lakes | May 6-10, 2023 | 3,375 | 5 cm | 11.51 | 1650 | See Airborne Imaging (2025e) for more details |
| DEDSFM Sudbury | May 13-14, 2023 & Apr 16, 2024 | 5,067 | 5 cm | 12 | 1650 | See Airborne Imaging (2025h) for more details |
| DEDSFM Sault Ste Marie | May 15-18, 2023 & Apr 6-24, 2024 | 7,172 | 5 cm | UTM16: 15.6 UTM17: 12.5 | 1650 | See Airborne Imaging (2025g) for more details |
| Muskoka 2023 | May 25-27, 2023 | 1,913 | 10 cm | 8 | 1600 / 2000 |  |
| DEDSFM Thunder Bay | Sep 26 to Oct 11, 2023 & Apr 25 to May 6, 2024 | 3,897 | 5 cm | 15.34 | 1650 / 2000 | See Airborne Imaging (2025i) for more details |
| DEDSFM Manitoulin | Apr 6-16, 2024 | 3,624 | 5 cm | 11.98 | 1650 / 2000 | See Airborne Imaging (2025f) for more details |
| DEDSFM Timmins | May 4-9, 2024 | 3,194 | 5 cm | 12.29 | 1650 | See Airborne Imaging (2025j) for more details |
| DEDSFM Dryden | Oct 27-31, 2024 | 1,899 | 5 cm | 11.72 | 2000 | See Airborne Imaging (2025d) for more details |
| DEDSFM Sioux Lookout | Oct 31 to Nov 8, 2024 | 1,049 | 5 cm | 11.53 | 2000 | See Airborne Imaging (2025d) for more details |
| DEDSFM Atikokan | Nov 1, 2024 | 600 | 5 cm | 11.7 | 2000 | See Airborne Imaging (2025a) for more details |
| DEDSFM Ignace | Nov 1, 2024 | 110 | 5 cm | 11.4 | 2000 | See Airborne Imaging (2025d) for more details |
| DEDSFM Cataraqui | Nov 3, 2024 | 570 | 5 cm | 12.15 | 1650 | See Airborne Imaging (2025b) for more details |
| DEDSFM Chapleau | Nov 9, 2024 | 1,120 | 5 cm | 11.67 | 2000 | See Airborne Imaging (2025c) for more details |
| DEDSFM Northeastern Ontario | Nov 9, 2024 & May 8 to Jun 2, 2025 | 9,058 | 5 cm | 13.12 | 1650 / 2000 | See Airborne Imaging (2026c) for more details |
| DEDSFM Central East | Apr 7-23, 2025 | 5,577 | 5 cm | 13.1 | 2000 | See Airborne Imaging (2026a) for more |
| DEDSFM Upper Thames-Grand River | Apr 24 to May 9 and Nov 14, 2025 | 7,779 | 5 cm | 14.6 | 1650 / 2000 | See Airborne Imaging (2026e) for more |
| DEDSFM Kenora | May 5, 2025 | 476 | 5 cm | 12.7 | 1650 | See Airborne Imaging (2026b) for more |
| DEDSFM Rainy River | May 5 to Jun 3, 2025 | 3,139 | 5 cm | 15.01 | 1650 | See Airborne Imaging (2026d) for more |
| DEDSFM Wasauksing-Parry Sound | Oct 28, 2025 | 437 | 5 cm | 8.43 | 2000 |  |
| DEDSFM Lower Thames | Nov 13-19, 2025 | 3,571 | 5 cm | 12.7 | 1650 | This is the first 2,733 tile delivery of a project to be completed in 2026 |

Table 2: Lidar system specifications and flight parameters by sensor system

| Project Area | Sensor System | Speed (kn) | Flightline Spacing (m) | Single Pass Swath Width (m) | Scan Angle or Field of View | Scan Freq (Hz) | Scan Pulse Rate (kHz) | Overlap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Belleville 2022 | Riegl-VQ1560ii | 160 |  | 1445 | 50° | 252 | 1100 | 50% |
| CLOCA & SNC Lidar | Riegl LMS-Q1560 | 150 | 930 |  | 58° effective | 313 | 800 | ≤ 60% |
| DEDSFM Atikokan, Chapleau, Dryden, Ignace, Sioux Lookout | Riegl VQ1560ii-S | 150 |  | ~2100 | 60° | 334 | 2600 | 40% |
| DEDSFM Cataraqui | Riegl VQ1560ii-S | 150 |  | 1780 | 60° | 330 | 2100 | 40% |
| DEDSFM Central East and Wasauksing-Parry Sound | Riegl VQ1560ii-S | 150 |  | ~2100 | 60° | 334 | 1733.3 | 40% |
| DEDSFM Huron-Georgian Bay | Riegl-VQ1560ii / VQ1560ii-S | 160/ 200 |  | 1445 | 60° | 300 / 340 / 342 | 2000 / 2100 / 2600 | 40% |
| DEDSFM Kawartha Lakes | Riegl VQ1560ii-S | 160 |  | ~1730 | 60° | 320 | 2100 | 40% |
| DEDSFM Kenora, and Rainy River | Riegl VQ1560ii | 150 |  | 1800 | 60° | 330 | 1400 | 40% |
| DEDSFM Lower Thames | Riegl VQ1560ii | 150 |  | 1800 | 60° | 140 / 330 | 1400 | 40% |
| DEDSFM Manitoulin | Riegl VQ1560ii / VQ1560ii-S | 150 |  | ~1700 / ~2100 | 60° | 330 / 334 | 2100 / 2600 | 40% |
| DEDSFM Northeastern Ontario | Riegl VQ1560ii / VQ1560ii-S | 150 |  | 1700 | 60° | 330 / 334 | 1400 / 1733.3 | 40% |
| DEDSFM Sault Ste Marie | Riegl VQ1560ii-S | 150 / 200 |  | ~1700 | 60° | 300 / 334 | 2000 / 2600 | 40% |
| DEDSFM Sudbury | Riegl VQ1560ii / VQ1560ii-S | 200 |  | ~1700 | 60° | 300 | 2000 | 40% |
| DEDSFM Thunder Bay | Riegl VQ1560ii / VQ1560ii-S | 160 / 150 |  | ~1700 | 60° | 341 / 334 | 2100 / 2600 | 40% |
| DEDSFM Timmins | Riegl VQ1560ii | 150 |  | ~1700 | 60° | 332 | 2200 | 40% |
| DEDSFM Upper Thames-Grand River | Riegl VQ1560ii / VQ1560ii-S | 150 |  | 1700 | 60° | 330 / 334 | 1400 / 1733.3 | 40% |
| Eastern Ontario | Riegl-VQ1560ii | 160 |  | 1445 | 60° | 202 | 1250 | 50% |
| GTA 2014 & 2015 | Leica ALS70 | 160 |  | 1283.4 | 40° | 52 | 400 | 50% |
| GTA (Brampton) | Leica ALS70 | 140 |  | 1283.4 | 40° | 53 | 480 | 50% |
| GTA (Halton, Milton & Peel) | Leica ALS70 | 160 |  | 1283.4 | 40° | 52.1 | 440 | 50% |
| GTA 2023 | Riegl LMS-VQ1560ii / Riegl LMS-VQ1560ii-S | 160 |  | 1445 | 60° | 334 | 2200 | 50% |
| Haliburton 2019 | Leica ALS70 | 160 |  | variable | 50° | 53 | 375 | 50% |
| Hamilton-Niagara | Riegl VQ1560i | 130 |  | 1212 | 50° | 105 | 700 | 50% |
| Huron Shores | Riegl VQ1560ii | 160 |  |  | 60° |  | 700 |  |
| LEAP | Optech ALTM 3100, Optech ALTM Gemini 167 | 120 | 864 | 1235 | 36° | 35 | 100 | 30% |
| Muskoka 2018 | Leica ALS70 | 140 / 160 |  |  | 40° | 53 | 300 / 400 | 40% / 100% |
| Muskoka 2021 | Riegl VQ1560ii | 160 |  |  | 60° | 128 / 142 | 650 / 400 | 40% |
| Muskoka 2023 | Riegl VQ1560ii-S | 160 |  |  | 60° | 456 | 3600 | 40% |
| North Bay & Lake Nipissing | Riegl LMS-Q1560 | 125 |  | 1539 | 50° | 187 | 800 | 60% |
| OMAFRA Lidar 2016-18 | Leica ALS70-HP | 140 | 550 | 690 | 38° effective | 53 | 500 | ≤ 40% |
| OMAFRA Lidar Lake Erie | Riegl LMS-Q1560 | 150 | 930 |  | 58° effective | 313 | 800 | ≤ 60% |
| OMAFRA 2022 | Riegl VQ1560ii | 160 |  | 1675 | 60° | 175 | 2100 | 40% |
| Ottawa-Gatineau | Optech ALTM Galaxy | 155 |  | 837 | 40° | 85 | 450 | 56% |
| Ottawa River | Optech ATLM Galaxy | 137 |  | 582 | 40° | 91 | 450 | 20% |
| York Region-Lake Simcoe | Riegl LMS-VQ1560 / Leica ALS 70 | 160 / 160 |  |  | 60° / 50° | 135 / 50 | 800 / 500 |  |

1.3 Geographic Extent*

Detailed boundaries of individual lidar projects can be viewed in the Ontario Lidar Coverage map. (https://mnrf.maps.arcgis.com/apps/mapviewer/index.html?webmap=275b5248fd3c4b5caccd224e9e020afc)

Figure 2: Extent of Lidar Derived data for Southern Ontario

Figure 3: Extent of Lidar Derived data for Northern Ontario

*Note: Raw point cloud data may extend beyond boundaries of the project area. See the Ontario Classified Point Cloud (Lidar-Derived) user guide for more details.

1.4 Reference System

1.4.1 Horizontal Reference System

The horizontal coordinate system of the Ontario DSM (Lidar-Derived) is Universal Transverse Mercator (UTM) with zone varying by project (Table 3). The horizontal datum of the products is the North American Datum of 1983 Canadian Spatial Reference System epoch 2010 (NAD83 (CSRS)).

The horizontal unit of measure (coordinate system axis units) for all raster grid cells is metres (m).

Table 3: Lidar project areas by UTM zone

| UTM Zone | Project Areas |
| --- | --- |
| 15 | - DEDSFM Atikokan, Dryden, Ignace, Kenora, Rainy River, and Sioux Lookout |
| 16 | - DEDSFM Sault Ste Marie and Thunder Bay |
| 17 | - CLOCA - DEDSFM Cataraqui, Central East, Chapleau, Huron-Georgian Bay, Kawartha Lakes, Lower Thames, Manitoulin, Northeastern Ontario, Sault Ste Marie, Sudbury, Timmins, Upper Thames-Grand River, and Wasauksing-Parry Sound - GTA 2014-18 - GTA 2023 - Haliburton 2019 - Hamilton-Niagara - Huron Shores 2021 - Muskoka 2018, 2021 & 2023 - North Bay-Mattawa & Lake Nipissing - OMAFRA 2016-18 (Cochrane, Lake Erie, Peterborough) - OMAFRA 2022 (Lake Huron & Lake Simcoe) - York-Lake Simcoe 2019 |
| 18 | - Belleville 2022 - Eastern Ontario 2021-22 - LEAP - Ottawa-Gatineau - Ottawa River - SNC |

1.4.2 Vertical Reference System

The vertical coordinate system of the Ontario DSM (Lidar-Derived) varies by acquisition project (Table 4). For more information on Canadian vertical datums see the Height Reference Systems of Natural Resources Canada (https://natural-resources.canada.ca/science-data/science-research/geomatics/geodetic-reference-systems/canadian-spatial-reference-system-csrs#hrs).

The vertical unit of measure (coordinate system axis units) for all raster grid cells in the products is metres (m). One single vertical elevation value represents each raster grid cell in the DSM.

Table 4: Vertical datums by projects

| Vertical Datum | Project Areas |
| --- | --- |
| CGVD28 | - LEAP - Muskoka 2018, 2021 & 2023 |
| CGVD2013 | - Belleville 2022 - CLOCA - DEDSFM 2022-26 - Eastern Ontario 2021-22 - GTA 2014-18 - GTA 2023 - Haliburton 2019 - Hamilton-Niagara - Huron Shores 2021 - Muskoka 2018, 2021 & 2023 - North Bay-Mattawa & Lake Nipissing - OMAFRA 2016-18 (Cochrane, Lake Erie, Peterborough) - OMAFRA 2022 (Lake Huron & Lake Simcoe) - Ottawa-Gatineau - Ottawa River - SNC - York-Lake Simcoe 2019 |

1.5 Accuracy Assessment

The Ontario Public Service Elevation Coordination and Consultation Committee (EC3) has published Elevation Accuracy Guidelines which explains and provided guidelines for assessing the accuracy of digital elevation data, which has been drafted to be consistent with the 2014 American Society for Photogrammetry and Remote Sensing (ASPRS) Positional Accuracy Standards for Digital Geospatial Data.

1.5.1 Vertical Accuracy

Vendor supplied vertical accuracy assessment reports are available for some project areas and are attached to the GeoHub record for the Ontario DSM (Lidar-Derived). These reports contain additional details about the method by which the vendor calibrated the data and quantified the accuracy of the data.

For the LEAP 2009 project additional details about the method by which the contractor has calibrated the data and quantified the accuracy of the data and included in to Appendices D and E of this document.

Non-vegetated Vertical Accuracy

Table 5: Airborne lidar non-vegetated vertical accuracy

| Lidar Project | Fast Static Vertical Accuracy (95%) | RTK Vertical Accuracy (95%) |
| --- | --- | --- |
| Belleville 2022 | N/A | 4.7 cm / 105 points |
| CLOCA | N/A | 4.1 cm / 26 points |
| DEDSFM Atikokan | N/A | 9 cm / 22 points |
| DEDSFM Cataraqui | N/A | 5.3 cm / 17 points |
| DEDSFM Central East | N/A | 5.9 cm / 79 points |
| DEDSFM Chapleau | N/A | 7.1 cm / 34 points |
| DEDSFM Dryden | N/A | 5.8 cm / 48 points |
| DEDSFM Huron-Georgian Bay | N/A | 8.3 cm / 151 points |
| DEDSFM Ignace | N/A | 5.3 cm / 21 points |
| DEDSFM Kawartha Lakes | N/A | 7.2 cm / 173 points |
| DEDSFM Kenora | N/A | 5.9 cm / 24 points |
| DEDSFM Lower Thames | N/A | To be completed in 2026 |
| DEDSFM Manitoulin | N/A | 8.2 cm / 67 points |
| DEDSFM Northeastern Ontario | N/A | 8.4 cm / 107 points |
| DEDSFM Rainy River | N/A | 7.4 cm / 65 points |
| DEDSFM Sault Ste Marie | N/A | 5.7 cm / 145 points |
| DEDSFM Sioux Lookout | N/A | 6.6 cm / 31 points |
| DEDSFM Sudbury | N/A | 9.0 cm / 83 points |
| DEDSFM Thunder Bay | N/A | 8.8 cm / 82 points |
| DEDSFM Timmins | N/A | 7.3 cm / 57 points |
| DEDSFM Upper Thames-Grand River | N/A | 7.3 cm / 88 points |
| DEDSFM Wasauksing-Parry Sound | N/A | 3.7 cm / 25 points |
| Eastern Ontario | 7.5 cm / 69 points | 8.6 cm / 26,008 points |
| GTA 2014-18 | N/A | 7.25 cm / 33,196 points |
| GTA 2023 | N/A | 5.68 cm / 82 points |
| Haliburton 2019 | N/A | 7.4 cm / 32 points |
| Hamilton-Niagara | N/A | 3 cm / 268 points |
| Huron Shores 2021 | N/A | 6.6 cm / 182 points |
| Lake Nipissing | N/A | 8.4 cm / 1,147 points |
| LEAP 2009 | N/A | 20.76 cm / 1,552 points |
| Muskoka 2018 | 8.0 cm / 3757 points | 5.5 cm / 63 points |
| Muskoka 2021 | N/A | 2.9 cm / 406 points |
| Muskoka 2023 | N/A | 9.8 cm / 1219 points |
| OMAFRA Cochrane | 6.6 cm / 129 points | 6.9 cm / 152 points |
| OMAFRA Lake Erie | 12.0 cm / 371 points | 6.9 cm / 813 points |
| OMAFRA Peterborough | 8.6 cm / 127 points | 9.4 cm / 223 points |
| OMAFRA 2022 Lake Huron | 5.3 cm / 8247 points | 8.0 cm / 97 points |
| OMAFRA 2022 Lake Simcoe | 6.1 cm / 2009 points | 5.1 cm / 122 points |
| Ottawa-Gatineau | N/A | 7.4 cm / 366 points |
| Ottawa River | N/A | 5.38 cm / 399 points |
| South Nation Conservation | N/A | 4.5 cm / 70 points |
| York-Lake Simcoe 2019 | N/A | 7.89 cm / 2,668 points |

1.5.2 Calculated Horizontal Accuracy

The calculated horizontal accuracy was determined from the parameters of the lidar system used in acquisition and the altitude the project was flown at.

Table 6: Airborne lidar horizontal accuracy

| Lidar Project | Horizontal Accuracy (95%) |
| --- | --- |
| Belleville 2022 | 15 cm |
| CLOCA | 29.7 cm |
| DEDSFM 2022-26 | 30 cm |
| Eastern Ontario 2021-22 | 30 cm |
| GTA 2014-18 | 28 – 37 cm |
| GTA 2023 | 15 cm |
| Haliburton 2019 | 50 cm |
| Muskoka 2018 | 35 cm |
| OMAFRA Cochrane | 30 cm |
| OMAFRA Lake Erie | 30 cm |
| OMAFRA Peterborough | 30 cm |
| OMAFRA 2022 Lake Huron | 30 cm |
| OMAFRA 2022 Lake Simcoe | 30 cm |
| Ottawa River | 15 cm |
| South Nation Conservation | 30 cm |
| York-Lake Simcoe 2019 | 30 cm |
| Hamilton-Niagara / Huron Shores / Lake Nipissing / Muskoka 2021 & 2023 / Ottawa-Gatineau | Not assessed |

1.6 Spatial Resolution

The grid spacing of the DSM is based on the UTM projection with a standard raster resolution of 0.5m. The LEAP and Muskoka (2018, 2021 & 2023) datasets are the only exception with a cell resolution of 1m.

2. Product Details

2.1 Data Delivery Format

The Ontario DSM (Lidar-Derived) is stored and distributed through the Ontario GeoHub (https://geohub.lio.gov.on.ca/). Data packages can be downloaded by clicking on features in the map interface of the record for the Ontario Digital Surface Model (Lidar-Derived) (https://geohub.lio.gov.on.ca/maps/mnrf::ontario-digital-surface-model-lidar-derived/about) or from the list of data packages. For additional information on working with the DSM see Appendix A. For details on the file sizes and contents of each data package see Appendix D.

The 1-km tiles are stored in IMG (ERDAS-Imagine) or TIFF format raster files and grouped into packages of tiles compressed using the ZIP archive file format. The standard file naming format of the tiles is explained in Figure 4.

Figure 4: Tile names reflect tile size, UTM zone, easting, northing, year, and project

Since the release of the Ontario Web Image Services (OWIS) in the fall of 2022, the lidar DSM is also available through an image service which exposes the data for visualization and geoprocessing.

The service is best accessed through the ArcGIS REST API, either directly or by setting up an ArcGIS server connection using the REST endpoint URL, which can be found in the GeoHub record linked above. The service draws using the WGS84 Web Mercator projection.

For more information on what functionality is available and how to work with the service, please refer to the Ontario Web Image Services User Guide. If you have questions about how to use the service, email Geospatial Ontario (GEO) at geospatial@ontario.ca.

2.2 Use Restrictions

The Ontario Digital Surface Model (Lidar-Derived) is Open Data. You are free to copy, modify, publish, translate, adapt, distribute or otherwise use the Information in any medium, mode or format for any lawful purpose. If you do any of the above, you must use the following attribution statement “Contains information licensed under the Open Government Licence – Ontario”. See Open Government Licence (https://www.ontario.ca/page/open-government-licence-ontario).

3. Links to Additional Information

Heidemann, H. K. 2018 “LIDAR base specification”. Ver. 1.3. February 2018 U.S. Geological Survey Techniques and Methods, book 11, chap. B4. https://pubs.er.usgs.gov/publication/tm11B4

Canada. Natural Resources Canada. 2025. “Spatial reference system updates”. https://natural-resources.canada.ca/science-data/science-research/geomatics/geodetic-reference-systems/spatial-reference-system-updates

Canada. Natural Resources Canada. 2025. “About the Canadian Spatial Reference System”. https://natural-resources.canada.ca/science-data/science-research/geomatics/geodetic-reference-systems/canadian-spatial-reference-system-csrs

Renslow, M. S. (ed.) 2012. “Manual of Airborne Topographic LIDAR”. American Society for Photogrammetry and Remote Sensing. ISBN 1-57083-097-5, ASPRS Stock # 4587

Whitebox Geospatial AT Project (https://www.whiteboxgeo.com)

4. References

Airborne Imaging 2018a. “Final Report for project: Cochrane LiDAR”. Report prepared for Ontario Ministry of Natural Resources and Forestry, Provincial Mapping Unit. https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar%20-%20Cochrane%20-%20Additional%20Metadata.pdf

Airborne Imaging 2018b. “Final Report for project: Peterborough LiDAR”. Report prepared for Ontario Ministry of Natural Resources and Forestry, Provincial Mapping Unit. https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar%20-%20Peterborough%20-%20Additional%20Metadata.pdf

Airborne Imaging 2019a. “Final Report for project: Lake Erie LiDAR”. Report prepared for Ontario Ministry of Natural Resources and Forestry, Provincial Mapping Unit. https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar%20-%20Lake%20Erie%20-%20Additional%20Metadata.pdf

Airborne Imaging 2019b. “Final Survey Report: 15221 South Central Durham Lidar Survey for Central Lake Ontario Conservation Authority” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar%20-%20CLOCA%20-%20Additional%20Metadata.pdf

Airborne Imaging 2019c. “Final Report for project: St Lawrence Hazard Mapping LiDAR”. Report prepared for South Nation River Conservation Authority. https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar%20-%20South%20Nation%20-%20Additional%20Metadata.pdf

Airborne Imaging 2022a. “Final Report for project: Lake Huron LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/LakeHuron %20Lidar%20Final%20Report%20October%202022.pdf

Airborne Imaging 2022b. “Final Report for project: Lake Simcoe LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/LakeSimcoe %20Lidar%20Final%20Report%20October%202022.pdf

Airborne Imaging 2024. “Final Report for project: Huron Georgian Bay LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Huron-GeorgianBay-Lidar-Report-Final-August-2024.docx

Airborne Imaging 2025a. “Final Report for project: Atikokan LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Atikokan-Additional-Contractor-Metadata.docx

Airborne Imaging 2025b. “Final Report for project: Cataraqui LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Cataraqui-Additional-Contractor-Metadata.docx

Airborne Imaging 2025c. “Final Report for project: Chapleau LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Chapleau-Additional-Contractor-Metadata.docx

Airborne Imaging 2025d. “Final Report for project: Dryden/Ignace/Sioux Lookout LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Dryden-Ignace-SiouxLookout-Additional-Contractor-Metadata.docx

Airborne Imaging 2025e. “Final Report for project: Kawartha Lakes LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-KawarthaLakes-Additional-Contractor-Metadata.docx

Airborne Imaging 2025f. “Final Report for project: Manitoulin LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Manitoulin-Additional-Contractor-Metadata.docx

Airborne Imaging 2025g. “Final Report for project: Sault Ste Marie LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-SaultSteMarie-Additional-Contractor-Metadata.docx

Airborne Imaging 2025h. “Final Report for project: Sudbury LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Sudbury-Additional-Contractor-Metadata.docx

Airborne Imaging 2025i. “Final Report for project: Thunder Bay LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-ThunderBay-Additional-Contractor-Metadata.docx

Airborne Imaging 2025j. “Final Report for project: Timmins LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Timmins-Additional-Contractor-Metadata.docx

Airborne Imaging 2026a. “Final Report for project: Central East LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/LidarDTM-CentralEast-Additional-Contractor-Metadata.docx

Airborne Imaging 2026b. “Final Report for project: Kenora LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-Kenora-Additional-Contractor-Metadata.docx

Airborne Imaging 2026c. “Final Report for project: Northeastern Ontario LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-NE-Ontario-Additional-Contractor-Metadata.docx

Airborne Imaging 2026d. “Final Report for project: Rainy River LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-RainyRiver-Additional-Contractor-Metadata.docx

Airborne Imaging 2026e. “Final Report for project: Upper Thames-Grand River LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-UpperThames-GrandRiver-Additional-Contractor-Metadata.docx

Airborne Imaging 2026f. “Final Report for project: Parry Sound LiDAR” https://www.publicdocs.mnr.gov.on.ca/mirb/Lidar-ParrySound-Additional-Contractor-Metadata.docx

American Society for Photogrammetry & Remote Sensing. “LAS SPECIFICATION”. Version 1.4 – R13. 15 July 2013. http://www.asprs.org/wp-content/uploads/2010/12/LAS_1_4_r13.pdf

American Society for Photogrammetry and Remote Sensing. “Positional Accuracy Standards for Digital Geospatial Data”. Version 1.0. November 2014. http://www.asprs.org/wp-content/uploads/2015/01/ASPRS_Positional_Accuracy_Standards_Edition1_Version100_November2014.pdf

Ontario. Elevation Coordination and Consultation Committee. “Elevation Accuracy Guidelines.” Version 1.1. Peterborough: King’s Printer for Ontario. 17 December 2020. https://www.publicdocs.mnr.gov.on.ca/mirb/Elevation%20Accuracy%20Guidelines%20-%20User%20Guide.docx

Ontario. Elevation Coordination and Consultation Committee. “Ontario Specifications for Lidar Acquisition”. Version 1.2. 23 December 2020. Peterborough: King’s Printer for Ontario. https://www.publicdocs.mnr.gov.on.ca/mirb/OntarioSpecificationsForLidarAcquisition.docx

Appendix A: Getting Started – Display and use of DSM Data

Opening and viewing the DSM

The DSM packages must be unzipped and will produce a folder corresponding to the name of the package, which contains the raster files (Figure 5).

Figure 5: Unzipped DSM package contents

The DSM tiles can be used for viewing or analysis using any one of the many GIS software packages capable of using IMG or TIFF raster files. The DSM represents a continuous surface of cells containing elevation (z) values in metres (Figure 6).

Figure 6: DSM displayed by elevation in greyscale

DSM tiles can be used as-is for topographic elevation analysis, but for visualization it is better to derive and display the DSM using a hillshade or a colour-shaded relief function. A hillshade is a grayscale image that represents a DEM in three-dimensional appearance as the human eye would interpret it with the sun set at a specified direction (azimuth) and altitude. A colour-shaded relief expands on this by adding color gradient based on cell z-values. Figure 7 depicts examples of a hillshade (left) with properties applied such that it is illuminated from 210° (Southwest), and a colour-shaded relief illuminated from 315° (Northwest).

Figure 7: DSM tile displayed as a hillshade and colour-shaded relief

Assembly, viewing and analysis of multiple DSM tiles: new dataset vs virtual

As with many other imagery and raster products delivered through LIO, the DSM product is provided in 1-km2 non-overlapping tiles and grouped into packages. There are several advantages to this method of storage and distribution including maintenance of Metadata, storage considerations and the flexibility to allow clients to request the data they need for their study area, without requiring custom data extracts to client supplied boundaries.

If your study area covers multiple DSM tiles, these individual tiles can be viewed simultaneously, but for the purposes of analysis, or simply for faster and less complicated viewing, it is usually better to put all tiles in your study area together as one. This can be done in several different ways.

The traditional method is to assemble or merge all the individual tiles into a new raster covering your entire study area. This method can allow for faster display and may be necessary for some analysis operations but has the disadvantage that a separate new dataset must be produced which has implications for storage space. Being a new dataset, the naming and metadata of the individual tile structure used as a source will be lost as well.

Raster data takes up a great deal of storage space, so it is often a preferred approach to make use of a virtual representation of multiple tiles as one entity. These methods can include raster mosaic datasets, and virtual raster catalogs. Most of these methods have the advantage that the original files, metadata, folder structure and locations are preserved and the data for contributing tiles or groups of tiles can even be stored in separate different locations as storage space requirements dictate.

The exact methods and options available will depend on the software being used, and the requirements of the analysis. Please consult the documentation of the GIS software you are using for specific details about the options available to you.

Appendix B: Common DEM Analysis Functions

A DEM raster can be used as an input for a variety of surface geometry analysis functions such as calculations of slope, aspect, and curvature. The actual algorithms used for any of these calculations may vary depending on the software used to perform the calculation.

Slope

Slope is a calculation based on difference between the rise and the run along a specified path. The output value assigned to a cell is usually based on the maximum difference in elevation values compared to adjacent cells and is commonly expressed in degrees or percent. See the left side of Figure 9 for an example.

Aspect

Aspect is a calculation that applies a numeric value to a cell based upon the bearing that the slope of the cell is facing, in degrees. These values can be further categorized into compass directions of North, South, Southeast, etc. For instance, in Figure 8 (right), the aspect of the landscape is predominately facing to the South and Southwest.

Figure 8: Examples of Slope and Aspect

Curvature

Curvature is a calculation based upon the shape of the surfaces which is calculated for each cell based compared to neighbouring cells. Positive curvature values indicate a convex surface, while negative values indicate a concave surface. It is also possible to extract the profile curvature of the slope, or the plan (horizontal) curvature values.

Appendix C: Important Considerations and Product Limitations

Vertical datum difference

As noted in Section 1.4.2 Vertical Reference System, the Lidar Eastern Acquisition Project (LEAP) was captured in the older CGVD 1928 vertical datum. If users wish to combine LEAP data with data from more recent acquisitions steps should be taken to transform the data to a common vertical datum. As seen in the profile graph in Figure 9, there is a consistent vertical offset between data from the LEAP and OMAFRA lidar acquisitions.

Figure 9: Vertical profile graphs along a 50 metre transect showing the difference between data in the CGVD 1928 and CGVD 2013 vertical datums

Scan line artefacts

In some parts of the DSM a ripple effect has been observed that corresponds with the swath overlap, spacing and angle of the scan lines from the lidar point cloud and has been introduced as a result of the DSM interpolation process. As seen in Figure 10, the effect can be noticeable in hillshade views of some areas depending on the parameters used, however the elevation differences in samples that have been examined in cross-section have been quite marginal (Figure 11).

Figure 10: Scan line artefacts with profile location and profile elevation values

Figure 11: Chart of elevation values along the profile

If this artefact poses an issue for user applications, some workarounds may include:

Display of DSM hillshade with different illumination parameters.

Resampling or aggregating the DSM to a coarser resolution.

Ensuring derivative products are mapped to appropriate scales/ resolutions to minimize the effect.

Creation of a custom DSM from the point cloud using different resolution or interpolation method settings.

Tile boundaries extend beyond the collection area boundary

Tile cells outside of the project extent area are recorded with a value of No Data however some raw point cloud data was collected that extends a short distance beyond the project boundary (Figure 12). If analysis is required in these boundary areas skilled users may consider deriving a DSM using the raw lidar point cloud data after editing and classification is performed.

Figure 12: Detail comparing raw data collection, project, and package extents

Hydro-flattening

The DSM has not been hydro-flattened. This is a trade off that preserves features overhanging water, such as vegetation and hydro lines, at the cost of having noise in open water (Figure 13).

Figure 13: Comparison of DTM (left) to DSM (right), with hydrographic breaklines displayed as blue lines. In the image on the right overhanging vegetation and overhead wires are visible, as well as elevation noise from the water surface.

It is possible to produce a DSM that both preserves overhanging features and is hydro-flattened. However, this would require additional classification of the point cloud. This would involve classifying vegetation and wires. With these classes available it would be possible to enforce the hydrographic breaklines in the creation of the DSM, then produce separate rasters from the vegetation and wire classes and mosaic them on top of the DSM.

Version 2324.1

This project is funded in part by the Government of Canada through the Flood Hazard Identification and Mapping Program.

Accuracy classes as described in the Ontario Specifications for Lidar Acquisition