Provincial Digital Elevation Model (PDEM)

User Guide

Geospatial Ontario

Digital, Data, Information and Business Services Branch

Corporate Management and Information Division

Ministry of Natural Resources

2026-03-12

Disclaimer

This technical documentation has been prepared by His Majesty the King in right of Ontario as represented by the Ministry of Natural Resources (the “Ministry”). No warranties or representations, express or implied, statutory or otherwise shall apply or are being made by the Ministry with respect to the documentation, its accuracy or its completeness. In no event will the Ministry be liable or responsible for any lost profits, loss of revenue or earnings, claims by third parties or for any economic, indirect, special, incidental, consequential or exemplary damage resulting from any errors, inaccuracies or omissions in this documentation; and in no event will the Ministry’s liability for any such errors, inaccuracies or omissions on any particular claim, proceeding or action, exceed the actual consideration paid by the claimant involved to the Ministry for the materials to which this instructional documentation relates. Save and except for the liability expressly provided for above, the Ministry shall have no obligation, duty or liability whatsoever in contract, tort or otherwise, including any liability or negligence. The limitations, exclusions and disclaimers expressed above shall apply irrespective of the nature of any cause of action, demand or action, including but not limited to breach of contract, negligence, strict liability, tort or any other legal theory, and shall survive any fundamental breach or breaches.

Cette publication spécialisée n’est disponible qu’en anglais.

Additional Information

This document does not comply with all the applicable guidelines for accessible digital documents. For an alternative format please email Geospatial Ontario at geospatial@ontario.ca.

For more information about this document, please contact Geospatial Ontario at geospatial@ontario.ca.

Executive Summary

Key Words

Digital Elevation Model, Digital Surface Model, Digital Terrain Model, cost path, elevation, terrain, topography, raw, interpolation, resample, unenforced, medium scale, orthoimagery, orthophoto.

Abstract

The Provincial DEM (PDEM) is designed to represent true ground elevation where possible across the province. Because the PDEM is based on best available source data for different areas of the province, it is a general-purpose dataset from which other special purpose datasets have been derived. This document provides a general description of the data sources used to create the PDEM, the structure of the dataset, as well as some data use and limitation considerations.

Revision History

| Date | Edition/ Version | Description |
| --- | --- | --- |
| 2018-05-17 | 1.0 | First public release |
| 2020-04-08 | 1.1 | Document references and hyperlinks updated |
| 2020-09-25 | 1.2 | Summer 2020 data source update and introduction of new vertical datum. |
| 2022-03-02 | 1.3 | Winter 2022 data source update. |
| 2022-08-03 | 1.4 | Updated OWIT and MNRF references |
| 2023-06-27 | 1.5 | Updated template; added note on the status of future data source updates to Figure 3; added access to OWIS and Ontario GeoHub Elevation Mapping Program page. |
| 2024-05-02 | 2.0 | Text updated along with French translation. |
| 2025-03-31 | 2.1 | Revised French translation. |
| 2025-08-26 | 2.2 | Summer 2025 data source update. |
| 2026-03-12 | 3.0 | Document references and hyperlinks updated, and French translation completed |

Table of Contents

Disclaimer	2

Additional Information	2

Executive Summary	3

Key Words	3

Abstract	3

Revision History	3

Table of Contents	4

List of Figures	6

List of Tables	6

List of Acronyms	7

List of Definitions	9

Contour	9

Digital Elevation Model (DEM)	9

Digital Terrain Model (DTM)	9

Digital Surface Model (DSM)	10

Hydrologic Enforcement	10

Interpolation	11

Mass Points	11

1. Introduction	12

Versioning	13

2. Product Details	13

Spatial Representation and Geographic Extent	13

Horizontal Reference System	17

Horizontal Datum and Coordinate System	17

Horizontal Unit of Measure (coordinate system axis units)	17

Vertical Reference System	18

Vertical Coordinate System	18

Vertical Unit of Measure (coordinate system axis units)	18

Data Delivery	18

Elevation Products Available in Ontario	20

3. Elevation Sources and Interpolation	21

Source Elevation Data	21

Ontario Basic Mapping Elevation Data	23

Ontario Radar DSM (ORDSM)	24

All Other Elevation Updates	24

Water Bodies	25

Forest Vegetation	26

4. Recommended Data Uses and Considerations	27

Recommended Data Uses	27

Considerations	28

DEM Interpolation	28

Raster Merging and Mosaicking	28

DEM Maintenance	29

5. References	29

General References	29

Guideline References	30

Metadata References and Resources	30

List of Figures

Figure 1: Extents of the PDEM showing the divide just south of 50 degrees latitude between the northern and southern tiles	15

Figure 2: Location of St Marys Cement Quarry – the lowest point in southern Ontario – approximately 78m below the average level of nearby Lake Ontario	16

Figure 3: Extent of source elevation datasets used to create the PDEM. Please consult the PDEM Spatial Metadata Index for the individual data sources, and the most current and detailed depiction of the product. Also refer to the Ontario Elevation Mapping Program page for the most current map.	22

List of Tables

Table 1: Key fields in the PDEM Spatial Metadata Index. The ‘Raster Update Date’ (RAS_UPD_DT) field tells the user when a particular area was updated. Consult the spreadsheet packaged with the index on Ontario GeoHub for other attribute descriptions.	21

Table 2: Ontario Basic Mapping specifications (adapted from MNR, 1994)	24

List of Acronyms

ANUDEM: Australian National University Digital Elevation Model software

ASL: Above Sea Level

CGVD28: Canadian Geodetic Vertical Datum 1928

CGVD2013a: Canadian Geodetic Vertical Datum of 2013

CHS: Canadian Hydrographic Service

CO-OPS: Center for Operational Oceanographic Products and Services

CNT: Canadian National Transformation

DEM: Digital Elevation Model

DSM: Digital Surface Model

DTM: Digital Terrain Model

ESRI or Esri: Environmental Systems Research Institute, Inc.

FOC: Fisheries and Oceans Canada

GEO: Geospatial Ontario

GTA: Greater Toronto Area

IfSAR: Interferometric Synthetic Aperture Radar

LCC: Lambert Conformal Conic

LiDAR or lidar: Light Detection and Ranging

MNR: (Ontario) Ministry of Natural Resources

MNRF: (Ontario) Ministry of Natural Resources and Forestry (former name)

MSL: Mean Sea Level

NAD83: North American Datum of 1983

NASA: National Aeronautics and Space Administration

NOAA: National Oceanic and Atmospheric Administration

OBM: Ontario Basic Mapping

OWIS: Ontario Web Image Services

OWIT: Ontario Watershed Information Tool

ORDSM: Ontario Radar Digital Surface Model

PDEM: Provincial Digital Elevation Model

SPARROW: SPAtially Referenced Regression On Watershed attributes

SRTM: Shuttle Radar Topography Mission

WRIP: Water Resources Information Program

List of Definitions

Contour

A contour is a linear vector feature representing isolines, or lines of equal elevation, in metres above mean sea level.

Digital Elevation Model (DEM)

DEM is a generic term for digital topographic and/or bathymetric data that is comprised of x/y coordinates and z-values to represent an elevation surface.

The term ‘DTM’ and ‘DSM’ should be used over the term ‘DEM’ to more specifically reference ‘bare-earth’ or ‘surface elevation’ model products when possible.

The term ‘DEM’ is to be used as a broader term when referencing a generic elevation data product.  The Provincial DEM is an example of a generic elevation product given that it has been constructed using a combination of both ‘DTM’ and ‘DSM’ elevation datasets to achieve Provincial coverage.

Digital Terrain Model (DTM)

The bare earth surface (lowest surface, last reflective surface, or lidar last-return) represents the surface of the "bare-earth" terrain, after removal of vegetation and constructed features.

Photogrammetry has traditionally generated DTMs when elevations are generated by manual compilation techniques. Unless specified to the contrary, the bare-earth surface includes the top surface of water bodies, rather than the submerged surface of underwater terrain.

Similar to a DSM, a DTM can be structured either as a vector dataset (comprised of mass points and optionally 3D breaklines) to model bare-earth elevations or a raster dataset that is interpolated from the vector elevation data to model bare-earth terrain elevations.

Using modern elevation point cloud classification algorithms and file formats, such as LAS, a DTM can represent a mass point dataset that has been classified for ‘bare-earth’ terrain elevations.

Digital Surface Model (DSM)

A DSM is the highest reflective surface of ground features captured by the sensor. This surface may also be referred to as the first reflective surface or lidar first-return. The DSM may include treetops, rooftops, and tops of towers, telephone poles, and other natural or manmade features; or it may include the ground surface if there is no vegetative ground cover. Photogrammetry, IfSAR, lidar and sonar can all provide this type of surface, yet characteristics such as accuracy and degree of detail (ability to resolve desired surface features) may vary significantly across technologies and even within the same technology. With sonar, the DSM may include sunken vessels and other artifacts, whereas the bathymetric surface reflects the natural underwater terrain. Similarly, with photogrammetry, lidar, and IfSAR the reflective surface may include any artifact present when the sensor mapped the area, including passing cars and trucks and similar features not normally considered to be part of a digital terrain model.

Similar to a DTM, a DSM can be structured either as a vector dataset (comprised of mass points and optionally 3D breaklines) to model surface elevations or a raster dataset that is interpolated from the vector elevation data to model surface elevations.

Using modern elevation point cloud classification algorithms and file formats, such as LAS, a DSM can represent a mass point dataset that has been classified for ‘surface’ elevation features.

Hydrologic Enforcement

Hydrologic enforcement is the process of conditioning a DEM to allow each cell to drain in a downward direction with barriers to flow being removed, where possible. The two main functions of hydrologic enforcement include clearing spurious sinks and ensuring that cells along streams decrease with elevation in a downstream direction.

Interpolation

The process of deriving elevations for a location on the Earth’s surface based on points where elevations have been measured. Many interpolation algorithms exist including natural neighbour, spline, inverse distance weighting, and kriging. Resampling to a different resolution (e.g., cell size) often involves re-interpolation.

Mass Points

Mass points are irregularly spaced points, each with x/y location coordinates and z-values, typically (but not always) used to form a TIN. When generated manually, mass points are ideally chosen to depict the most significant variations in the slope or aspect of TIN triangles. However, when generated automatically (e.g., by lidar or IfSAR scanner), mass point spacing and patterns depend upon the characteristics of the technologies used to acquire the data.

Introduction

The Provincial Digital Elevation Model (PDEM) is a seamless 3-dimensional model that represents ground surface elevations in metres above sea level (ASL).  The PDEM can be used for a number of applications such as landscape classification and general terrain analysis and has not been conditioned for any specific application.

DEMs are used for a broad range of studies including projects tied to the fields of geology, forestry, ecology, climatology, and hydrology. A DEM that more closely represents ground elevations, such as the PDEM, can be used to determine terrain attributes such as slope, aspect, or elevation at a specific location. DEMs are also used for remotely predicting geologic features such as fault lines, landforms and inferring material types. One of the main drivers and dependencies is the need for an elevation model to support the generation of Provincial integrated hydrology data. For hydrologic analysis, such as watershed generation and flow tracing, use of the Ontario Integrated Hydrology Data product is recommended (MNR, 2026a). However, the PDEM can serve as a useful reference for watershed analysis particularly when unmodified ground elevations are required along river corridors, which are altered during the hydrologic conditioning process.

The PDEM has traditionally been compiled using some broad scale data sources derived from the Ontario Basic Mapping (OBM) program and NASA’s Shuttle Radar Topographic Mission (SRTM) including the Ontario Radar Digital Surface Model (ORDSM), OBM DTM, OBM Spot Height, and OBM Contour. Contours and spot heights had been used as interpolation sources where coverage of DTM data was lacking. Since then, the province has been continuing to incorporate best available data from newer elevation data acquisitions starting with photogrammetric data sources like the 2002 GTA DEM, 2015 SWOOP DTM, and now various recent lidar acquisitions in Ontario. For the majority of Ontario’s Far North region, the ORDSM was used to provide complete provincial coverage for the product.

Versioning

To meet broader end-user needs and maintenance requirements, the PDEM is now produced seamlessly for the province. The older tiled versions of the PDEM have been retired.

Version numbering for the PDEM is no longer supported. Instead all versioning information is now tracked in what is referred to as the ‘PDEM Spatial Metadata Index’, which stores the data source used and the date the data source was incorporated (see section 3 for more information).

Product Details

Spatial Representation and Geographic Extent

A raster grid is used to represent the DEM where each raster grid cell has a single elevation value associated. The regular grid spacing is based on the Lambert Conformal Conic (LCC) projection with a raster cell resolution of 30 metres across the entire extent of the province.

The DEM is divided into 2 regions called “PDEM North” and “PDEM South”, both snapped to the nearest 30m at the origin. The horizontal extent of the data provides continuous spatial coverage for the entire province of Ontario. The approximate bounding extent for the data set is:

West-bounding coordinate: -96 degrees longitude

East-bounding coordinate: -74 degrees longitude

North-bounding coordinate: 57 degrees latitude

South-bounding coordinate: 41 degrees latitude

Some notable extent enhancements include the following:

Areas were added beyond the provincial border which drain into northern Ontario to support community-based planning initiatives for indigenous communities in the far north;

The entire Rainy River Basin stateside was added to support various activities such as water quality analysis (IJC SPARROW nutrient loading modeling), watershed analysis (OWIT and USGS StreamStats online tools), and watershed delineation (IJC International Watersheds Initiative) occurring along the international border;

The geographic extent of the Great Lakes has been captured primarily as an aesthetic addition to the PDEM. However, the lake flattening applied honours the long-term average lake levels reported in partnership by Fisheries and Oceans Canada – Canadian Hydrographic Service (FOC-CHS) and the U.S. National Oceanic and Atmospheric Administration – Center for Operational Oceanographic Products and Services (NOAA-CO-OPS).

See Figure 1 for more extent details.

Figure 1: Extents of the PDEM showing the divide just south of 50 degrees latitude between the northern and southern tiles

The vertical extent of the dataset is expressed in metres. The lowest vertical extent naturally occurs along the coast of James Bay and Hudson Bay which reaches the elevation of mean sea level (MSL). However, there are select areas of the province where the elevation can drop below MSL which is typically caused by anthropogenic influences. For example, the St. Marys Cement mine in southern Ontario is the lowest point recorded at 1.54m below MSL (see Figure 2). The highest vertical extent of the PDEM is 688.23 metres ASL, located at Ishpatina Ridge (47.320N, 80.750W) in Lady Evelyn-Smoothwater Provincial Park located approximately 70 kilometres west of the town of Latchford. Note another source estimates this point to be approximately 693m ASL (NRCan, 2009). The difference can be attributed to the rounding effect of the ANUDEM interpolation process and is within the stated accuracies for OBM data sources (see the Interpolation Process section below for more details).

Figure 2: Location of St Marys Cement Quarry – the lowest point in southern Ontario – approximately 78m below the average level of nearby Lake Ontario

Horizontal Reference System

Horizontal Datum and Coordinate System

The horizontal datum and coordinate system used for the data is North American Datum 1983 (NAD83) / CSRS Ontario MNR Lambert. This datum and coordinate system are defined within the European Petroleum Survey Group’s (EPSG) Geodetic Parameter Registry (Code 3162) (https://epsg.io/3162). The following parameters are used in association with the coordinate system:

False Easting: 930000.00

False Northing: 6430000.00

Central Meridian: -85.00

1st Standard Parallel: 44.50

2nd Standard Parallel: 53.50

Latitude of Origin: 0.00

Linear Unit: Metres

The LCC projection provides minimal distortion for shape, area and direction with scale being accurate along the standard parallels and is best suited for representing broader regions up to a latitude range of 35 degrees (Esri, 2013). As only one coordinate system and set of parameters are required, the entire province can be effectively represented using one spatial resolution.

Horizontal Unit of Measure (coordinate system axis units)

The horizontal unit of measure is metres (m).

Vertical Reference System

Vertical Coordinate System

The PDEM is currently available in two vertical datums including:

- the Canadian Geodetic Vertical Datum 1928 (CGVD28) of the Geodetic Survey Division and is measured in metres above MSL, determined by tidal gauges across Canada. This datum is defined within the European Petroleum Survey Group’s (EPSG) Geodetic Parameter Registry (Code 5713) (https://epsg.io/5713);

- the Canadian Geodetic Vertical Datum of 2013 (CGVD2013a) of the Geodetic Survey Division and is measured in metres above MSL, realized through the Canadian gravimetric geoid 2013a. This datum has been adopted by the Government of Canada as the new reference standard for heights across Canada and is defined within the European Petroleum Survey Group’s (EPSG) Geodetic Parameter Registry (Code 9245) (https://epsg.io/9245).

The older CGVD28 datum will continue to be made available until all PDEM-derivative products Geospatial Ontario manages have been updated with the new vertical datum; and the province officially adopts CGVD2013a as the new reference standard for Ontario.

Vertical Unit of Measure (coordinate system axis units)

The vertical unit of measure for all raster grid cells in the DEM is metres (m). One single vertical elevation value is associated with each raster grid cell in the DEM.

Data Delivery

The PDEM and metadata are currently stored and distributed through Geospatial Ontario (https://www.ontario.ca/page/geospatial-ontario). Metadata and distribution packages for the PDEM can be accessed through the Ontario GeoHub website.
(https://geohub.lio.gov.on.ca/search?q=PDEM)

If you navigate to the landing page of Ontario GeoHub, you can also type in “PDEM” when searching to bring up the record.
(https://geohub.lio.gov.on.ca)

The physical data is available for download as two GeoTIFF files labelled “PDEM North” and “PDEM South” (see Figure 1). Each GeoTIFF file is approximately 1-2 gigabytes in size when zipped. There is also a ‘PDEM – Boundary’ polygon shape file available for download which outlines the tiled extents of the PDEM.

When using the GeoTIFF for the first time, building an overview pyramid file (extension .OVR) is recommended for significantly faster viewing and rendering in GIS software applications. For ArcGIS users, under ArcCatalog, simply right-click the raster, select Build Pyramids… and accept the default options for best performance. It should take only a couple minutes to generate. ArcMap may also prompt you to build pyramids when first adding the DEM to the project. If you click OK, pyramids will be built based on how your Raster Storage options are set under the Environment Settings. If you are unsure of these settings, click Cancel and build manually as described above.

Pyramid Setting Tips:

“NEAREST” resampling is preferred for non-discrete data since this technique is the fastest for generating pyramids. The other resampling methods employ a weighting or averaging based on neighbouring values, which is not necessary if the primary use for pyramids here is display performance.

You may choose to further compress the pyramid file, but little advantage will be gained for floating point data and rendering may in fact be slower accessing highly compressed pyramid files.

For other GIS software, please consult the appropriate help documentation for properly generating an overview pyramid file.

In ArcMap or ArcGIS Pro, we also recommend building statistics for each DEM tile to accurately display the elevation range in the symbology. Changing the settings Symbology > Statistics to “From Current Display Extent”, and Symbology > Stretch to “Percent Clip” will give the best contrast dynamically at any scale. You can also use the Image Analysis Window to dynamically create hill shades or other derivatives if you desire. Consult your specific software package help documentation for the appropriate optimum display settings.

Since the release of the Ontario Web Image Services (OWIS) in the fall of 2022, the PDEM is also available through an image service which exposes the data for visualization and geoprocessing.

The service is best accessed through the ArcGIS REST API, either directly or by setting up an ArcGIS server connection using the REST endpoint URL. The service draws using the Web Mercator projection.

For more information on what functionality is available and how to work with the service, please refer to the Ontario Web Image Services User Guide. If you have questions about how to use the service, email Geospatial Ontario at geospatial@ontario.ca.

Elevation Products Available in Ontario

In October 2014, GEO released the ‘Ontario Elevation Data Index’. It allows users to discover the extents and the metadata associated for elevation products within the province of Ontario. To discover the index and view all the available elevation products check out the Ontario Elevation Data Index page on the Ontario GeoHub. (https://geohub.lio.gov.on.ca/datasets/mnrf::ontario-elevation-data-index-1)

Elevation Sources and Interpolation

Source Elevation Data

The PDEM was created to provide a broad scale elevation data product that closely represents true ground elevations where possible. The PDEM was constructed using several data sources to provide seamless coverage for the entire province. These data sources and their extents are illustrated in Figure 3. The current version of the ‘PDEM Spatial Metadata Index’ is also available for download as a shape file directly from the PDEM metadata page (https://geohub.lio.gov.on.ca/search?q=PDEM) on the Ontario GeoHub website or viewed online from the Ontario Elevation Mapping Program page (https://geohub.lio.gov.on.ca/pages/ontario-elevation-mapping-program).

Consult the online dataset for the most up-to-date version because the image portrayed in Figure 3 may not always reflect the latest updates, and the figure’s small scale does not allow for a clear depiction of where all the updates occurred. Also note that this dataset may contain abbreviated names for the data sources used to compile the PDEM. Please consult the ‘Ontario Elevation Data Index’ available in Ontario GeoHub for the official names of all elevation products acquired for Ontario.

Table 1: Key fields in the PDEM Spatial Metadata Index. The ‘Raster Update Date’ (RAS_UPD_DT) field tells the user when a particular area was updated. Consult the spreadsheet packaged with the index on Ontario GeoHub for other attribute descriptions.

| Key Field Name | Data Type | Description |
| --- | --- | --- |
| ProjectID | INTEGER | Project Identifier for each data source ordered by time entered in database. |
| RAS_UPD_DT | DATE | Date when data source was incorporated into the PDEM. Full Name: Raster Update Date. |
| SqKm | DOUBLE | Area in square kilometres calculated in Canada Albers Equal Area Conic projection. |
| Source | STRING | The elevation data source abbreviated name, interpolation method (if necessary), and/or the year the data source was acquired. |

Figure 3: Extent of source elevation datasets used to create the PDEM. Please consult the PDEM Spatial Metadata Index for the individual data sources, and the most current and detailed depiction of the product. Also refer to the Ontario Elevation Mapping Program page for the most current map.

Ontario Basic Mapping Elevation Data

As the ministry continues to acquire new elevation data in different areas of the province, the vintage OBM data which currently exists in the PDEM will gradually be replaced. In this release of the product, southwestern Ontario has now been updated with more recent acquisitions. Central Ontario may soon follow suit with data acquired through the Forest Resource Inventory program depending on its suitability to represent true ground.

Elevation base data acquired through the OBM program between 1976 and 1996 was interpreted using traditional stereo plotter photogrammetric techniques (for more information on the acquisition history, download the “OBM Photo Block Index” from the Data Distribution section of the Ontario GeoHub metadata page). Data was captured at a scale of 1:10,000 in southern Ontario and 1:20,000 in northern Ontario based on the data capture specifications (MNR, 1994).

Table 2 provides scales, accuracy and mapping specifications for the OBM data. Even though these specifications pertain to OBM, the stated accuracies can serve as a guide for the other data sources as well. In particular, the ORDSM described in the next section is comparable in scale and accuracy. The newer data sources will have significantly greater accuracy in their native resolutions. However, when they are generalized to a 30m resolution, the range in elevation over 30m pixels can be highly variable particularly in steep terrain resulting in an accuracy in some areas that is more consistent with the regional data sources (MNR, 2022).

All OBM data was initially captured using the Universal Transverse Mercator (UTM) projection within four UTM Zones (Zones 15, 16, 17 and 18). Elevation data was originally captured using the North American Datum 1927 (NAD27) but has been subsequently converted to the GO-ITS Standard datum of North American Datum 1983 (NAD83) using the National Transformation Version 2 (NTv2) specifications.

Table 2: Ontario Basic Mapping specifications (adapted from MNR, 1994)

| Specification | Southern Ontario | Northern Ontario |
| --- | --- | --- |
| Nominal Scale of Photography | 1:30,000 | 1:50,000 |
| Nominal Scale of Mapping | 1:10,000 | 1:20,000 |
| Absolute Positional Accuracy | 5 metres | 10 metres |
| Absolute Vertical Accuracy (contours & DTM) | 2.5 metres | 5 metres |
| Contour Interval | 5 metres | 10 metres |
| DTM Elevation Point Spacing (max) | 60 metres | 100 metres |
| Spot Elevation Point (vertical accuracy) | 1.25 metres | 2.5 metres |

Ontario Radar DSM (ORDSM)

Coverage in northern Ontario has been sourced from 1 arc-second spaceborne C-band interferometric radar data acquired by NASA from the 2000 Shuttle Radar Topography Mission (SRTM), which was used to create the ORDSM data product (see Figure 3). During product construction, the spatial resolution and geospatial coordinate system was made consistent with the PDEM, which allowed for the elevation to be integrated seamlessly. It should be noted that, unlike the other source elevation data mentioned, elevation values in SRTM based data sources are more influenced by vegetation and other surface features which reduces the accuracy of representing true ground elevations.  For more information on this data source please refer to the ORDSM metadata (MNR, 2020).

All Other Elevation Updates

For all newer elevation data sources that were used to update the PDEM (starting with the 2002 GTA product), please consult the appropriate holding in Ontario GeoHub for detailed information concerning these products. For data improvements and methods used to compile the PDEM, please refer to the ‘PDEM Technical Report’ available for download from the Ontario GeoHub metadata page (MNR, 2022).

Water Bodies

A consistent lake and river flattening process was applied to the Great Lakes, the Connecting Channels and the upper St. Lawrence River, which respects the long-term average lake and river levels observed by NOAA and FOC-CHS binational partners. Since the Niagara River is not a navigable river, it is not as rigorously monitored by these agencies, and therefore a different method using orthophotography breaklines was employed. Data sources included 2002 GTA breaklines (Ontario side) and 2010 SWOOP breaklines (U.S. side). For more information, please refer to the ‘PDEM Technical Report’ (MNR, 2022).

The lower St. Lawrence River (from the foot of the Iroquois Dam to the Ontario border) used an older method to adjust shoreline elevations which involved the creation of a ‘Great Lake Shoreline Contour’ dataset. The contour was derived from the original OBM representation of the water body edge and the elevation values were extrapolated from nearby OBM spot heights within the water body. The accuracy of these points could not be validated without proper control, so the adjustments were based on the interpreter’s best judgement in ensuring a downward trend along the shoreline edge. As a result, some anomalies may also exist along the interface between where the Great Lakes and upper St. Lawrence were later flattened and the adjacent inland cells that may have been previously adjusted by the Great Lake Shoreline Contour dataset in areas where OBM still exists. These areas would include the Lake Huron shoreline, St. Marys River, Lake Superior shoreline, the St. Lawrence River and select islands. New enhanced FRI data sources or similar large-scale acquisitions in the future, are expected to rectify this issue where it may exist.

Where possible, shoreline elevations for inland water bodies have also been included across the province where OBM still exists. Lake elevations were photogrammetrically captured during the OBM program and included within the OBM Spot Height dataset. Actual elevations within these lakes are not necessarily maintained as areas of constant elevation (e.g., flat surfaces) when replaced by a new data source.

For other areas of the province, different techniques for lake and river flattening were employed depending on the acquisition (this includes ORDSM/SRTM and large-scale data sources). Not all features were flattened, a decision that was typically based on a minimum width criterion. Please consult each data holding in LIO for more information on the acquisition specifications used where available.

Forest Vegetation

Some data sources may not represent true ground in some cases and behave more like a DSM capturing the top of features (e.g. tree canopy, building rooftops, and so on).

As mentioned earlier, the SRTM/ORDSM data product is one example where this may be evident particularly in dense vegetation cover. Little can be done here to rectify the situation without a proper baseline for comparison for the majority of the far north of Ontario.

The SWOOP 2015 data product is another example where capturing true ground elevation through pixel autocorrelation was not always possible. This is particularly the case in the Bruce Peninsula / Beaver Valley region where dense vegetation cover is predominant. However, there are multiple data sources to draw upon here for potential validation and correction. The OBM data product, for example, was relatively good in capturing significant trends in the terrain. Even though the OBM is considered a vintage dataset and may not reflect current conditions, the Bruce Peninsula / Beaver Valley region is a largely protected natural landscape with distinct morphology which has changed very little over time since the OBM elevation data was acquired.

Therefore, the SWOOP product was validated against the OBM and areas that were significantly different often occurred where there was dense forest cover. Areas greater than 10 metres difference were completely replaced by OBM derived elevation. In areas between 4 to 10 metres difference, a percentage gradation or weighting was applied to smooth out the transition between SWOOP and OBM data sources (see the inset of Figure 3 for the area where this enhancement was applied). The end result was an improved elevation product which better represented true ground in this area. For more details on how this enhancement was conducted, please refer to the ‘PDEM Technical Report’ (MNR, 2022).

Recommended Data Uses and Considerations

Recommended Data Uses

The PDEM has several important applications and uses where true ground elevations are required especially along riparian channels and other areas altered in the hydro-enforcement process. Some suitable uses include:

Generating slope and aspect derived products;

Querying elevation values in a given area, such as a watershed unit;

As a reference or input dataset for predictive mapping associated with the study or mapping of surficial geology, land cover, morphology, and terrestrial and aquatic ecology; and,

Where users prefer to apply their own hydrology enforcement techniques to address site-specific issues not addressed in the Ontario Integrated Hydrology (OIH) data product, the PDEM can serve as a suitable input data source.

MNRF uses the PDEM for several broad scale applications and serves as a useful reference and source dataset in the compilation of OIH, OHN, and Ontario’s land cover products to name just a few.

Considerations

DEM Interpolation

The PDEM employs various interpolation techniques including:

ANUDEM/Esri Topogrid spline interpolation for areas covered by OBM;

Esri Local Polynomial Interpolation (LPI) algorithm for areas covered by SRTM (MNR, 2020); and,

Esri Bilinear Resampling of finer resolution DTM’s derived from more recent large-scale elevation data acquisitions. Please refer to the ‘PDEM Technical Report’ for more information on methods employed (MNR, 2022).

It is the responsibility of the user to determine if the PDEM is suitable for their application or whether another product should be used. To obtain information on other elevation data products available within the province, please refer to the LIO Metadata Management Tool website (click the ‘Elevation’ link for a list of data or lookup the Ontario Elevation Data Index). If you require further assistance, please contact Geospatial Ontario at geospatial@ontario.ca.

Raster Merging and Mosaicking

When two or more raster datasets need to be assembled to cover a specific study area a raster merge or mosaic function must be used. The method typically employed for the PDEM is the ‘Path of Least Difference’ or the Esri Least Cost Path method as it is referred to in ArcGIS (Esri, 2016). This allows for a smooth transition along the raster boundaries with only a minor amount of interpolation along a one-cell wide overlap between different data sources. For more information on this method, please refer to the ‘PDEM Technical Report’ (MNR, 2022) or ‘User Guide for Least Cost Path Analysis Tool’ (MNR, 2026b).

DEM Maintenance

Ongoing maintenance will continue with the PDEM product to address errors and data improvements over time. Updates will include the incorporation of large-scale elevation products that become available for certain areas of the province to improve vertical accuracy and to better represent current conditions where possible. This will depend on feasibility or expressed need and suitability for incorporation. In some areas a change detection analysis could be used to determine if the current PDEM needs to be updated, depending on the magnitude of change. Large scale elevation data would need to be re-processed or resampled into a suitable medium scale data product before incorporating into the PDEM. For more information, please refer to the ‘PDEM Technical Report’ (MNR, 2022).

References

General References

Esri, 2016. Creating the least cost path, ArcGIS Help 10.3, Environmental Science Research Institute. (http://desktop.arcgis.com/en/arcmap/10.3/tools/spatial-analyst-toolbox/creating-the-least-cost-path.htm)

Esri, 2013. ArcGIS Desktop 9.3 Help – Lambert Conformal Conic, Environmental Science Research Institute. (http://webhelp.esri.com/arcgisdesktop/9.3/index.cfm?TopicName=Lambert_Conformal_Conic)

Ministry of Natural Resources and Forestry (MNRF), 2008. Provincial DEM Version 2.0.0 Product Specification. Geographic Information Branch, Science and Information Resources Division, Ontario Ministry of Natural Resources, 25p.

Zhao, J., Todd, K., Hogg, A. and Kenny, F. 2008. Improving Ontario’s Digital Elevation Model, Internal Report. Water Resources Information Program, Ontario Ministry of Natural Resources, 90p.

Guideline References

MNR, 1994. Ontario Government Specifications for the Delivery of Digital Topographic Data and Cartographic Representation Products at Medium Scales. Geographic Information Branch, Science and Information Resources Division, Ontario Ministry of Natural Resources, 59p.

Metadata References and Resources

MNR, 2020. Ontario Radar DSM, Metadata record. Geospatial Ontario, Digital, Data, Information and Business Services Branch, Corporate Management and Information Division. (https://geohub.lio.gov.on.ca/search?q=ORDSM_en)

MNR, 2022. Provincial Digital Elevation Model (PDEM) Methods and Processes Technical Report. Provincial Mapping, Mapping and Information Resources Branch, Corporate Management and Information Division.
(https://geohub.lio.gov.on.ca/search?q=PDEM_en)

MNR, 2026a. Ontario Integrated Hydrology Data, Metadata record. Geospatial Ontario, Digital, Data, Information and Business Services Branch, Corporate Management and Information Division. (https://geohub.lio.gov.on.ca/search?q=OIH_en)

MNR, 2026b. Updating Provincial Elevation Data Using Least Cost Path Analysis Technical User Guide. Geospatial Ontario, Digital, Data, Information and Business Services Branch, Corporate Management and Information Division.
(https://geohub.lio.gov.on.ca/search?q=PDEM_en)

Version 2324.1

All new data sources acquired by the province are not necessarily incorporated and are evaluated in terms of available resources, the perceived benefits, and the return on investment for supporting the suite of derived products produced by the province.

All elevation values referenced in this document are in CGVD28 unless otherwise stated.